3GPP TSG-RAN5 Meeting #94-e	R5-221126

# Electronic Meeting, 21st Feb – 04th Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1587 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating FR1 ACLR for intra-band CA test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In FR1, MPR for intra-band non-contiguos UL CA is introduced in R5-221122. Thus ACLR for CA test case need to be update accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating ACLR for Intra-band contiguous CA and Intra-band non-contiguous CA testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 6.5A.2.4.1 is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0565 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| Y |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5A.2.4.1	NR ACLR

6.5A.2.4.1.0	Minimum conformance requirements

6.5A.2.4.1.0.1	NR ACLR for intra-band contiguous CA

For intra-band contiguous carrier aggregation the carrier aggregation the Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent aggregated channel bandwidth at nominal channel spacing. The assigned aggregated channel bandwidth power and adjacent aggregated channel bandwidth power are measured with rectangular filters with measurement bandwidths specified in Table 6.5A.2.4.1.0.1-1. If the measured adjacent channel power is greater than –50dBm then the NRACLR shall be higher than the value specified in Table 6.5A.2.4.1.0.1-1.

Table 6.5A.2.4.1.0.1-1: General requirements for intra-band contiguous CA ACLR

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 30 dB |
| CA Measurement bandwidth (NOTE 1) | Nominal channel space+MBWACLR,low/2+ MBWACLR,high/2 |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |
| Difference between ACLR MBW center and Fc,low | MBWshift= (MBWACLR_CA-MBWACLR,low)/2 |
| NOTE 1: MBWACLR,low and MBWACLR,high are the single-channel ACLR measurement bandwidths specified for channel bandwidths BWchannel(low) and BWchannel(high) in 6.5.2.4.1.3, respectively. |  |

6.5A.2.4.1.0.2	NR ACLR for intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation, CA Adjacent Channel Leakage power Ratio(CAACLR) is the ratio of the sum of the filtered mean power centred on each assigned channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing. In case the gap bandwidth Wgap between 2 uplink CCs is smaller than maximum of the 2 uplink channel bandwidths then no CAACLR requirement is set for the gap. Each assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1.3-1. If the measured adjacent channel power is greater than –50dBm then the ACLR shall be higher than the value specified in Table 6.5A.2.4.1.0.2-1.

Table 6.5A.2.4.1.0.2-1: General requirements for intra-band non-contiguous CA ACLR

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 30 dB |
| CA Measurement bandwidth for each sub block (NOTE 1) | MBWACLR |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel / - BWChannel |
| NOTE 1: MBWACLR is the single-channel ACLR measurement bandwidths specified in 6.5.2.4.1.3. |  |

When the signalling is absent for dualPA-Architecture IE, if carrier leakage or I/Q image lands inside the gap spectrum between 2 UL CCs when UL CCs are synchronized with frequencies in the gap , exception to the ACLR requirement with 3dB relaxation applies.

6.5A.2.4.1.0.3	NR ACLR for Inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the NR Adjacent Channel Leakage power Ratio (NRACLR) is defined per component carrier while both component carriers are active and the requirement is specified in clause 6.5.2.4.1.3.

6.5A.2.4.1.1	NR ACLR for CA (2UL CA)

Editor’s Note: The following aspects are not yet determined:

-	PC1 and PC4 requirements are missing in TS 38.101-1 [2].

-	MU and TT for aggregate BW>100MHz are FFS.

6.5A.2.4.1.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for 2UL CA.

6.5A.2.4.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.2.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.2.4.1.0.

6.5A.2.4.1.1.4	Test description

6.5A.2.4.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.2.4.1.1.4.1-1 through Table 6.5A.2.4.1.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5A.2.4.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest for both PCC and SCC Highest for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  | Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
|  |  |  |  | PCC | SCC |
| 13 | Low | N/A | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 23 | High |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 33 | Default |  | DFT-s-OFDM PI/2 BPSK | Outer_Full | Outer_Full |
| 4 | Default |  | DFT-s-OFDM PI/2 BPSK | Inner_Full | Inner_Full |
| 5 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 6 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 7 | Default |  | DFT-s-OFDM QPSK | Outer_Full | Outer_Full |
| 8 | Default |  | DFT-s-OFDM QPSK | Inner_Full | Inner_Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner_Full | Inner_Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer_Full | Outer_Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer_Full | Outer_Full |
| 16 | Default |  | DFT-s-OFDM 256 QAM | Outer_Full | Outer_Full |
| 17 | Default |  | CP-OFDM QPSK | Inner_Full | Inner_Full |
| 18 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 19 | High |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 20 | Default |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| 21 | Default |  | CP-OFDM 16 QAM | Inner_Full | Inner_Full |
| 22 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 16 QAM | Outer_Full | Outer_Full |
| 25 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 64 QAM | Outer_Full | Outer_Full |
| 28 | Default |  | CP-OFDM 256 QAM | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

Table 6.5A.2.4.1.1.4.1-2: Intra band contiguous CA Test Configuration Table (contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A | DFT-s-OFDM |  | Pi/2 BPSK | Inner Full |
| 2 |  |  |  | Pi/2 BPSK | Outer Full |
| 3 |  |  |  | QPSK | Inner Full |
| 4 |  |  |  | QPSK | Outer Full |
| 5 |  |  |  | 16QAM | Inner Full |
| 6 |  |  |  | 16QAM | Outer Full |
| 7 |  |  |  | 64QAM | Inner Full |
| 8 |  |  |  | 64QAM | Outer Full |
| 9 |  |  |  | 256QAM | Inner Full |
| 10 |  |  |  | 256QAM | Outer Full |
| 11 |  | CP-OFDM |  | QPSK | Inner Full |
| 12 |  |  |  | QPSK | Outer Full |
| 13 |  |  |  | 16QAM | Inner Full |
| 14 |  |  |  | 16QAM | Outer Full |
| 15 |  |  |  | 64QAM | Inner Full |
| 16 |  |  |  | 64QAM | Outer Full |
| 17 |  |  |  | 256QAM | Inner Full |
| 18 |  |  |  | 256QAM | Outer Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 6.5A.2.4.1.1.4.1-3:

Intra-band non-contiguous CA Test Configuration Table (MPRIM3 to meet -13dBm/MHz) when UE supporting IE dualPA-Architecture

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  |  |  |  | Low range High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL config for PCC & SCC | UL configuration |  |  |  |  |  |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  | RB allocation LCRB@RBSTART (NOTE 2) |  |  |  |  |  |  |
|  |  |  |  | SCS 15 kHz |  |  | SCS 30 kHz |  | SCS 60 kHz |  |
|  |  |  |  | PCC | SCC |  | PCC | SCC | PCC | SCC |
| 1 | N/A | DFT-s-OFDM | Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 2 |  |  | Pi/2 BPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 3 |  |  | Pi/2 BPSK | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 4 |  |  | Pi/2 BPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 53 |  |  | Pi/2 BPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 64 |  |  | Pi/2 BPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 75 |  |  | Pi/2 BPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 85 |  |  | Pi/2 BPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 9 |  |  | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 10 |  |  | QPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 11 |  |  | QPSK | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 12 |  |  | QPSK | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 133 |  |  | QPSK | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 144 |  |  | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 155 |  |  | QPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 165 |  |  | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 17 |  |  | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 18 |  |  | 16QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 19 |  |  | 16QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 20 |  |  | 16QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 213 |  |  | 16QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 224 |  |  | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 235 |  |  | 16QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 245 |  |  | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 25 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 26 |  |  | 64QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 27 |  |  | 64QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 28 |  |  | 64QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 293 |  |  | 64QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 304 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 315 |  |  | 64QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 325 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 33 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 34 |  |  | 256QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 35 |  |  | 256QAM | 24@0 | 24@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 36 |  |  | 256QAM | 45@0 | 45@0 |  | 24@0 | 24@0 | 12@0 | 12@0 |
| 373 |  |  | 256QAM | 75@0 | 75@0 |  | 40@0 | 40@0 | 20@0 | 20@0 |
| 384 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 395 |  |  | 256QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 40@0 | 40@0 |
| 405 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 41 |  | CP-OFDM | QPSK | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 42 |  |  | QPSK | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 43 |  |  | QPSK | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 44 |  |  | QPSK | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 453 |  |  | QPSK | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 464 |  |  | QPSK | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 475 |  |  | QPSK | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 485 |  |  | QPSK | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 49 |  |  | 16QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 50 |  |  | 16QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 51 |  |  | 16QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 52 |  |  | 16QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 533 |  |  | 16QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 544 |  |  | 16QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 555 |  |  | 16QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 565 |  |  | 16QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 57 |  |  | 64QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 58 |  |  | 64QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 59 |  |  | 64QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 60 |  |  | 64QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 613 |  |  | 64QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 624 |  |  | 64QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 635 |  |  | 64QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 645 |  |  | 64QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| 65 |  |  | 256QAM | Edge_1RB_Left | Edge_1RB_Right |  | Edge_1RB_Left | Edge_1RB_Right | Edge_1RB_Left | Edge_1RB_Right |
| 66 |  |  | 256QAM | 8@0 | 8@0 |  | 4@0 | 4@0 | 2@0 | 2@0 |
| 67 |  |  | 256QAM | 23@0 | 23@0 |  | 12@0 | 12@0 | 6@0 | 6@0 |
| 68 |  |  | 256QAM | 45@0 | 45@0 |  | 23@0 | 23@0 | 12@0 | 12@0 |
| 693 |  |  | 256QAM | 75@0 | 75@0 |  | 38@0 | 38@0 | 19@0 | 19@0 |
| 704 |  |  | 256QAM | 120@0 | 120@0 |  | 60@0 | 60@0 | 30@0 | 30@0 |
| 715 |  |  | 256QAM | 150@0 | 150@0 |  | 75@0 | 75@0 | 38@0 | 38@0 |
| 725 |  |  | 256QAM | Outer_Full | Outer_Full |  | Outer_Full | Outer_Full | Outer_Full | Outer_Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1 NOTE 2: Edge_1RB_Left, Edge_1RB_Right and Outer_Full RB allocation are specified in table 6.1-1 for each component carrier. NOTE 3: Only being tested when the channel bandwidth of both carriers are larger than 10 MHz NOTE 4: Only being tested when the channel bandwidth of both carriers are larger than 20 MHz NOTE 5: Only being tested when the channel bandwidth of both carriers are larger than 25 MHz. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.5A.2.4.1.1.4.1-4: Intra-band non-contiguous CA Test Configuration Table (MPRIM3 to meet -30dBm/MHz) when UE supporting IE dualPA-Architecture

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  |  |  |  |  |  |  |  |  |  | Low range High range |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  |  |  |  |  |  |  | Lowest, Highest |  |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID |  | DL config for PCC & SCC |  | UL configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Modulations for all CCs (NOTE 2) |  |  |  |  |  | RB allocation LCRB@RBSTART (NOTE 2) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | SCS 15 kHz |  |  |  | SCS 30 kHz |  |  | SCS 60 kHz |  |
|  |  |  |  | PCC | SCC |  | PCC | SCC | PCC | SCC |
| 1 |  | N/A |  | DFT-s-OFDM |  |  | Pi/2 BPSK | Edge_1RB_Left |  |  | Edge_1RB_Right |  | Edge_1RB_Left |  | Edge_1RB_Right | Edge_1RB_Left |  | Edge_1RB_Right |
| 2 |  |  |  |  |  |  | Pi/2 BPSK | 15@0 |  |  | 15@0 |  | 8@0 |  | 8@0 | 4@0 |  | 4@0 |
| 3 |  |  |  |  |  |  | Pi/2 BPSK | 45@0 |  |  | 45@0 |  | 24@0 |  | 24@0 | 12@0 |  | 12@0 |
| 43 |  |  |  |  |  |  | Pi/2 BPSK | 75@0 |  |  | 75@0 |  | 40@0 |  | 40@0 | 20@0 |  | 20@0 |
| 54 |  |  |  |  |  |  | Pi/2 BPSK | 120@0 |  |  | 120@0 |  | 60@0 |  | 60@0 | 30@0 |  | 30@0 |
| 65 |  |  |  |  |  |  | Pi/2 BPSK | 216@0 |  |  | 216@0 |  | 108@0 |  | 108@0 | 54@0 |  | 54@0 |
| 75 |  |  |  |  |  |  | Pi/2 BPSK | Outer_Full |  |  | Outer_Full |  | Outer_Full |  | Outer_Full | Outer_Full |  | Outer_Full |
| 8 |  |  |  |  |  |  | QPSK | Edge_1RB_Left |  |  | Edge_1RB_Right |  | Edge_1RB_Left |  | Edge_1RB_Right | Edge_1RB_Left |  | Edge_1RB_Right |
| 9 |  |  |  |  |  |  | QPSK | 15@0 |  |  | 15@0 |  | 8@0 |  | 8@0 | 4@0 |  | 4@0 |
| 10 |  |  |  |  |  |  | QPSK | 45@0 |  |  | 45@0 |  | 24@0 |  | 24@0 | 12@0 |  | 12@0 |
| 113 |  |  |  |  |  |  | QPSK | 75@0 |  |  | 75@0 |  | 40@0 |  | 40@0 | 20@0 |  | 20@0 |
| 124 |  |  |  |  |  |  | QPSK | 120@0 |  |  | 120@0 |  | 60@0 |  | 60@0 | 30@0 |  | 30@0 |
| 135 |  |  |  |  |  |  | QPSK | 216@0 |  |  | 216@0 |  | 108@0 |  | 108@0 | 54@0 |  | 54@0 |
| 145 |  |  |  |  |  |  | QPSK | Outer_Full |  |  | Outer_Full |  | Outer_Full |  | Outer_Full | Outer_Full |  | Outer_Full |
| 15 |  |  |  |  |  |  | 16QAM | Edge_1RB_Left |  |  | Edge_1RB_Right |  | Edge_1RB_Left |  | Edge_1RB_Right | Edge_1RB_Left |  | Edge_1RB_Right |
| 16 |  |  |  |  |  |  | 16QAM | 15@0 |  |  | 15@0 |  | 8@0 |  | 8@0 | 4@0 |  | 4@0 |
| 17 |  |  |  |  |  |  | 16QAM | 45@0 |  |  | 45@0 |  | 24@0 |  | 24@0 | 12@0 |  | 12@0 |
| 183 |  |  |  |  |  |  | 16QAM | 75@0 |  |  | 75@0 |  | 40@0 |  | 40@0 | 20@0 |  | 20@0 |
| 194 |  |  |  |  |  |  | 16QAM | 120@0 |  |  | 120@0 |  | 60@0 |  | 60@0 | 30@0 |  | 30@0 |
| 205 |  |  |  |  |  |  | 16QAM | 216@0 |  |  | 216@0 |  | 108@0 |  | 108@0 | 54@0 |  | 54@0 |
| 215 |  |  |  |  |  |  | 16QAM | Outer_Full |  |  | Outer_Full |  | Outer_Full |  | Outer_Full | Outer_Full |  | Outer_Full |
| 22 |  |  |  |  |  |  | 64QAM | Edge_1RB_Left |  |  | Edge_1RB_Right |  | Edge_1RB_Left |  | Edge_1RB_Right | Edge_1RB_Left |  | Edge_1RB_Right |
| 23 |  |  |  |  |  |  | 64QAM | 15@0 |  |  | 15@0 |  | 8@0 |  | 8@0 | 4@0 |  | 4@0 |
| 24 |  |  |  |  |  |  | 64QAM | 45@0 |  |  | 45@0 |  | 24@0 |  | 24@0 | 12@0 |  | 12@0 |
| 253 |  |  |  |  |  |  | 64QAM | 75@0 |  |  | 75@0 |  | 40@0 |  | 40@0 | 20@0 |  | 20@0 |
| 264 |  |  |  |  |  |  | 64QAM | 120@0 |  |  | 120@0 |  | 60@0 |  | 60@0 | 30@0 |  | 30@0 |
| 275 |  |  |  |  |  |  | 64QAM | 216@0 |  |  | 216@0 |  | 108@0 |  | 108@0 | 54@0 |  | 54@0 |
| 285 |  |  |  |  |  |  | 64QAM | Outer_Full |  |  | Outer_Full |  | Outer_Full |  | Outer_Full | Outer_Full |  | Outer_Full |
| 29 |  |  |  |  |  |  | 256QAM | Edge_1RB_Left |  |  | Edge_1RB_Right |  | Edge_1RB_Left |  | Edge_1RB_Right | Edge_1RB_Left |  | Edge_1RB_Right |
| 30 |  |  |  |  |  |  | 256QAM | 15@0 |  |  | 15@0 |  | 8@0 |  | 8@0 | 4@0 |  | 4@0 |
| 31 |  |  |  |  |  |  | 256QAM | 45@0 |  |  | 45@0 |  | 24@0 |  | 24@0 | 12@0 |  | 12@0 |
| 323 |  |  |  |  |  |  | 256QAM | 75@0 |  |  | 75@0 |  | 40@0 |  | 40@0 | 20@0 |  | 20@0 |
| 334 |  |  |  |  |  |  | 256QAM | 120@0 |  |  | 120@0 |  | 60@0 |  | 60@0 | 30@0 |  | 30@0 |
| 345 |  |  |  |  |  |  | 256QAM | 216@0 |  |  | 216@0 |  | 108@0 |  | 108@0 | 54@0 |  | 54@0 |
| 355 |  |  |  |  |  |  | 256QAM | Outer_Full |  |  | Outer_Full |  | Outer_Full |  | Outer_Full | Outer_Full |  | Outer_Full |
| 36 |  |  |  | CP-OFDM |  |  | QPSK | Edge_1RB_Left |  |  | Edge_1RB_Right |  | Edge_1RB_Left |  | Edge_1RB_Right | Edge_1RB_Left |  | Edge_1RB_Right |
| 37 |  |  |  |  |  |  | QPSK | 15@0 |  |  | 15@0 |  | 8@0 |  | 8@0 | 4@0 |  | 4@0 |
| 38 |  |  |  |  |  |  | QPSK | 45@0 |  |  | 45@0 |  | 23@0 |  | 23@0 | 12@0 |  | 12@0 |
| 393 |  |  |  |  |  |  | QPSK | 75@0 |  |  | 75@0 |  | 38@0 |  | 38@0 | 19@0 |  | 19@0 |
| 404 |  |  |  |  |  |  | QPSK | 115@0 |  |  | 115@0 |  | 58@0 |  | 58@0 | 29@0 |  | 29@0 |
| 415 |  |  |  |  |  |  | QPSK | 210@0 |  |  | 210@0 |  | 105@0 |  | 105@0 | 53@0 |  | 53@0 |
| 425 |  |  |  |  |  |  | QPSK | Outer_Full |  |  | Outer_Full |  | Outer_Full |  | Outer_Full | Outer_Full |  | Outer_Full |
| 43 |  |  |  |  |  |  | 16QAM | Edge_1RB_Left |  |  | Edge_1RB_Right |  | Edge_1RB_Left |  | Edge_1RB_Right | Edge_1RB_Left |  | Edge_1RB_Right |
| 44 |  |  |  |  |  |  | 16QAM | 15@0 |  |  | 15@0 |  | 8@0 |  | 8@0 | 4@0 |  | 4@0 |
| 45 |  |  |  |  |  |  | 16QAM | 45@0 |  |  | 45@0 |  | 23@0 |  | 23@0 | 12@0 |  | 12@0 |
| 463 |  |  |  |  |  |  | 16QAM | 75@0 |  |  | 75@0 |  | 38@0 |  | 38@0 | 19@0 |  | 19@0 |
| 474 |  |  |  |  |  |  | 16QAM | 115@0 |  |  | 115@0 |  | 58@0 |  | 58@0 | 29@0 |  | 29@0 |
| 485 |  |  |  |  |  |  | 16QAM | 210@0 |  |  | 210@0 |  | 105@0 |  | 105@0 | 53@0 |  | 53@0 |
| 495 |  |  |  |  |  |  | 16QAM | Outer_Full |  |  | Outer_Full |  | Outer_Full |  | Outer_Full | Outer_Full |  | Outer_Full |
| 50 |  |  |  |  |  |  | 64QAM | Edge_1RB_Left |  |  | Edge_1RB_Right |  | Edge_1RB_Left |  | Edge_1RB_Right | Edge_1RB_Left |  | Edge_1RB_Right |
| 51 |  |  |  |  |  |  | 64QAM | 15@0 |  |  | 15@0 |  | 8@0 |  | 8@0 | 4@0 |  | 4@0 |
| 52 |  |  |  |  |  |  | 64QAM | 45@0 |  |  | 45@0 |  | 23@0 |  | 23@0 | 12@0 |  | 12@0 |
| 533 |  |  |  |  |  |  | 64QAM | 75@0 |  |  | 75@0 |  | 38@0 |  | 38@0 | 19@0 |  | 19@0 |
| 544 |  |  |  |  |  |  | 64QAM | 115@0 |  |  | 115@0 |  | 58@0 |  | 58@0 | 29@0 |  | 29@0 |
| 555 |  |  |  |  |  |  | 64QAM | 210@0 |  |  | 210@0 |  | 105@0 |  | 105@0 | 53@0 |  | 53@0 |
| 565 |  |  |  |  |  |  | 64QAM | Outer_Full |  |  | Outer_Full |  | Outer_Full |  | Outer_Full | Outer_Full |  | Outer_Full |
| 57 |  |  |  |  |  |  | 256QAM | Edge_1RB_Left |  |  | Edge_1RB_Right |  | Edge_1RB_Left |  | Edge_1RB_Right | Edge_1RB_Left |  | Edge_1RB_Right |
| 58 |  |  |  |  |  |  | 256QAM | 15@0 |  |  | 15@0 |  | 8@0 |  | 8@0 | 4@0 |  | 4@0 |
| 59 |  |  |  |  |  |  | 256QAM | 45@0 |  |  | 45@0 |  | 23@0 |  | 23@0 | 12@0 |  | 12@0 |
| 603 |  |  |  |  |  |  | 256QAM | 75@0 |  |  | 75@0 |  | 38@0 |  | 38@0 | 19@0 |  | 19@0 |
| 614 |  |  |  |  |  |  | 256QAM | 115@0 |  |  | 115@0 |  | 58@0 |  | 58@0 | 29@0 |  | 29@0 |
| 625 |  |  |  |  |  |  | 256QAM | 210@0 |  |  | 210@0 |  | 105@0 |  | 105@0 | 53@0 |  | 53@0 |
| 635 |  |  |  |  |  |  | 256QAM | Outer_Full |  |  | Outer_Full |  | Outer_Full |  | Outer_Full | Outer_Full |  | Outer_Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1 NOTE 2: Edge_1RB_Left, Edge_1RB_Right and Outer_Full RB allocation are specified in table 6.1-1 for each component carrier. NOTE 3: Only being tested when the channel bandwidth of both carriers are larger than 10 MHz NOTE 4: Only being tested when the channel bandwidth of both carriers are larger than 20 MHz NOTE 5: Only being tested when the channel bandwidth of both carriers are larger than 25 MHz. NOTE 6: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.2.4.1.1.4.1-1 through Table 6.5A.2.4.1.1.4.1-4.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.2.4.1.1.4.3

6.5A.2.4.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.2.4.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.2.4.1.1.4.1-1 through Table 6.5A.2.4.1.1.4.1-3 on both PCC and SCC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test.

7.	For intra-band contiguous 2UL CA UE, execute 7a) to 7c):

7a)	Measure the rectangular filtered mean power for the assigned NR aggregated channel bandwidth.

7b)	Measure the rectangular filtered mean power of the first NR adjacent aggregated channel on both lower and upper side of the assigned NR channel, respectively.

7c)	Calculate the ratios of the power between the values measured in step 7a) over step 7b) for lower and upper CA NRACLR, respectively.

8.	For intra-band non-contiguous 2UL CA UE, execute 8a) to 8e):

8a)	Measure the rectangular filtered mean power for the assigned NR channel on the lowest sub-block and highest sub-block respectively.

8b)	Measure the rectangular filtered mean power of the first NR adjacent channel on lower side of the lowest sub-block, and the higher side of the highest sub-block (out of gap measurement).

8c)		If Wgap is larger or equeal to either of the sub-block bandwidths, perform this step, otherwise continue to next step. Measure the rectangular filtered mean power of the first NR adjacent channel on higher side of the lowest sub-block, and/or the lower side of the highest sub-block (in gap measurement).

8d)	Calculate the ratio of the power between the values measured in step 8a) over step 8b) for CA NRACLR (out of gap measurement).

8e)	Calculate the ratio of the power between the values measured in step 8a) over step 8c) for CA NRACLR (in gap measurement).

9.	For inter-band 2UL CA UE, execute 9a) to 9f):

9a)	Measure the rectangular filtered mean power for the assigned NR channel on PCC.

9b)	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR channel on PCC, respectively.

9c)	Calculate the ratios of the power between the values measured in step 7 over step 8 for lower and upper NR ACLR, respectively.

9d)	Measure the rectangular filtered mean power for the assigned NR channel on SCC.

9e)	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR channel on SCC, respectively.

9f)	Calculate the ratios of the power between the values measured in step 9d over step 9e for lower and upper NR ACLR, respectively.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5A.2.4.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.2.4.1.1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA Test IDs 3-9, 12, 15, 17, 20, 21, 24 |
|  | 17 |  | Power class 3 and Inter-band CA Test IDs 10, 11, 13, 14, 27 |
|  | 16 |  | Power class 3 and Inter-band CA Test IDs 1, 2, 16, 18, 19, 22, 23, 25, 26 |
|  | 14 |  | Power class 3 and Inter-band CA Test IDs 28 |

6.5A.2.4.1.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 6, shall fulfil requirements in clause 6.2A.2.1 as appropriate, and if the measured adjacent channel power is greater than -50 dBm, then the measured NR ACLR derived in step 7c, step 8e, step 6c and step 9f, shall be higher than the limits in Table 6.5A.2.4.1.1.5-1, 6.5A.2.4.1.1.5-2 and 6.5A.2.4.1.1.5-3 respectively.

Table 6.5A.2.4.1.1.5-1: General requirements for intra-band contiguous CA ACLR

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 30 - TT dB |
| CA Measurement bandwidth (NOTE 1) | Nominal channel space+MBWACLR,low/2+ MBWACLR,high/2 |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |
| Difference between ACLR MBW center and Fc,low | MBWshift= (MBWACLR_CA-MBWACLR,low)/2 |
| NOTE 1: MBWACLR,low and MBWACLR,high are the single-channel ACLR measurement bandwidths specified for channel bandwidths BWchannel(low) and BWchannel(high) in 6.5.2.4.1, respectively. |  |

Table 6.5A.2.4.1.1.5-2: General requirements for intra-band non-contiguous CA ACLR

|  | ACLR / Measurement bandwidth |
| --- | --- |
| CA ACLR | 30 - TT dB |
| CA Measurement bandwidth for each sub block (NOTE 1) | MBWACLR |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel / - BWChannel |
| NOTE 1: MBWACLR is the single-channel ACLR measurement bandwidths specified in 6.5.2.4.1. |  |

Table 6.5A.2.4.1.1.5-3: General requirements for inter-band CA ACLR

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR |  | 31 - TT dB | 30 - TT dB |

Table 6.5A.2.4.1.1.5-4: NR ACLR measurement bandwidth for inter-band CA

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |

Table 6.5A.2.4.1.1.5-5: Test Tolerance for NR ACLR

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 100MHz | 0.8dB | 0.8dB |
| Aggregated BW ≤ 200MHz | FFS | FFS |

<End of Changes>