3GPP TSG-RAN5 Meeting #94-e

# Electronic Meeting, 21st Feb – 04th Mar. 2022

Source:        Huawei, Hisilicon, Samsung

Title:           TP analysis on FR1 Spectrum emission mask for CA (2UL CA)

Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Spectrum emission mask for CA (2UL CA) test case in FR1.

Discussion

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

Test Environment

In LTE, SEM for CA is tested under Normal condition. Hence it is proposed to apply normal condition as test environment to align with LTE case.

Proposal 1: Define test environment as Normal for SEM in FR1 for UL inter-band CA.

Test Subcarrier Spacing

In NR the smallest and highest SCS are selected for Single Carrier SEM verification. It is proposed to reuse the same way for CA.

Proposal 2: Select smallest and highest SCS for SEM in FR1 for CA.

Test Frequencies

In LTE, SEM for CA is tested for both Low range and High range. The same test coverage as for LTE would be enough to ensure that the SEM obeys the test requirement at low range and high range.

Proposal 3: Test SEM for Low range and High range frequencies in FR1 for UL CA.

Test Channel Bandwidth

In LTE, SEM for inter-band CA is tested for Lowest and Highest NRB_agg. Hence it proposed to reuse the LTE method for SEM test for inter-band UL CA.

Proposal 4: Test SEM for Lowest and Highest NRB_agg in FR1 for UL CA.

Test Modulations and waveforms

For single carrier TP analysis presented in [3] both DFT-s-OFDM and CP-OFDM for all modulation permutation are selected for testing. For CA case it’s proposed to reuse this configuration.

Proposal 5: Test SEM for CA with DFT-s-OFDM and CP-OFDM with all modulation permutation.

RB allocation

In LTE, SEM for CA are tested with edge partial allocation is located on the lower and upper channel edge for QPSK and 16QAM without test on 64QAM and 256QAM. For NR single carrier case, Outer_1RB_Left , Outer_1RB_Right and Outer_Full are selected in [3] . It’s proposed to verify the inter-band CA for SEM with Outer_1RB_Left and Outter_1RB_Right for QPSK and 16QAM. And Test on Outer_Full only for 64QAM and 256QAM.

Proposal 6: Test SEM for inter-band CA with Outer_1RB_Left and Outer_1RB_Right for PI/2 BPSK , QPSK and 16QAM and with Outer_Full for 64QAM and 256QAM.

For intra-band contiguous CA, in NR the SEM requirement for contiguous RB allocation and non-contiguous RB allocations is the same. In MPR for intra-band contiguous CA testing, Inner_Full RB allocation and Outer_Full RB allocation are tested. In SEM testing, select contiguous RB allocation with Outer_Full RB allocation will be enough.

As for intra-band non-contiguous CA, the most critical RB allocation is Edge_1RB_Left for PCC and Edge_1RB_Right for SCC.

Proposal 7: Test SEM for intra-band contiguous CA with Outer_Full RB allocation for contiguous RB allocation. Test SEM for intra-band non-contiguous CA with Edge_1RB_Left for PCC and Edge_1RB_Right for SCC.

Number of test points for CA SEM

|  | Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-band | 1 | 2 | 2 | 2 | 14 | 112 |
| Intra-band contiguous | 1 | 2 | 2 | 2 | 9 | 72 |
| Intra-band non-contiguous | 1 | 2 | 2 | 2 | 9 | 72 |

Conclusion

As the inter-band UL CA NR ACLR requirement is defined the same as single carrier requirement with the update of condition of 2UL and 2DL CA active. Hence the test condition is proposed in the same way of NR single

Proposal 1: Define test environment as Normal for SEM in FR1 for UL inter-band CA.

Proposal 2: Select smallest and highest SCS for SEM in FR1 for UL CA.

Proposal 3: Test SEM for Low range and High range frequencies in FR1 for UL CA.

Proposal 4: Test SEM for Lowest and Highest NRB_agg in FR1 for UL CA.

Proposal 5: Test SEM for CA with DFT-s-OFDM and CP-OFDM with all modulation permutation.

Proposal 6: Test SEM for inter-band CA with Outer_1RB_Left and Outer_1RB_Right for PI/2 BPSK , QPSK and 16QAM and with Outer_Full for 64QAM and 256QAM.

Proposal 7: Test SEM for intra-band contiguous CA with Outer_Full RB allocation for contiguous RB allocation. Test SEM for intra-band non-contiguous CA with Edge_1RB_Left for PCC and Edge_1RB_Right for SCC.

Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-180886, "Discussion on test point selection for NR SEM in FR1"

[4] 3GPP TS 36.521-1 v16.9.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v17.3.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 17).

[6] 3GPP TS 38.101-1 v16.10.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16).