3GPP TSG-RAN5 Meeting # 94-e	R5-221144

# Electronic Meeting, 21st Feb- 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR |  | 5400 |  |  |  |  | rev |  |  | - |  |  | Current version: |  |  |  |  |  |  | 17.1.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  | UICC apps |  |  |  |  |  |  | ME |  |  | Radio Access Network |  |  |  |  |  |  | Core Network |  |  |
| Title: |  |  | Update to maximum output power for UL CA Band combo 2A-46A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  |  | Qualcomm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  | R5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  | TEI14_test, LTE_CA_R14-UEConTest |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  | 2022-1-22 |  |  |  |  |  |
| Category: |  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |  |  |  |  |
| Reason for change: |  |  |  |  | Band CA 2A-46A is not updated for the maximum output power |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  | Updated band combo CA-2A-46A to the test case |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  | Band combo would be missing for UL maximum output power test case. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | Table 6.2.2A.2.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  |  |  |  | Other core specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| affected: |  |  |  |  |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  |  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.2.2A.2	UE Maximum Output Power for CA (inter-band DL CA and UL CA)

##### 6.2.2A.2.1	Test purpose

Same as in clause 6.2.2A.1.1, but for the inter-band carrier aggregation case.

##### 6.2.2A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

##### 6.2.2A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2.2A.0.

##### 6.2.2A.2.4	Test description

6.2.2A.2.4.1	Initial conditions

Same as in clause 6.2.2A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.2.2A.1.4.1-1 use Table 6.2.2A.2.4.1-1.

-	Instead of clause 6.2.2A.1.4.3 use clause 6.2.2A.2.4.3.

Table 6.2.2A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC (Note 3) |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@0 | S_8@0 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@1 | S_8@17 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@0 | S_12@0 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@1 | S_12@38 | - | - |
| 15 | 25 |  | QPSK | 2 | S_1@0 | P_1@0 |  |  |
| 15 | 25 |  | QPSK | 2 | S_1@14 | P_1@24 |  |  |
| 15 | 25 |  | QPSK | 13 | S_5@0 | P_8@0 |  |  |
| 15 | 25 |  | QPSK | 13 | S_5@10 | P_8@17 |  |  |
| 25 | 6 |  | QPSK | 2 | P_1@0 | P_1@0 |  |  |
| 25 | 6 |  | QPSK | 2 | P_1@24 | P_1@5 |  |  |
| 25 | 6 |  | QPSK | 13 | P_8@0 | P_5@0 |  |  |
| 25 | 6 |  | QPSK | 13 | P_8@17 | P_5@1 |  |  |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@0 | S_5@0 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@17 | S_5@10 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@17 | S_8@17 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@0 | S_12@0 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@17 | S_12@38 | - | - |
| 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 25 |  | QPSK | 2 | P_1@49 | S_1@24 | - | - |
| 50 | 25 |  | QPSK | 8 | P_12@0 | S_8@0 | - | - |
| 50 | 25 |  | QPSK | 8 | P_12@38 | S_8@17 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@38 | S_12@38 | - | - |
| 25 | 75 |  | QPSK | 2 | P_1@0 | P_1@0 |  |  |
| 25 | 75 |  | QPSK | 2 | P_1@24 | P_1@74 |  |  |
| 25 | 75 |  | QPSK | 24 | P_8@0 | P_16@0 |  |  |
| 25 | 75 |  | QPSK | 24 | P_8@17 | P_16@59 |  |  |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@0 | S_18@0 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@38 | S_18@82 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@59 | S_8@17 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@59 | S_16@59 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 | - | - |
| 100 | 50 |  | QPSK | 30 | P_18@0 | S_12@0 | - | - |
| 100 | 50 |  | QPSK | 30 | P_18@82 | S_12@38 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@82 | S_16@59 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@0 | S_18@0 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@82 | S_18@82 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 3: For Low range use only test points with RBstart=0. For High range use only test points with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |  |

6.2.2A.2.4.2	Test procedure

Same as in clause 6.2.2A.1.4.2 with the following exceptions:

-	Instead of Table 6.2.2A.1.4.1-1 use Table 6.2.2A.2.4.1-1.

-	Instead of clause 6.2.2A.1.4.3 use clause 6.2.2A.2.4.3.

6.2.2A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

##### 6.2.2A.2.5	Test Requirements

The maximum output power for the CA configuration, derived in step 6 shall be within the range prescribed by the CA UE Power Class and tolerance in Table 6.2.2A.2.5-1.

Table 6.2.2A.2.5-1: UE Power Class for dual uplink interband CA test requirements

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_1A-5A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_1A-7A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_1A-8A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_1A-18A |  |  |  |  | 23 | +2.7/-3.75 |  |  |
| CA_1A-19A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_1A-21A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_1A-26A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_1A-28A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_1A-42A |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| CA_2A-4A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_2A-5A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_2A-7A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_2A-12A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_2A-13A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_2A-30A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_2A-46A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_2A-66A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_3A-5A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_3A-7A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_3A-8A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_3A-18A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_3A-19A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_3A-20A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_3A-26A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_3A-28A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_3A-41A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_4A-5A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_4A-7A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_4A-12A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_4A-13A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_4A-17A |  |  |  |  | 23 | +2/.7-3.7 |  |  |
| CA_5A-7A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_5A-12A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_5A-17A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_5A-30A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_5A-66A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_7A-20A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_7A-28A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_11A-18A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_12A-30A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_12A-66A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_18A-28A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA_19A-21A |  |  |  |  | 23 | +2.7/-3.7 |  |  |
| CA 39A-41A |  |  |  |  | 23 | +2.7/-3.72 |  |  |
| CA_41A-42A |  |  |  |  | 23 | +3.0/-4.02 |  |  |
| NOTE 1: Void. NOTE 2: 2 refers to the transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c in Table 6.2.5.3-2 shall be applied for applicable bands. For the UE which supports an inter-band carrier aggregation configuration from Table 6.2.5.3-3 the ΔTIB,c in Table 6.2.5.3-3 shall be applied for applicable bands.

In case the UE supports more than one of the above inter-band carrier aggregation configurations and an E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 6.2.5.3-2, truncated to one decimal place for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 6.2.5.3-2 that applies for that operating band among the supported CA configurations.

In case the UE supports more than one of the inter-band carrier aggregation configurations from Table 6.2.5.3-3 and an E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied.

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations.

PAG