3GPP TSG-RAN WG5 #95-e		R5-221192
Electronic Meeting, 21st Feb – 4th March 2022

Agenda item:	5.3.1.17

Source:	Rohde & Schwarz

Title:	Update on RRM issue #8

Document for:	Endorsement

# Introduction

This contribution gives an update on the status of RRM Issue #8, captured in the following table from [1].

| # | Issue | Test types | Owner | RAN4 Status | RAN4 Target | RAN5 Status | RAN5 Tdoc# | RAN5 Target | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | Handling of 2 Angles of Arrival from 1 cell | RLM, TCI state switch | R&S | Needs research | RAN4 #101-e | Needs research |  | RAN5 #93-e | Define TDM? |
| … |  |  |  |  |  |  |  |  |  |

# Discussion

The issue occurs in test cases where 2SSBs of the same cell are transmitted from different direction. The question is how to configure the OCGN, AWGN and all other channels of the cell, except the SSBs? From which angle should they be transmitted?

The issue has been solved already for RLM test cases by the definition of a TDM model that determines what channels are transmitted from which AoA in every instant of time:

Figure 5.5.1.1.4-2: Time multiplexed downlink transmissions

As part of the analysis for this issue, following test cases where identified, that are affected by the same problem and need a correction in RAN4:

- 5.5.8.1	EN-DC FR2 MAC-CE based active TCI state switch

- 5.5.8.2	EN-DC FR2 RRC based active TCI state switch

A RAN4 CR [2] has been provided for providing a TDM model similar to the one in the Figure 5.5.1.1.4-2 for the test cases mentioned above. Our understanding is that this TDM model will solve the issue.

Observation 1: Known Issue #8 can be closed if [2] is endorsed by RAN4.

# Conclusion

Observation 1: Known Issue #8 can be closed if [2] is endorsed by RAN4.

# References

- R5-217564, “FR2 RRM test cases: Known Issue List”, Ericsson, RAN5#93-e

[2]	R4-2203596, “Draft CR to TS 38.133: Corrections to active TCI state switch test cases (Rel 15)”, Rohde & Schwarz, RAN4#102-e