3GPP TSG-WG5 Meeting #94-e	R5-221258

# Electronic Meeting, 21st Feb 2022 – 4th Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2854 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of testcase 7.1.3.5.7 Ethernet header compression and decompression / Correct functionality of ethernet header compression and decompression |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IioT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Removal of editors note if EHC test method changes are approved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of editors note. Refined test purpose section to 4 different test points with more detais instead of one general test point. Pre test conditions changed, primary DRB to use PDU type ethernet. Test procedure changed according to new test points and edited steps to be more clear. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EHC functionality testing method will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.7 Ethernet header compression and decompression / Correct functionality of ethernet header compression and decompression |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.508-1 CR 2286 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Start of changes -------------------------------------------------

------------------------------------------------- Skipped chapters -------------------------------------------------

##### 7.1.3.5.7	Ethernet header compression and decompression / Correct functionality of ethernet header compression and decompression

7.1.3.5.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { SS sends PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and with full ethernet header A to UE }

then { UE sends PDCP Control PDU to SS containing EHC feedback packet with Context ID A }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { SS sends PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and with full ethernet header A to UE }

then { UE sends PDCP Data PDU to SS containing EHC compressed ethernet packet generated from the looped back ethernet packet with Context ID B and with full ethernet header A }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { SS sends PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and without ethernet header to UE }

then { UE sends PDCP Data PDU to SS containing EHC compressed ethernet packet generated from the looped back ethernet packet with decompressed ethernet header with Context ID B and with full ethernet header A }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { SS has sent PDCP Control PDU to UE containing EHC feedback packet with Context ID B and SS sends PDCP Data PDU to UE containing EHC compressed ethernet packet with Context ID A without ethernet header to UE }

then { UE sends PDCP Data PDU to SS containing EHC compressed ethernet packet generated from the the looped back ethernet packet with decompressed ethernet header with Context ID B and without ethernet header  }

}

7.1.3.5.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.12. These are Rel-16 requirements.

[TS 38.323, clause 5.12.2]

PDCP entities associated with DRBs can be configured by upper layers TS 38.331 [3] to use EHC. Each PDCP entity carrying user plane data may be configured to use EHC. Every PDCP entity uses at most one EHC compressor instance and at most one EHC decompressor instance.

[TS 38.323, clause 5.12.3]

The usage and definition of the parameters shall be as specified below.

-	MAX_CID_EHC_UL: This is the maximum CID value that can be used for uplink. One CID value shall always be reserved for uncompressed flows. The parameter MAX_CID_EHC_UL is configured by upper layers (maxCID-EHC-UL in TS 38.331 [3]);

[TS 38.323, clause 5.12.4]

If EHC is configured, the EHC protocol generates two types of output packets:

-	EHC compressed packets (i.e. EHC full header packets and EHC compressed header packets), each associated with one PDCP SDU;

-	standalone packets not associated with a PDCP SDU, i.e. EHC feedback.

An EHC compressed packet is associated with the same PDCP SN and COUNT value as the related PDCP SDU. The header compression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP SDU.

EHC feedback are not associated with a PDCP SDU. They are not associated with a PDCP SN and are not ciphered.

[TS 38.323, clause 5.12.5]

If EHC is configured by upper layers for PDCP entities associated with user plane data, the PDCP Data PDUs are decompressed by the EHC protocol after performing deciphering as explained in clause 5.8. The header decompression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP Data PDU.

[TS 38.323, clause 5.12.6.1]

When an EHC feedback is generated by the EHC protocol, the transmitting PDCP entity shall:

-	submit to lower layers the corresponding PDCP Control PDU as specified in clause 6.2.3.3 i.e. without associating a PDCP SN, nor performing ciphering.

[TS 38.323, clause 5.12.6.2]

At reception of a PDCP Control PDU for EHC feedback from lower layers, the receiving PDCP entity shall:

-	deliver the corresponding EHC feedback to the EHC protocol without performing deciphering.

7.1.3.5.7.3	Test description

7.1.3.5.7.3.1	Pre-test conditions

Pre-test conditions as in clause 7.1.3.0 with condition pc_NG_RAN_NR, PDN/PDU session type Ethernet used in 4.5A.2 and message with condition UM used for step 7 in 4.5.4.2 according to [4] for primary DRB.

7.1.3.5.7.3.2	Test procedure sequence

Table 7.1.3.5.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure DRB to use ethernet header compression. | <-- | RRCReconfiguration | - | - |
| 2 | UE transmits RRCReconfigurationComplete message to confirm that reconfiguration with ethernet header compression configuration is completed. | --> | RRCReconfigurationComplete | - | - |
| 3 | SS transmits PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and with full ethernet header information A. | <-- | PDCP Data PDU #0 | - | - |
| 4 | CHECK: Does UE transmit PDCP Control PDU containing unciphered EHC feedback packet with Context ID A? | --> | PDCP Control PDU #0 | 1 | P |
| 5 | CHECK: Does UE transmit PDCP Data PDU containing EHC compressed ethernet packet with Context ID B and with full ethernet header information A? | --> | PDCP Data PDU #0 | 2 | P |
| 6 | SS transmits PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and without ethernet header information. | <-- | PDCP Data PDU #1 | - | - |
| 7 | CHECK: Does UE transmit PDCP Data PDU containing EHC compressed ethernet packet with Context ID B and with full ethernet header information A? | --> | PDCP Data PDU #1 | 3 | P |
| 8 | SS transmits PDCP Control PDU containing unciphered EHC feedback packet with Context ID B. | <-- | PDCP Control PDU #1 | - | - |
| 9 | SS transmits PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and without ethernet header information. | <-- | PDCP Data PDU #2 | - | - |
| 10 | CHECK: Does UE transmit PDCP Data PDU containing EHC compressed ethernet packet with Context ID B and without ethernet header information? | --> | PDCP Data PDU #2 | 4 | P |

7.1.3.5.7.3.3	Specific message contents

Table 7.1.3.5.7.3.3-1: RRCReconfiguration (step 1, Table 7.1.3.5.7.3.2-1)

| Derivation Path: TS 38.508 [4], clause 4.6.1, table 4.6.1-13 RRCReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig-EHC |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.5.7.3.3-2: RadioBearerConfig-EHC (Table 7.1.3.5.7.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity for DRB created in pre-test conditions |  |  |
| pdcp-Config | PDCP-Config-EHC |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.7.3.3-3: PDCP-Config-EHC

| Derivation Path: TS38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |  |
| ethernetHeaderCompression-r16 SEQUENCE { |  |  |  |  |
| ehc-Common-r16 SEQUENCE { |  |  |  |  |
| ehc-CID-Length-r16 |  | bits15 |  |  |
| } |  |  |  |  |
| ehc-Downlink-r16 SEQUENCE { |  |  |  |  |
| drb-ContinueEHC-DL-r16 |  | true |  |  |
| } |  |  |  |  |
| ehc-Uplink-r16 SEQUENCE { |  |  |  |  |
| maxCID-EHC-UL-r16 |  | 32767 |  |  |
| drb-ContinueEHC-UL-r16 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

### 7.1.4	SDAP

7.1.4.1	SDAP Data Transfer and PDU Header Handling UL/DL

------------------------------------------------- Skipped chapters -------------------------------------------------

------------------------------------------------- End of changes -------------------------------------------------