3GPP TSG-RAN5 Meeting #94-e	R5-221265

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2285 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of DMRS-DownlinkConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Table 5.4.2.0-24: DMRS-DownlinkConfig, dmrs-Type is specified with value Type 1 and maxLength with value len1. However, TS 38.331 clause 6.3.2 specifies that the UE uses DMRS type1 for dmrs-Type if the field is absent and that it applies the value len1 for maxLength if the field is not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Since Table 5.4.2.0-24 is derived from Tabel 4.6.3-50 and the dmrs-Type and maxLength are not altered by this message exception, the superfluous rows have been removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specificazion will remain incorrect resulting in non-conformant signaling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 5.4.2.0	Parameters common to all Demod and CSI tests

< Unchanged tables omitted >

##### PDSCH DMRS Configuration

###### DMRS-DownlinkConfig

Table 5.4.2.0-24: DMRS-DownlinkConfig

| Derivation Path: Table 4.6.3-50 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos1 |  |  |
| phaseTrackingRS | Not present |  | DEMOD_FR1 |
| phaseTrackingRS SEQUENCE { |  |  | DEMOD_FR2 |
| epre-Ratio | 0 |  |  |
| resourceElementOffset | Offset10 |  |  |
| } |  |  |  |
| } |  |  |  |

< End of changes >