3GPP TSG-RAN5 Meeting #94-e                                                    R5-221278

Electronic Meeting, 8th – 19th November 2021

Title: 	Discussion on LTE_NR_B41_Bn41_PC29dBm-UEConTest Work plan review to include dependencies with NR_RF_TxD-UEConTest work plan

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.29.4

Document for:	Discussion and Endorsement

# 1.	Introduction

RAN5 has been working in Work item for UE conformance Test Aspects – 29 dBm UE Power Class for LTE Band 41 and NR Band n41 for several meetings already and its status reports indicates that the completion level reached is 61% with a target completion date in Jun’22.

Having a look to core requirements, it can be seen that such power class is achieved via dual Tx:

It implies that UL MIMO and Tx diversity core requirements should be tested as part of the on LTE_NR_B41_Bn41_PC29dBm-UEConTest, depending on UE capabilities.

RAN4 and RAN5 are currently working on work items for UE RF requirements for Transparent Tx Diversity (TxD) for NR ([3], [4]).

This document makes an analysis on the dependencies between the work plan for these 2 features and makes a proposal for reviewing UE conformance Test Aspects – 29 dBm UE Power Class for LTE Band 41 and NR Band n41 work plan.

# 2.	Discussion

In [5], some analysis is performed regarding the details to be defined in the work plan for Tx diversity feature and proposes to define new test cases with new clause suffix G in test specifications aligned with core requirements for Tx Diversity:

  - UE maximum output power

  - MPR

  - Configured transmitted power

  - Minimum output power

  - Transmit OFF power

  - Transmit ON/OFF time mask

  - Power Control

  - Transmit signal quality

  - Out of band emission

  - ACLR

The definition of the new Tx Diversity test cases, if proposals in [5] are endorsed, require that some of the changes performed in the test specification for LTE_NR_B41_Bn41_PC29dBm-UEConTest should be revisited to update the proper test cases.  These changes should be included in LTE_NR_B41_Bn41_PC29dBm work plan.

There should be no conflict on Release applicability as power class 1.5 functionality has been defined in 38.307 as release independent from Rel-15, same as current understanding for Tx Diversity:

Proposal 1: Review LTE_NR_B41_Bn41_PC29dBm work plan to align to Tx Diversity work plan and review Work item completion in Status Report accordingly.

# 3. Conclusion

This document makes an analysis on the dependencies between the 29 dBm UE Power Class for LTE Band 41 and NR Band n41and Tx Diversity work plans and makes the following proposal:

Proposal 1: Review LTE_NR_B41_Bn41_PC29dBm work plan to align to Tx Diversity work plan and review Work item completion in Status Report accordingly.

# 4.	References

[1] RP-211443, “New WID on UE Conformance Test Aspects - 29 dBm UE Power Class for LTE Band 41and NR Band n41”, T-Mobile US, CMCC, RAN#91e.

[2] RP-212807, “Status report for UE Conformance Test Aspects - 29 dBm UE Power Class for LTE Band 41and NR Band n41”, T-Mobile US, CMCC, RAN#94e.

[3] RP-213229, “New WID on UE Conformance – UE RF requirements for Transparent diversity (TxD) for NR”, Huawei, HiSilicon, RAN5#93e.

[4] RP-211940, “WID on UE RF requirements for Transparent Tx Diversity (TxD) for NR”, Qualcomm, RAN#93e.

[5] R5-221281, “Discussion on Transparent Tx diversity work plan”, Keysight Technologies UK Ltd, RAN5#94e.