Title:	Discussion on test point selection for FR2 Spurious emission UL MIMO test case

Document for:	Discussion and approval

# Introduction

This contribution provides test point analysis for FR2 Spurious emission UL MIMO test cases 6.5D.3.1 (Transmitter Spurious emission for UL MIMO), 6.5D.3.2 (Spurious emission band UE coex for UL MIMO) and 6.5D.3.3 (Additional spurious emission for UL MIMO) respectively.

# Discussion

In NR specification TS 38.521-2, the Test configuration for transmitter spurious emission for single carrier mode is shown in table below for reference.

Table 6.5.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s -OFDM QPSK | Inner_Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s -OFDM QPSK | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

Observation: The test points agreed for FR2 single carrier spurious emission test cases 6.5.3.1, 6.5.3.2 and 6.5.3.3 also apply for FR2 spurious emission UL MIMO test cases 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 respectively. The worst case configurations remain the same irrespective of single carrier or MIMO modes.

## Test Environment

It is proposed to reuse the FR2 spurious emission single carrier test environment agreed in test cases 6.5.3.1, 6.5.3.2 and 6.5.3.3 for FR2 spurious emission UL MIMO test cases 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 respectively.

Proposal 1: Define test environment as NC for FR2 spurious emission UL MIMO test cases in 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3.

## Test Subcarrier Spacing

It is proposed to reuse the FR2 spurious emission single carrier SCS agreed in test cases 6.5.3.1, 6.5.3.2 and 6.5.3.3 for FR2 spurious emission UL MIMO test cases 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 respectively. The SCS has the same effect on spurious emission profile irrespective signal carrier or MIMO. Testing only 120kHz SCS provides necessary coverage.

Proposal 2: Select 120kHz SCS for FR2 spurious emission UL MIMO test cases in 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3.

## Test Frequencies

It is proposed to reuse the FR2 spurious emission single carrier test frequencies agreed in test cases 6.5.3.1, 6.5.3.2 and 6.5.3.3 for FR2 spurious emission UL MIMO test cases 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 respectively.

Proposal 3: Test for Low range and/or High range frequencies for FR2 spurious emission UL MIMO test cases in 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3.

## Test Channel Bandwidth

It is proposed to test only highest bandwidth as this is worst case and in line with configuration chosen for all other FR2  spur emission test cases.

Proposal 4: Test for Highest bandwidth for FR2 spurious emission UL MIMO test cases 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 respectively.

## Test Modulations and waveforms

It is proposed to reuse the FR2 spurious emission single carrier modulation and waveforms for FR2 spurious emission UL MIMO as they remain the worst case irrespective of single carrier or MIMO. The worst-case modulation and waveform does not change for single carrier and MIMO modes and the worst case config remains the same.

Proposal 5: Test for DFT-s-OFDM QPSK in for FR2 spurious emission UL MIMO test cases 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 respectively.

## RB allocation

Although no 0dB MPR config is defined for UL MIMO, it is prudent to do test with the lowest MPR RB allocations which remain the same for single carrier and MIMO.

Proposal 6: For transmitter spurious emission and coex UL MIMO, test Inner Full and Inner 1RB as agreed for single carrier test cases 6.5.3.1, 6.5.3.2 as they are applicable to UL MIMO test cases 6.5D.3.1 and 6.5D.3.2 respectively. Further for FR2 additional spurious emission UL MIMO test cases, test Outer Full, Inner Full and Inner 1RB/Partial as applicable and as agreed for single carrier test case 6.5.3.3 as they are applicable for additional spur UL MIMO test case 6.5D.3.3.

# 2.7 Number of test points each for 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 UL MIMO spurious emission test cases

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 1 | 1 | 2 | 4 |

# Conclusion

Observation: The test points agreed for FR2 single carrier spurious emission test cases 6.5.3.1, 6.5.3.2 and 6.5.3.3 also apply for FR2 spurious emission UL MIMO test cases 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 respectively. The worst case configurations remain the same irrespective of single carrier or MIMO modes.

Proposal 1: Define test environment as NC for FR2 spurious emission UL MIMO test cases in 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3.

Proposal 2: Select 120kHz SCS for FR2 spurious emission UL MIMO test cases in 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3.

Proposal 3: Test for Low range and/or High range frequencies for FR2 spurious emission UL MIMO test cases in 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3.

Proposal 4: Test for Highest bandwidth for FR2 spurious emission UL MIMO test cases 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 respectively.

Proposal 5: Test for DFT-s-OFDM QPSK in for FR2 spurious emission UL MIMO test cases 6.5D.3.1, 6.5D.3.2 and 6.5D.3.3 respectively.

Proposal 6: For transmitter spurious emission and coex UL MIMO, test Inner Full and Inner 1RB as agreed for single carrier test cases 6.5.3.1, 6.5.3.2 as they are applicable to UL MIMO test cases 6.5D.3.1 and 6.5D.3.2 respectively. Further for FR2 additional spurious emission UL MIMO test cases, test Outer Full, Inner Full and Inner 1RB/Partial as applicable and as agreed for single carrier test case 6.5.3.3 as they are applicable for additional spur UL MIMO test case 6.5D.3.3.