3GPP TSG-RAN5 Meeting #94-e	R5-221349

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2288 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new messages and procedure for test function to limit Pcell Power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the outline of necessary messages and procedures to allow limiting of PCell power via a conformance test function |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test function messages added to limit PCell power in UL-CA test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCell drop in UL-CA tests due to expected power prioritization rules |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Sections skipped>

#### NSSAI DELETE RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

Table 4.7A.6-2: NSSAI DELETE RESPONSE

| Derivation Path: 38.509 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 1 1 1 |  |  |

### 4.7A.7	PCELL Power Limit Messages

#### -	PCELL POWER LIMIT REQUEST

TBD

#### PCELL POWER LIMIT RESPONSE

TBD

<End of changes>