3GPP TSG-RAN5 Meeting #94-e	R5-221366

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1746 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce Test case for LTE PSCell addition and release in NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has introduced NE-DC test case for LTE PSCell addition in TS 38.133. Introduction of this test in RAN5 test specification aligns with RAN4 core requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce initial test case structure in new cluase |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing conformance test case for LTE PSCell addition and release in NE-DC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Unchanged Sections Skipped>

# 4A	NE-DC with all NR cells in FR1

This clause contains test scenarios for NR and E-UTRA dual connectivity with NR as PCell and E-UTRA and PSCell. This configuration is also known as NSA Option 4 and 4a. All NR cells are in Frequency Range 1.

### <Unchanged Sections Skipped>

### 4A.5.1	E-UTRA PSCell addition and release delay

#### 4A.5.1.0	Minimum conformance requirements

##### 4A.5.1.0.1	E-UTRA PSCell Addition Delay Requirement

This clause defines requirements for the delay within which the UE shall be able to configure an E-UTRAN PSCell in NR - E-UTRA dual connectivity. The requirements are applicable to an NR - E-UTRA dual connectivity capable UE.

Upon receiving E-UTRAN PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards E-UTRAN PSCell no later than in subframe n + Tconfig_EUTRAN-PSCell:

Where:

Tconfig_EUTRAN-PSCell = TRRC_delay + Tactivation_time + 50ms + TE-UTRAN-PSCell_ DU

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

Tactivation_time is the E-UTRAN PSCell activation delay. If the E-UTRAN PSCell is known, then Tactivation_time is 20ms. If the E-UTRAN PSCell is unknown, then Tactivation_time is 30ms provided the E-UTRAN PSCell can be successfully detected on the first attempt.

TE-UTRAN-PSCell_DU is the delay uncertainty in acquiring the first available PRACH occasion in the E-UTRAN PSCell. TE-UTRAN-PSCell_DU is up to 30ms.

E-UTRAN PSCell is known if it has been meeting the following conditions:

-	During the last 5 seconds before the reception of the E-UTRAN PSCell configuration command:

-	the UE has sent a valid measurement report for the E-UTRAN PSCell being configured and

-	the E-UTRAN PSCell being configured remains detectable according to the cell identification conditions specified in clause 8.8 of TS 36.133 [15],

-	E-UTRAN PSCell being configured also remains detectable during the E-UTRAN PSCell configuration delay Tconfig_EUTRAN-PSCell according to the cell identification conditions specified in clause 8.8 of TS 36.133 [15].

otherwise it is unknown.

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2].

##### 4A.5.1.0.2	E-UTRA PSCell Release Delay Requirement

The requirements in this clause shall apply for a UE which is configured with PCell and E-UTRAN PSCell and may also be configured with one or more SCells and/or E-UTRAN SCells.

Upon receiving E-UTRAN PSCell release in subframe n, the UE shall accomplish the release actions specified in TS 38.331 [2] no later than in subframe n+TRRC_delay:

Where

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2].

#### 4A.5.1.1	NE-DC FR1 addition and release delay of known PSCell

##### 4A.5.1.1	Test purpose

The purpose of this test is to verify that the LTE PSCell addition/release delay and interruption under NE-DC are within the requirements stated in clause 8.8 and clause 8.2.3.2.3 for the case when the PSCell is known by the UE at the time of addition.

##### 4A.5.1.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and NE-DC from Release 15 onwards.

##### 4A.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause [4A.5.X]1 and [4.5.7.X]

The normative reference for this requirement is TS 38.133 [6] clause [A.4A.x.1].

##### 4A.5.1.4	Test description

4A.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table [4A.5.1-1].

Table 4.5.7.1.4.1-1: Test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 4.5.7.1.4.1-2.

Table 4.5.7.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | FFS |  |  |
| Channel bandwidth | FFS |  |  |
| Propagation conditions | AWGN |  | FFS |
| Connection Diagram | TE Part 2Rx | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | FFS |  |
|  | DUT Part 2Rx | FFS |  |
|  | DUT Part 4Rx | FFS |  |
| Exceptions to connection diagram | N/A |  |  |

4A.5.1.4.1	Test Procedure

FFS

##### 4A.5.1.5	Test Requirements

FFS

### <End of Changes>