3GPP TSG-RAN5 Meeting #94-e	R5-221425

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2177 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test procedure 4.9.12 IMS Emergency call or eCall over IMS establishment in 5GC without IMS emergency registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Change in CR #2137 (R5-217286) is also applicable to Test procedure 4.9.12. SRB2 needs to be added in Step 14. Default configuration of RRCReconfiguration-DRB(n, m) has secondaryCellGroup IE, which is not applicable to procedure 4.9.12 and shall be replaced by RRCReconfiguration-SRB2-DRB(n, m). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added exception steps before step 17 to handle RRC and NAS message in any order. SRB2 was added to step 14 by RRCReconfiguration-SRB2-DRB(n, m). Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformat UE will fail TC 11.4.2/3/5/9 unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-220112 with two revisions. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-220112 -> R5-221425 |  |  |  |  |  |  |  |  |

[Start of change]

4.9.12	Test procedure for IMS Emergency call establishment in 5GC without IMS emergency registration

4.9.12.1	Scope

This procedure aims at verifying the UE establishment of IMS Emergency call in 5GC without the need for IMS emergency registration to take place beforehand e.g. under Limited Service or SIM/USIM not available, the SIM/USIM is considered invalid by the UE conditions.

4.9.12.2	Procedure description

4.9.12.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters unless specified otherwise. PLMN/TAI which the cell belongs to, shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it

User Equipment:

-	Whether the Test UICC shall be inserted or not, and, its settings e.g. in regard to the provision of Emergency Numbers, shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.12.2.2	Procedure sequence

Table 4.9.12.2.2-1: Test procedure sequence UE IMS Emergency call establishment in 5GC without IMS emergency registration

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 1 | Check: Does the UE transmits an RRCSetupRequest message with 'establishmentCause' set to 'emergency'? | --> | NR RRC: RRCSetupRequest | - | P |
| 2 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 3 | Check: Does the UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message with 'Service type' set to 'emergency services'? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | P |
| 4 | The SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message with 'Selected NAS security algorithms' set to "null integrity protection algorithm" (5G-IA0), "null ciphering algorithm" (5G-EA0). | <-- | NR RRC: DLInformationTransfer 5GMM: SECURITY MODE COMMAND | - | - |
| 5 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: SECURITY MODE COMPLETE | - | P |
| 6 | The SS transmits a SecurityModeCommand message with cipheringAlgorithm set to 'NULL' ciphering algorithm (nea0) and integrityProtAlgorithm set to 'NULL' integrity protection algorithm (nia0). | <-- | NR RRC: SecurityModeCommand | - | - |
| 7 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | P |
| 8 | The SS transmits a UECapabilityEnquiry message. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 9 | The UE transmits a UECapabilityInformation message. | --> | NR RRC: UECapabilityInformation | - | - |
| 10 | The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 11 | The UE transmits an ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| 12 | Void | - | - | - | - |
| 13 | Check: Does the UE transmits an UL NAS TRANSPORT message with 'Request type' set to 'initial emergency request', and, a PDU SESSION ESTABLISHMENT REQUEST? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | P |
| - | EXCEPTION: In parallel to the events described in steps 14-15 below the events specified in steps 1a1 of Table 4.9.12.2.2-2 take place. | - | - | - | - |
| 14 | The SS transmits an RRCReconfiguration message and an PDU SESSION ESTABLISHMENT ACCEPT to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to the events described in steps 16-18 below the events specified in steps 2 of Table 4.9.12.2.2-2 take place. | - | - | - | - |
| 16 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| - | EXCEPTION: Depending upon UE implementation, steps 17 and 18 can occur in any order. | - | - | - | - |
| 17 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 18 | Check: Does the UE transmits an ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |

Table 4.9.12.2.2-2: IMS signalling and Emergency call establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 1a1 | The generic procedure for IP address allocation in the user plane specified in subclause 4.5A.3 takes place. | - | - | - | - |
| 2 | Generic test procedure for setting up IMS Emergency Voice Call as defined in TS 34.229-5 [47] annex A.6 is performed. | - | - | - | - |

4.9.12.2.3	Specific Message content

All specific message contents shall be according subclause 4.6 and 4.7 with the exceptions below.

NOTE:	Some of the specific message contents provided below assume that the UE is in the state 5GMM-DEREGISTERED.LIMITED-SERVICE or 5GMM-REGISTERED.LIMITED-SERVICE (e.g. the selected cell over 3GPP access is in a forbidden PLMN or is in a forbidden tracking area respectively), or, in 5GMM-DEREGISTERED.NO-SUPI as defined in TS 24.501 [28], subclauses 5.1.3.2.1.3.3 and 5.1.3.2.1.3.6 respectively. All necessary details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

Table 4.9.12.2.3-1: SIB1 (at any time prior and during the procedure, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.6.1-28. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| ims-EmergencySupport |  | Present | true |  |
| } |  |  |  |  |

Table 4.9.12.2.3-2: RRCSetupRequest (step 1, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.6.1-23. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |  |
| establishmentCause |  | emergency |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.12.2.3-3: REGISTRATION REQUEST (step 3, Table 4.9.12.2.2-1)

Derivation Path: Table 4.7.1-6, condition EMERGENCY.

Table 4.9.12.2.3-4: SECURITY MODE COMMAND (step 4, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.1-25. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Selected NAS security algorithms |  |  |  |  |
| Type of ciphering algorithm |  | 5G-EA0 | null ciphering algorithm |  |
| Type of integrity protection algorithm |  | 5G-IA0 | null integrity protection algorithm |  |
| ngKSI |  |  |  |  |
| NAS key set identifier |  | '000'B |  |  |

Table 4.9.12.2.3-5: SecurityModeCommand (step 6, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.6.1-25. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| securityModeCommand SEQUENCE { |  |  |  |  |
| securityConfigSMC SEQUENCE { |  |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |  |
| cipheringAlgorithm |  | nea0 | 'NULL' ciphering algorithm |  |
| integrityProtAlgorithm |  | nia0 | 'NULL' integrity protection algorithm |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.12.2.3-6: REGISTRATION ACCEPT (step 10, Table 4.9.12.2.2-1)

Derivation Path: Table 4.7.1-7, condition EMERGENCY.

Table 4.9.12.2.3-7: UL NAS TRANSPORT (step 13, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.1-10, condition INITIAL_PDU_REQUEST. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | ‘011’B | initial emergency request |  |
| S-NSSAI |  | Not Present |  |  |
| DNN |  | Not Present |  |  |

Table 4.9.12.2.3-8: PDU SESSION ESTABLISHMENT REQUEST (step 13, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | A value that is not currently being used by another PDU session |  |  |
| PTI |  | A value currently not used |  |  |
| SSC mode |  | ‘001’B | SSC mode 1 |  |

Table 4.9.12.2.3-9: DL NAS TRANSPORT (step 14, Table 4.9.12.2.2-1)

Derivation Path: Table 4.7.1-11, condition 5GSM_MESSAGE.

Table 4.9.12.2.3-10: PDU SESSION ESTABLISHMENT ACCEPT (step 14, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Selected SSC mode |  | ‘001’B | SSC mode 1 |  |
| Authorized QoS rules |  | Reference QoS rule #2 as defined in Table 4.8.2.1-2. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #2 as defined in Table 4.8.2.3-2. |  |  |

Table 4.9.12.2.3-11: RRCReconfiguration (step 14, Table 4.9.12.2.2-1)

Derivation Path: Table 4.8.1-1: RRCReconfiguration-SRB2-DRB (1, 0)

Table 4.9.12.2.3-12: PDU SESSION MODIFICATION COMMAND (step 16, Table 4.9.12.2.2-1)

| Derivation Path: Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  | Reference QoS rule #7 as defined in Table 4.8.2.1-7 using condition IMS_VOICE. |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #5 as defined in Table 4.8.2.3-5. |  |  |

Table 4.9.12.2.3-13: RRCReconfiguration (step 16, Table 4.9.12.2.2-1)

Derivation Path: 4.8.1-1C RRCReconfiguration-Speech

Table 4.9.12.2.3-14: INVITE (step 2, Table 4.9.12.2.2-2)

Derivation Path: TS 34.229-5 [47], Annex A.6, Step 1 with conditions A6 and A28

[End of change]