3GPP TSG-RAN5 Meeting #94-e	R5-221505

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2214 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PC5 RRC message RRCReconfigurationCompleteSidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To update default configuration of PC5 RRC message RRCReconfigurationCompleteSidelink - Table 4.6.1A-4 is specified from the DUT's side. But in some test case prose, these messages are sent by the NR-SS-UE so we need all fields to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table 4.6.1A-4 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-220575r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	RRCReconfigurationCompleteSidelink

Table 4.6.1A-4: RRCReconfigurationCompleteSidelink

| Derivation Path: TS 38.331 [6], clause 6.6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationCompleteSidelink ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier-r16 |  | RRC-TransactionIdentifier |  |  |  | TX |  |
|  |  | Set to the same value as the rrc-TransactionIdentifier-r16 field in corresponding RRCConfigurationSidelink message |  |  |  | RX |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationCompleteSidelink-r16 SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| nonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| TX | UE transmits and NR-SS-UE receives. |
| RX | UE receives and NR-SS-UE transmits. |

<End of modified section 1>