3GPP TSG-RAN5 Meeting #94-e	R5-221533

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2801 | rev | 1 | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.5.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose , the TP1 mentions : Quote (1) with { UE in Manual network/SNPN selection mode and an SNPN cell is available for which an entry does not exist in the "list of subscriber data" } ensure that { when { UE is switched on or return to coverage } then { UE does not indicate to the user any available SNPNs } } Unquote The highlighted UE hehavior may never happen. This is because the 23.122 cl 4.9.3.1.0 mentions : Quote … If registration is not possible on recovery from lack of coverage due to the registered SNPN being unavailable, an MS may, optionally, continue looking for the registered SNPN for an implementation dependent time. … Unquote Some UEs may perform a search for 10s, some may carry on for 300s. It is not possible to define a timeframe within which a UE could “display” the detected SNPN Ids. Moreover, the highighted text is unclear as to whats exactly expected. Is the expectation limited to a UE NOT “displaying” a SNPN ID ? Or is it expected from the UE that in case it displayed the detected SNPN ID, its NOT expected to camp on it when user manually selects this ID ? This needs to be clarified. Similarly for TP2, Quote (2) with { UE in Manual network/SNPN selection mode } ensure that { when { an SNPN cell is available for which an entry exists in the "list of subscriber data" } then { UE indicates to the user that an SNPN is available and user selects the SNPN } } Unquote As explained above (23.122 cl 4.9.3.1.0) it is not possible to define a timeframe within which a UE could “display” the detected SNPN Ids. Thus the behvior expected in highlighted text cannot be tested. Also, it is unclear what UE behaviour is expected by the highlighted text. Does the UE perform REGISTRATON, or just cell re-selection ? This also needs clarification. At Step 26 of the current Prose, NR Cell 2 is set to -88 dB and NR Cell 4 to -78dB. As soon as the SNPN selection mode is changed to automatic at Step 28, a fast UE could immediately try to camp on NR Cell 4 before the power level setting T4 is applied at Step 29. This could cause testcase failure, and needs to be addressed. The REGISTRATION REQ sent by the UE at Steps 9 and 32 should be setting the IE registration type to “initial registration” which is inline with 24.501 cl 5.2.3.2.5. This is not being checked in current Prose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TP1 text. Changed the text to “Upon manual selection of a SNPN cell not present in the list of subscriber data the UE does not register on the selected SNPN” Updated the TP2 text. Changed the text to “Upon manual selection of a SNPN cell present in the list of subscriber data the UE performs registration on the selected SNPN” Also updated the text of Step 3 and 6 to tell the user to manually select the SNPN Id of the cells. Introduced a NOTE which clarfies that the manual SNPN selection is performed by a MMI or AT command. Applied power setting T4 at Step 28, and changed the SNPN selection mode to Automatic at Step 29. Also introduced a specific message content for Registration REQ for Step 9 and 32. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN based on current Prose may fail a Conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is revision of R5-220683 |  |  |  |  |  |  |  |  |

#### 6.5.1.1	SNPN Selection in Manual Mode

6.5.1.1.1	Test Purpose (TP)

(1)

with { UE in Manual network/SNPN selection mode and an SNPN cell is available for which an entry does not exist in the "list of subscriber data" }

ensure that {

when { UE is switched on or return to coverage }

then { UE does not indicate to the user any available SNPNs }

}

(2)

with { UE in Manual network/SNPN selection mode }

ensure that {

when { an SNPN cell is available for which an entry exists in the "list of subscriber data" }

then { UE indicates to the user that an SNPN is available and user selects the SNPN }

}

(3)

with { UE in Manual network/SNPN selection mode and has registered on an SNPN selected by the user }

ensure that {

when { an SNPN cell is available for which an entry exists in the "list of subscriber data" }

then { UE does not automatically register on the SNPN cell until the user selects automatic SNPN selection mode }

}

6.5.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.122 clauses 4.9.3.1.0 and 4.9.3.1.2. Unless otherwise stated these are Rel-16 requirements.

[TS 23.122, clause 4.9.3.1.0]

At switch on, or following recovery from lack of coverage, the MS selects the registered SNPN (if it is available) using NG-RAN access technology and if necessary (in the case of recovery from lack of coverage, see subclause 4.5.2) attempts to perform an LR.

NOTE 1:	The MS in automatic SNPN selection mode can end the SNPN search procedure once the registered SNPN is found on NG-RAN access technology.

NOTE 2:	An MS in automatic SNPN selection mode can use location information to determine which SNPNs can be available in its present location.

If successful registration is achieved, the MS indicates the selected SNPN.

If there is no registered SNPN, or registration is not possible due to the SNPN being unavailable or registration failure, the MS follows the procedure in subclause 4.9.3.1.1 or subclause 4.9.3.1.2 depending on its SNPN selection mode. At switch on, the MS shall use the SNPN selection mode that was used before switching off.

NOTE 3:	If successful registration is achieved, then the current serving SNPN becomes the registered SNPN and the MS does not store the previous registered SNPN for later use.

If registration is not possible on recovery from lack of coverage due to the registered SNPN being unavailable, an MS may, optionally, continue looking for the registered SNPN for an implementation dependent time.

NOTE 4:	An MS registered to an SNPN should behave as described above only if one or more PDU sessions are currently active.

[TS 23.122, clause 4.9.3.1.2]

The MS indicates to the user one or more SNPNs, which are available and each of them is identified by an SNPN identity in an entry of the "list of subscriber data" in the ME. This includes SNPNs in the list of "permanently forbidden SNPNs", and the list of "temporarily forbidden SNPNs". The MS may indicate to the user whether the available SNPNs are present in the list of "temporarily forbidden SNPNs" or the list of "permanently forbidden SNPNs". The order in which those SNPNs are indicated is MS implementation specific.

For each of the SNPNs indicated to the user, the UE shall forward a human-readable network name along with the SNPN identity to the upper layers if the system information broadcasted for the SNPN includes the human-readable network name for the SNPN.

The MS shall limit its search for the SNPN to the NG-RAN access technology.

The user may select his desired SNPN and the MS then initiates registration on this SNPN using the NG-RAN access technology, the subscriber identifier and the credentials from an entry of the "list of subscriber data", with the SNPN identity matching the selected SNPN (this may take place at any time during the presentation of SNPNs).

Once the MS has registered on an SNPN selected by the user, the MS shall not automatically register on a different SNPN unless the user selects automatic SNPN selection mode.

NOTE:	Emergency services are not supported in SNPN access mode.

If the user does not select an SNPN, the selected SNPN shall be the one that was selected either automatically or manually before the SNPN selection procedure started. If no such SNPN was selected or that SNPN is no longer available, then the MS shall attempt to camp on any acceptable cell and enter the limited service state.

6.5.1.1.3	Test description

6.5.1.1.3.1	Pre-test conditions

System Simulator:

-	3 SNPN cells: NR Cell 1, NR Cell 2, and NR Cell 4 are configured broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-12 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The UE is in Manual SNPN selection mode.

-	The UE is provisioned with a “list of subscriber data” to allow access to SNPN identified by NR Cell 2 and NR Cell 4.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN and the UE is in state Switched OFF (state 0-A).

6.5.1.1.3.2	Test procedure sequence

Table 6.5.1.1.3.2-1/2 shows the cell configurations used during the test. Subsequent configurations marked “T1”, “T2” “T3” “T4”etc are applied at the points indicated in the Main behaviour description in Table 6.5.1.1.3.2-3. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.1.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” | “Off” |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | “Off” |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | -78 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” | -78 |  |

Table 6.5.1.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |

Table 6.5.1.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.5.1.1.3.2-1/2. | - | - | - | - |
| 2 | The UE is Switched ON. | - | - | - | - |
| 3 | Manual selection of SNPN identified by NR Cell 1 is performed (NOTE 1). | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | RRCSetupRequest | 1 | F |
| 5 | SS adjusts cell levels according to row T2 of table 6.5.1.1.3.2-1/2. | - | - | - | - |
| 6 | Manual selection of SNPN identified by NR Cell 2 is performed (NOTE 1). | - | - | - | - |
| 7 | Check: Does the UE send a RRCSetupRequest on NR Cell 2? | --> | RRCSetupRequest | 2 | P |
| 8-25 | Steps 3 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed on NR Cell 2. | - | - | - | - |
| 26 | SS adjusts cell levels according to row T3 of table 6.5.1.1.3.2-1/2. | - | - | - | - |
| 27 | Check: Does the UE send a RRCSetupRequest on NR Cell 4 within the next 60 s? | --> | RRCSetupRequest | 3 | F |
| 28 | SS adjusts cell levels according to row T4 of table 6.5.1.1.3.2-1/2 | - | - | - | - |
| 29 | The user sets the UE in Automatic SNPN/Network selection mode (NOTE 1). | - | - | - | - |
| 30 | Check: Does the UE send an RRCSetupRequest on NR Cell 4? | --> | RRCSetupRequest | 3 | P |
| 31-47 | Steps 3 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed on NR Cell 4. | - | - | - | - |
| NOTE 1: This is performed via MMI or AT command. |  |  |  |  |  |

6.5.1.1.3.3	Specific message contents

Table 6.5.1.1.3.3-1: REGISTRATION REQUEST (step 9, 32 Table 6.5.1.1.3.2-3)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type value |  | ‘001’B | Initial registration |  |