3GPP TSG-RAN5 Meeting #94-e	R5-221621

# Electronic Meeting, February 21 - March 4 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0539 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of reference sensitivity test point analysis for CA_n5A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TP analysis needs to be done and documented in TR 38.905 before test requirements for a new carrier aggregation configuration can be introduced in TS 38.521-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding reference TP analysis for CA_n5A-n78A. This configuration has reference sensitivity exceptions due to harmonic interference |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TP analysis of CA_n5A-n78A will still be missing and testing of this CA configuration can not be implemented in TS 38.521-1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.1, B4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 CR1526 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-220358 and the outcome of 1 revisions to this document. |  |  |  |  |  |  |  |  |

## [Start of changes]

#### 4.1.3.1	Reference Sensitivity test cases for FR1 NR CA

Editor's note:

-	Not all CA configurations completed in TS 38.521-1 refsens test cases are listed in Table 4.1.3.1-1.

-	Columns for “Refsens exception, victim band” vs. "Need to test fallback" in Table 4.1.3.1-1 need to be clarified.

This clause contains information on test point analysis and status for FR1 NR CA test cases in TS 38.521-1 [2] clause 7. The analyses are performed per CA configuration in Table 4.1.3.1-1.

Table 4.1.3.1-1: Reference Sensitivity test cases per CA configuration

| CA config | Number of test points | Refsens exception, victim band (Note 2) | Requirement coverage (Note 2) | Need to test fallback | Fallback CA config. | Test point analysis updated |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41C |  |  | NE | No |  | RAN5#89-e |
| CA_n66B |  |  | NE | No |  | RAN5#86-e |
| CA_n66(2A) |  |  | NE | No |  | RAN5#86-e |
| CA_n78C |  |  | NE | No |  | RAN5#87-e |
| CA_n1A-n77A | 2 | n77 (HD2) | HD | No |  | RAN5#89-e |
| CA_n1A-n78A | 1 |  | NE | No |  | RAN5#89-e |
| CA_n3A-n77A | 2 | n78 (HD2) | HD | No |  | RAN5#84 |
| CA_n3A-n78A | 4 | n3 (IMD2 ¦fn78-fn3¦, IMD4 ¦fn78-3*fn3¦), n78 (HD2) | HD, IMD | No |  | RAN5#87-e |
| CA_n5A-n78A | 3 | n78 (HD2) | HD | No |  | RAN5#94-e |
| CA_n8A-n78A | 2 | n8 (IMD4 ¦fn78-3*fn8¦), n78 (HD4) | HD, IMD | No |  | RAN5#87-e |
| CA_n28A-n41A |  |  | NE | No |  | RAN5#91-e |
| CA_n28A-n79A |  |  | NE | No |  | RAN5#91-e |
| CA_n41A-n79A |  |  | NE | No |  | RAN5#83 |
| CA_n70A-n71A | 3 | n70 (HD3) | HD, IMD | No |  | RAN5#85 |
| CA_n66A-n70A-n71A |  | n66 |  | No |  | RAN5#87-e |
| CA_n29A-n66A-n70A |  |  |  | No |  | RAN5#89-e |
| NOTE 1: If single UL is allowed in a DC_nXA-nYA configuration the IMD requirements does not apply for a single UL UE and non-exception requirements are tested. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 |  |  |  |  |  |  |

## [Several Clauses skipped]

# B.4	Test Frequencies selection

In E-UTRA DL CA and UL CA reference sensitivity testing, Low range and High range are selected for intra-band CA testing. Mid range is selected for inter band E-UTRA CA testing.

Considering NR CA testing scenario is very similar to E-UTRA_CA test case, similar Test Frequencies should be used for NR CA default case.

It is proposed that Low and High Range are tested as default for intra-band and mid range for inter-band. For CA combinations containing intra-band configuration in an inter-band CA configuration it is proposed that low and high range are tested for intra-band CA and mid channel for inter-band without intra-band component. It is also proposed that the fallback configurations from intra-band configuration to single carrier component do not need to be tested even if the test frequency would differ (e.g. XA-YC -> XA-YA fallback does not need to be tested even if YC is tested with low/high frequency and YA would be tested with mid frequency).

For CA configurations affected by exceptions the test frequency cannot be freely chosen. One test frequency per exception requirement is sufficient to test the requirement, in addition to testing the case where the exception is avoided. This is indicated in table 2.3-2. Exceptions in TS 38.101-1 [5] clause 7.3A.5 (2UL intermodulation) are not specified since the test frequency and channel bandwidth is already specified in TS 38.101-1 [5].

Table B.4-1: Test frequency selection per band pair for UL harmonics exception

| Band pair (Note 2) | Frequency | Channel BW [MHz] | Comment |  |
| --- | --- | --- | --- | --- |
| n1A / n77A | Mid / Low | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | Mid / 3900 | 20/ Highest | Exception Note 2 of TS 38.101 table 7.3A.4-1 |  |
|  | Mid / 3870 | 20/20 | Exception Note 3 of TS 38.101 table 7.3A.4-1 |  |
| n3A / n77A | TBD/TBD | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | TBD/TBD | Highest/Highest | Exception Note 2 of TS38.101 table 7.3A.4-1 |  |
|  | TBD/TBD | 20/20 | Exception Note 3 of TS38.101 table 7.3A.4-1 |  |
| n3A / n78A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | Mid/3495 MHz | Highest/Highest | Exception Note 2 of TS38.101 table 7.3A.4-1 |  |
|  | Mid/3465 MHz | 20/20 | Exception Note 3 of TS38.101 table 7.3A.4-1 |  |
| n5A / n78A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 in TS 38.521-1 [2] |  |
|  | Mid/3346 MHz | Highest/Highest | Exception Note 4 of TS 38.101 [5] table 7.3A.4-1 |  |
|  | Mid/3316 MHz | 20/20 | Exception Note 5 of TS 38.101 [5] table 7.3A.4-1 |  |
| n8A / n78A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | Mid/3590 MHz | Highest/Highest | Exception Note 5 of TS38.101 table 7.3A.4-1 |  |
| n70A / n71A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | Low/Low | Highest / 10 | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 20@10 |  |
|  | Low/Low | 5 / 5 | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 8@10 |  |
| NOTE 1: This selection is used in test case 7.3A.1_1 unless otherwise stated. NOTE 2: Aggressor band in bold. |  |  |  |

Proposal 2a: Low range and high range for intra band CA and mid range for inter-band CA shall be selected for NR in default test cases 7.3A.1 to 7.3A.4 in general, but final selection is band dependent. For CA combinations containing intra-band configuration in an inter-band CA configuration low and high range shall be selected for intra-band CA and mid channel for inter-band without intra-band component. The fallback configurations from intra-band configuration to single carrier component do not need to be tested even if the test frequency would differ.

Proposal 2b: In the additional test case, one test frequency per exception is selected.

## [End of changes]