3GPP TSG-RAN5 Meeting #94-e	R5-221649

# Electronic Meeting, 21th Feb– 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0290 | rev | 1 | Current version: | 16.10.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT analysis for Mob_enh RRM TCs 7.3.1.4 and 7.3.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. To provide TT and MU analysis for FR1-FR2 inter-band DAPS handover test cases: - 7.3.1.4, 7.3.1.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. TT and MU analysis for FR1-FR2 inter-band DAPS handover test cases are added: - 7.3.1.4, 7.3.1.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 1619,1620, 1622 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-220723r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Add the following attached .zip file to TR 38.903:

"38.533 7.3.1.4+7.3.1.5 TT.zip "

<End of modified section 1>

<Start of modified section 2>

Table 8-2: Grouping of FR2 test cases defined in Clauses 5, 7 and 8 of TS 38.533

| Group | Test Case Numbers | .zip file name | Comments |
| --- | --- | --- | --- |
| Timing_Advance_01 | 5.4.3.1 7.4.3.1 | “38.533 5.4.3.1+7.4.3.1 TT.zip“ | 1 NR FR2 cell, no fading |
| iRAT_SS-RSRP_01 | 8.5.2.1.2 | “38.533 8.5.2.1.2 TT.zip” | “1 NR FR2 cell, 1 E-UTRA serving cell, 2 sub-tests, No fading” |
| iRAT_SS-RSRQ_01 | 8.5.2.2.2 | “38.533 8.5.2.2.2 TT.zip” | “1 NR FR2 cell, 1 E-UTRA serving cell, 2 sub-tests, No fading” |
| iRAT_SS-SINR_01 | 8.5.2.3.2 | “38.533 8.5.2.3.2 TT.zip” | “1 NR FR2 cell, 1 E-UTRA serving cell, 2 sub-tests, No fading” |
| Interruption_Transition_01 | 5.5.2.1 5.5.2.2 | “38.533 5.5.2.1+5.5.2.2 TT.zip” | ”1 E-UTRAN Cell, 1 NR FR2 Cell, 1 time period, No fading” |
| Inter_Freq_Meas_01 | 5.6.2.1 5.6.2.3 7.6.2.1 7.6.2.3 | “38.533 5.6.2.1+5.6.2.3+7.6.2.1+7.6.2.3 TT v2.zip” | “2 Inter Frequency NR FR2 Cells, 2 time periods, Various number of sub-tests, No fading” |
| Inter_Freq_Meas_02 | 5.6.2.5 5.6.2.7 7.6.2.5 7.6.2.7 | “38.533 5.6.2.5+5.6.2.7+7.6.2.5+7.6.2.7 TT.zip” | “2 Inter Frequency NR Cells (Cell 1 on FR1 and Cell 2 on FR2), 2 time periods, Various number of sub-tests, No fading” |
| Inter_Freq_Meas_03 | 5.6.2.6 5.6.2.8 7.6.2.6 7.6.2.8 | “38.533 5.6.2.6+5.6.2.8+7.6.2.6+7.6.2.8 TT.zip” | “2 Inter Frequency NR Cells (Cell 1 on FR1 and Cell 2 on FR2), 2 time periods, Various number of sub-tests, No fading” |
| Inter_Freq_Meas_04 | 5.6.2.2 5.6.2.4 7.6.2.2 7.6.2.4 | “38.533 5.6.2.2+5.6.2.4+7.6.2.2+7.6.2.4 TT.zip” | “2 Inter Frequency NR Cells (both on FR2), 2 time periods, Various number of sub-tests, No fading” |
| SSB_Based_L1-RSRP-Meas | 5.6.3.1 5.6.3.2 7.6.3.1 7.6.3.2 | “38.533 5.6.3.1+5.6.3.2+7.6.3.1+7.6.3.2 TT.zip” | “1 NR FR2 Cell (1 E-UTRA Cell for NSA case), 2 time periods, No fading” |
| SS-RSRP_01 | 5.7.1.1 7.7.1.1 | “38.533 5.7.1.1+7.7.1.1 TT v2.zip” | “2 Intra-Frequency NR FR2 Cells, 2 sub-tests, No fading” |
| SS-RSRP_02 | 5.7.1.2 7.7.1.2 | “38.533 5.7.1.2+7.7.1.2 TT v2.zip” | “2 Inter-Frequency NR FR2 Cells, 2 sub-tests, No fading” |
| SS-RSRQ_01 | 5.7.2.1 7.7.2.1 | “38.533 5.7.2.1+7.7.2.1 TT.zip” | “2 Intra-Frequency NR FR2 Cells, 2 sub-tests, No fading” |
| SS-RSRQ_02 | 5.7.2.2 7.7.2.2 | “38.533 5.7.2.2+7.7.2.2 TT.zip” | “2 Inter-Frequency NR FR2 Cells, 2 sub-tests, No fading” |
| SS-SINR_01 | 5.7.3.1 7.7.3.1 | “38.533 5.7.3.1+7.7.3.1 TT.zip” | “2 Intra-Frequency NR FR2 Cells, 2 sub-tests, No fading” |
| SS-SINR_02 | 5.7.3.2 7.7.3.2 | “38.533 5.7.3.2+7.7.3.2 TT v2.zip” | “2 Inter-Frequency NR FR2 Cells, 3 sub-tests, No fading” |
| CSI-RS_Based_L1-RSRP-Meas | 5.6.3.3 5.6.3.4 7.6.3.3 7.6.3.4 | “38.533 5.6.3.3+6.6.3.4+7.6.3.3+7.6.3.4 TT.zip” | “1 NR FR2 Cell (1 E-UTRA Cell for NSA case), 1 time period, No fading” |
| Inter_band_DAPS_HO | 7.3.1.4 7.3.1.5 | "38.533 7.3.1.4+7.3.1.5 TT.zip " | "1 NR FR2 Cell, 1 SSB, 5 time periods, 1 AoA in Rx peak and rough beam" |

<End of modified section 2>