3GPP TSG-RAN5 Meeting #94-e	R5-221650

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1685 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion 4.5.5.5 including TT anaysis results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM Test Case 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX is not complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been updated with the following information: Test requirements including TT analysis results, and MU and TT should be a combination of 4.5.5.1 and 4.5.5.3, where 4.5.5.1 is for SSB based BFD and LR, and 4.5.5.3 is for CSI-RS based BFD and LR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case 4.5.5.5 will be still incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0146 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Initial condition is changed - Descriptions under message contents are changed for format. - Reintroduce the Editor’s note as test configuration needs to be fixed in RAN4. This is the final outcome of R5-220945 and the result of 1 revision |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

4.5.5.5	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	Some test configuration needs to be fixed in RAN4, BW and BWP configuration is missing in test case

4.5.5.5.1	Test purpose

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving SCell and that the UE performs correct SSB-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the SCell without schedulingRequestID-BFR-SCell-r16 configuration, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

4.5.5.5.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward supporting EN-DC FR1, CSI-RS based RLM, and SSB link recovery.

4.5.5.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.5.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.4.5.5.4.

4.5.5.5.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 4.5.5.5.4-1 shows the SNR of the CSI-RS in set q0 in the active SCell to emulate beam failure. Figure 4.5.5.5.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 4.5.5.5.4-1: SNR and L1-RSRP variation for EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX

4.5.5.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.5.5.4.4.1-1.

Table 4.5.5.5.4.1-1: Supported test configurations for EN-DC FR1 Scell CSI-RS-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 4.5.5.5-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4.5.5.5-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.5.5.5-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4.5.5.5-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4.5.5.5-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.5.5.5-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 4.5.5.5.4.1-2.

Table 4.5.5.5.4.1-2: Initial conditions for EN-DC FR1 Scell CSI-RS-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.5.5.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. The general test parameter settings are set up according to Table 4.5.5.5.4.1-3.

2. Message contents are defined in clause 4.5.5.5.4.3.

3. Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is the NR cell (PSCell) and Cell 3 is the NR cell (SCell) with the power level set according to Annex C.1.2 and C.1.3 for this test

Table 4.5.5.5.4.1-3: General test parameters for EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Active PSCell |  |  |  | Cell 2 |  |
| RF Channel Number |  |  |  | 2 |  |
| Active SCell |  |  |  | Cell 3 |  |
| RF Channel Number |  |  |  | 3 |  |
| Duplex mode |  | Config 1, 4 |  | FDD |  |
|  |  | Config 2, 3, 5, 6 |  | TDD |  |
| TDD Configuration |  | Config 1, 4 |  | Not Applicable |  |
|  |  | Config 2, 5 |  | TDDConf.1.1 |  |
|  |  | Config 3, 6 |  | TDDConf.2.1 |  |
| CORESET |  | Config 1, 4 |  | CR.1.1 FDD | A.3.1.2 |
| Reference Channel |  | Config 2, 5 |  | CR.1.1 TDD |  |
|  |  | Config 3, 6 |  | CR.2.1 TDD |  |
| SSB Configuration |  | Config 1, 4 |  | SSB.1 FR1 | A.3.10 |
|  |  | Config 2, 5 |  | SSB.1 FR1 |  |
|  |  | Config 3, 6 |  | SSB.2 FR1 |  |
| SMTC |  | Config 1, 2, 4, 5 |  | SMTC.1 | A.3.11 |
| Configuration |  | Config 3, 6 |  | SMTC.1 |  |
| PDSCH/PDCCH |  | Config 1, 2, 4, 5 |  | 15 KHz |  |
| subcarrier spacing |  | Config 3, 6 |  | 30 KHz |  |
| PRACH Configuration |  | Config 1, 2, 4, 5 |  | Table A.3.8.2.1-1 |  |
|  |  | Config 3, 6 |  | Table A.3.8.2.1-1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 in activated SCell |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 | A.2.1 |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure |  | DCI format |  | 1-0 |  |
| detection transmission parameters |  | Number of Control OFDM symbols |  | 2 |  |
|  |  | Aggregation level | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  |  | DMRS precoder granularity |  | REG bundle size |  |
|  |  | REG bundle size |  | 6 |  |
| DRX |  |  |  | OFF |  |
| Gap pattern ID |  |  |  | N.A. |  |
| schedulingRequestID-BFR-SCell-r16 |  |  |  | absent | When the field is absent, the random access procedure will be triggered for SCell BFR |
| SSB Index assigned as CBD RS (q1) in activated SCell |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp- | Config 1, 2, 4, 5 |  | dBm/SCS | -98 | Threshold used |
| ThresholdSSB | Config 3, 6 |  | kHz | -95 | for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [7], clause 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [7], clause 5.17 |
| CSI-RS |  | Config 1, 4 |  | CSI-RS.1.2 FDD | A.3.14 |
| configuration for q0 |  | Config 2, 5 |  | CSI-RS.1.2 TDD |  |
|  |  | Config 3, 6 |  | CSI-RS.2.2 TDD |  |
| CSI-RS |  | Config 1, 4 |  | CSI-RS.1.1 FDD | A.3.14 |
| configuration for |  | Config 2, 5 |  | CSI-RS.1.1 TDD |  |
| CSI reporting |  | Config 3, 6 |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  | Config 1, 4 |  | TRS.1.1 FDD |  |
|  |  | Config 2, 5 |  | TRS.1.1 TDD |  |
|  |  | Config 3, 6 |  | TRS.1.2 TDD |  |
| csi-RS-Index |  | Config 1, 4 |  | CSI-RS.1.2 FDD | A.3.14 |
| assigned as RLM |  | Config 2, 5 |  | CSI-RS.1.2 TDD |  |
| RS |  | Config 3, 6 |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 0.18 |  |
| T3 |  |  | s | 0.14 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 0.17 |  |
| D1 |  |  | s | 0.13 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

4.5.5.5.4.2	Test procedure

Same test procedure as described in section 4.5.5.3.4.2, except following exception and step 8:

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1, cell 2 and cell 3. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled.

8.	If the iteration or random access procedure for BFD fails, the SS shall first attempt to release and add the FR1 SCell, by ensuring the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5. If that also fails, then the UE is switched OFF/ON to proceed with the next iteration.

4.5.5.5.4.3	Message contents

Same message contents as described in section 4.5.5.3.4.3 with following exceptions:

Table 4.5.5.5.4.3-1: Common Exception messages for EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-10 with Condition SSB CBD |

4.5.5.5.5	Test requirements

Table 4.5.5.5.5.1-3: Cell specific test parameters for FR1 PSCell and SCell for beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Cell2 | Test 1 Cell3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 to T5 | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1, 4 | dB | 5.8 | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2, 5 |  | 5.8 | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3, 6 |  | 5.8 | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_CSI-RS of set q1 | Config 1, 4 | dB | -10.2 | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2, 5 |  | -10.2 | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3, 6 |  | -10.2 | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1, 4 | dBm/SCS kHz | -108.2 | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2, 5 |  | -108.2 | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3, 6 |  | -105.2 | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1, 4 | dBm/15 kHz | -98 | -98 |  |  |  |  |
|  | Config 2, 5 |  | -98 | -98 |  |  |  |  |
|  | Config 3, 6 |  | -98 | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause [A.3.6]. |  |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 2.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 2 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 2.

During T3 the UE shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 120+10 ms after the start of T5, the UE shall transmit preamble for UL-SCH resource application, followed by MAC-CE on the assigned uplink resources containing  a beam associated with the candidate beam set q1. The UE shall not transmit preamble earlier than time point B.

During T5, the System Simulator shall transmit a Random Access Response to UE after the System Simulator receives the preamble from UE. The UE shall transmit the msg.3 containing candidate beam set q1 for SCell BFR if UE receives the Random Access Response.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

## << End of changes >>