3GPP TSG-RAN WG5 Meeting #94-e                                                                                                                                       R5-221661

# Electronic Meeting, 21st February– 4th March 2022

Agenda Item:	5.3.1.17

Source: 	CAICT

Title: 	Discussion of Additional Information for 6.2.2 and 6.2.3 of 38.521-1 and 6.2B.2.3 and 6.2B.3.3 of 38.521-3

Document for:	Discussion and Endorsement

Introduction

Additional Information in 38.522 for 6.2.2 and 6.2.3 of 38.521-1 and 6.2B.2.3 and 6.2B.3.3 of 38.521-3 are incomplete and lead to redundant or incomplete execution in certification.

Discussion

Test case skipped rules are defined in 38.522:

4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 |  |
| 6.2.2 | UE maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC1.5 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5.2.4.1 is executed. Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.3 has been executed. |

| 6.5.2.4.1 | NR ACLR | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC1.5 PC2 PC3 | Skip TC 6.5.2.4.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.2.3 or 6.5B.2.3.3 has been executed. |
| --- | --- | --- | --- | --- | --- | --- | --- |

4.1.3	NR interworking between NR FR1 and NR FR2 and between NR and LTE conformance test cases

Table 4.1.3-1: Applicability of RF EN-DC FR1 and FR2 conformance test cases, ref. TS 38.521-3 [3]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA/DC-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter characteristics |  |  |  |  |  |  |
| 6.2B | Transmitter power for DC |  |  |  |  |  |  |
| 6.2B.1.1 | UE Maximum Output Power for Intra-Band Contiguous EN-DC | Rel-15 | C009 | UEs supporting Intra-Band Contiguous EN-DC (2UL CCs) | E003 |  | NOTE 1 |

| 6.2B.2.3 | UE Maximum Output Power reduction for Inter-Band EN-DC within FR1 (1 NR CC) | Rel-15 | C011 | UEs supporting Inter-Band EN-DC within FR1 with 1 NR UL CC | E005b | PC3 PC2 | NOTE 5 Test execution is not necessary if TS 38.521-3 TC 6.5B.2.3.3 is executed. Skip TC 6.2B.2.3 if UE supports SA and TS 38.521-1 TC 6.2.2 has been executed. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Observation 1: 6.2.2 MPR of 38.521-1 can be skipped if 6.5.2.4.1 ACLR of 38.521-1 or 6.2B.2.3 MPR of 38.521-3 is executed.  6.5.2.4.1 ACLR of 38.521-1 and 6.2B.2.3 MPR of 38.521-3 can be skipped if 6.5B.2.3.3 ACLR is executed.

In a real certification there is a chance that 6.5B.2.3.3.1 of 38.521-3 is executed, and neither 6.2.5.4.1 of 38.521-1 nor 6.2B.2.3 of 38.521-3 is executed. Then the spec reader cannot come to the conclusion that 6.2.2 of 38.521-1 can be skipped as the condition in the Additional Information column of 6.2.2 of 38.521-1 is not met. But as the requirements of 6.2.2 of 38.521-1 have been tested in 6.5B.2.3.3.1 of 38.521-3, 6.22 of 38.521-1 can be skipped if 6.5B.2.3.3.1 of 38.521-3 is executed.

Proposal 1: Revise the Additional Information of 6.2.2 of 38.521-1 to imply that 6.2.2 of 38.521-1 can be skipped if 6.5B.2.3.3.1 of 38.521-3 is executed.

Similar issue is with 6.2B.2.3 of 38.521-3.

Proposal 2: Revise the Additional Information of 6.2B.2.3 of 38.521-3 to imply that 6.2B.2.3 of 38.521-3 can be skipped if 6.5.2.4.1 of 38.521-1 is executed.

Test applicability of 6.2.3 A-MPR of 38.521-1 is as below:

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, and NS_35 to all types of NR UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply in test case 6.5.2.4 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and 3 UE release 15 and forward.

Observation 2: 6.5.2.3, 6.5.2.4 and 6.5.3.3 include part of the requirements of 6.2.3 separately.

There are two test cases included in 6.5.2.4 of 38.521-1 and only 6.5.2.4.2 includes part of the tests of A-MPR.

Proposal 3: Revise 6.5.2.4 in test applicability of 6.2.3 A-MPR to 6.5.2.4.2 and add a NOTE in 6.2.3.2 to imply that 6.2.3 can be skipped if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

The Additional Information of 6.2.3 A-MPR of 38.521-1in 38.522 is as below:

| 6.2.3 | UE additional maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 PC3 | D001 | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5.2.3 and 6.5.3.3 are executed. Skip TC 6.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.3 has been executed. |
| --- | --- | --- | --- | --- | --- | --- | --- |

The Additional Information of 6.2B.3.3 A-MPR of 38.521-3in 38.522 is as below:

| 6.2B.3.3 | UE Additional Maximum Output power reduction for inter-band EN-DC within FR1 (1 NR CC) | Rel-15 | C011z | UEs supporting Inter-Band EN-DC within FR1 with 1 NR UL CC and modified MPR behaviour | E005b | PC3 PC2 | NOTE 5 Test execution is not necessary if TS 38.521-3 TC 6.5B.2.3.2 is executed. Skip TC 6.2B.3.3 if UE supports SA and TS 38.521-1 TC 6.2.3 has been executed. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Observation 3: 6.5.2.4.2 is missing in the Additional Information of 6.2.3 of 38.521-1.

Observation 4:  6.5B.2.3.3.2 and 6.5B.4.3 are missing in the Additional Information of 6.2B.3.3 of 38.521-3.

Proposal 4: Revise the Additional Information of 6.2.3 of 38.521-1 to imply that 6.2.3 of 38.521-1 can be skipped if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 of 38.521-1 are executed. Proposal 5: Revise the Additional Information of 6.2B.3.3 of 38.521-3 to imply that 6.2B.3.3 of 38.521-3 can be skipped if 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 of 38.521-3 are executed.

Similar issue as Proposal 1 for 6.2.2 is for 6.2.3 and 6.2B.3.3.

Proposal 6: Revise the Additional Information of 6.2.3 of 38.521-1 to imply that 6.2.3 of 38.521-1 can be skipped if 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 of 38.521-3 are executed.

Proposal 7: Revise the Additional Information of 6.2B.3.3 of 38.521-3 to imply that 6.2B.3.3 of 38.521-3 can be skipped if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 of 38.521-1 are executed.

Conclusion

Observation 1: 6.2.2 MPR of 38.521-1 can be skipped if 6.5.2.4.1 ACLR of 38.521-1 or 6.2B.2.3 MPR of 38.521-3 is executed.  6.5.2.4.1 ACLR of 38.521-1 and 6.2B.2.3 MPR of 38.521-3 can be skipped if 6.5B.2.3.3 ACLR is executed.

Proposal 1: Revise the Additional Information of 6.2.2 of 38.521-1 to imply that 6.2.2 of 38.521-1 can be skipped if 6.5B.2.3.3.1 of 38.521-3 is executed.

Proposal 2: Revise the Additional Information of 6.2B.2.3 of 38.521-3 to imply that 6.2B.2.3 of 38.521-3 can be skipped if 6.5.2.4.1 of 38.521-1 is executed.

Observation 2: 6.5.2.3, 6.5.2.4 and 6.5.3.3 include part of the requirements of 6.2.3 separately.

Proposal 3: Revise 6.5.2.4 in test applicability of 6.2.3 A-MPR to 6.5.2.4.2 and add a NOTE in 6.2.3.2 to imply that 6.2.3 can be skipped if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

Observation 3: 6.5.2.4.2 is missing in the Additional Information of 6.2.3 of 38.521-1.

Observation 4:  6.5B.2.3.3.2 and 6.5B.4.3 are missing in the Additional Information of 6.2B.3.3 of 38.521-3.

Proposal 4: Revise the Additional Information of 6.2.3 of 38.521-1 to imply that 6.2.3 of 38.521-1 can be skipped if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 of 38.521-1 are executed. Proposal 5: Revise the Additional Information of 6.2B.3.3 of 38.521-3 to imply that 6.2B.3.3 of 38.521-3 can be skipped if 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 of 38.521-3 are executed.

Proposal 6: Revise the Additional Information of 6.2.3 of 38.521-1 to imply that 6.2.3 of 38.521-1 can be skipped if 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 of 38.521-3 are executed.

Proposal 7: Revise the Additional Information of 6.2B.3.3 of 38.521-3 to imply that 6.2B.3.3 of 38.521-3 can be skipped if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 of 38.521-1 are executed.