3GPP TSG-RAN5 Meeting #94-e	R5-221708

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0500 | rev | 1 | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to testability of test requirements due to achievable SNR improvements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current specification has many 64QAM test points for bands n260/n259 untestable. All 256QAM test points are currently untestable due to lack of MU analysis. Based on the outcome of discussion paper R5-221164, many test points can be made testable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on endorsement of prop4 in R5-221164, updated the max testable SNR and test point testability table Retained editor’s note for 256QAM since the associated fading backoff margin was not agreed in this meeting Updated testability table for CSI test cases -- Typo in the channel BW of test point 2-4 -- Table 7.1.1_1-2 updated to reflect testability of Demod test points -- Table 8.1.1_1-2 updated to reflect testability of CSI test points |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test points will remain untested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1_1, 8.1.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependent on R5-221164 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-221163r1 R5-221708 |  |  |  |  |  |  |  |  |

### 7.1.1_1	Applicability of test requirements due to maximum achievable SNR

Editor’s note: The applicability of test requirements with a modulation above 64 QAM is FFS.

Table 7.1.1_1-1 specifies the current assumption of maximum testable SNRBB for indirect farfield (IFF), PC3, Max device size ≤ 30 cm under fading conditions.

Table 7.1.1_1-1: maximum testable SNRBB under fading conditions for modulations up to 64 QAM

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | 30.6 | 27.5 | 24.4 |
| n258 mid | 30.6 | 27.5 | 24.4 |
| n259 mid | 20.4 | 17.2 | 14.1 |
| n260 mid | 24.4 | 21.2 | 18.2 |
| n261 mid | 30.6 | 27.5 | 24.4 |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 7.1.1_1-2 for indirect farfield (IFF), PC3, Max device size size ≤ 30 cm under fading conditions.

Table 7.1.1_1-2: Testability of test requirements due to maximum achievable SNR per band

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | x | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 16.0 | x | x | x | x | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | x | x | x |
|  | 2-4 | 200 | Yes | 15.8 | x | x | - | x | x |
|  | 2-5 | 50 | Yes | 16 | x | x | x | x | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | - | x | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | - | x | x |
| 7.2.2.2.1_3 | 1-4 | 50 | Yes | 22.0 | TBD | TBD | TBD | TBD | TBD |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |
| 7.3.2.2.3 | 1-1 | 100 | Yes | 4.7 | x | x | x | x | x |

<several sections skipped>

### 8.1.1_1	Applicability of test requirements due to maximum achievable SNR

Editor’s note: The applicability of test requirements with a modulation above 64 QAM is FFS.

The current assumption of maximum testable SNRBB for PC3, Max device size ≤ 30 cm under fading conditions is specified in Table 7.1.1_1-1.

The current assumption of maximum testable SNRBB for indirect far field (IFF), PC3, Max device size ≤ 30 cm without fading conditions is specified in Table 8.1.1_1-1.

Table 8.1.1_1-1: Current assumption of maximum testable SNRBB without fading for modulations up to 64 QAM

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | [28.7] | [25.5] | [22.5] |
| n258 mid | [28.7] | [25.5] | [22.5] |
| n259 mid | [18.4] | [15.2] | [12.1] |
| n260 mid | [22.5] | [19.3] | [16.3] |
| n261 mid | [28.7] | [25.5] | [22.5] |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 8.1.1_1-2 for indirect far field (IFF), PC3, Max device size ≤ 30 cm under fading conditions.

Table 8.1.1_1-2: Testability of test requirements due to maximum achievable SNR per band

| Test Case | Test point | CHBW / MHz | Fading |  | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 8.2.2.2.1.1 | 1 | 100 | No |  | 9 | x | x | x | x | x |
|  | 2 | 100 | No |  | 15 | x | x | x | x | x |
| 8.2.2.2.2.1 | 1 | 100 | Yes |  | 7 | x | x | x | x | x |
|  | 2 | 100 | Yes |  | 13 | x | x | x | x | x |
| 8.2.2.2.2.1_1 | 3 | 50 | Yes | 8 |  | TBD | TBD | TBD | TBD | TBD |
|  | 4 | 50 | Yes | 21 |  | TBD | TBD | TBD | TBD | TBD |
| 8.4.2.2.1 | 1 | 100 | Yes |  | 0 | x | x | x | x | x |
|  | 2 | 100 | Yes |  | 16 | x | x | x | x | x |
|  | 3 | 100 | Yes |  | 16 | x | x | x | x | x |

<End of changes>