3GPP TSG-RAN5 Meeting #94-e	R5-221867

# Electronic Meeting, 21st Feb – 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0479 | rev | 1 | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of minimum test time for 1% residual BLER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC -UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The minimum test time for 1% residual BLER test cases are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the minimum test time for 1% residual BLER test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases can’t be executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.6_1, 5.2.2.2.6_1, 5.2.3.1.6_1, 5.2.3.2.6_1, F.1.1.2, F.1.3.2, G.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

5.2.2.1.6_1.3.4	Test requirement

Table 5.2.2.1.6.0-3 defines the primary level settings.

The target BLER percentage for the downlink reference measurement channels specified in Annex FFS for each BLER test shall meet or exceed the specified value in Table 5.2.2.1.6_1.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.6_1.3.4-1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Target BLER | SNR (dB) |
| 1-1 | R.PDSCH.1-11.1 FDD | 10 / 15 | 16QAM, 0.54 | TDLA30-10 | 2x2, ULA Low | 1% (Note 1) | 1.6 + TT |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

5.2.2.2.6_1.3.4	Test requirement

Table 5.2.2.2.6.0-3 defines the primary level settings.

The target BLER percentage for the downlink reference measurement channels specified in Annex FFS for each BLER test shall meet or exceed the specified value in Table 5.2.2.2.6_1.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.2.6_1.3.4-1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Target BLER | SNR (dB) |
| 1-1 | R.PDSCH.1-16.1 TDD | 40 / 30 | 16QAM, 0.54 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 1% (Note 1) | 1.4 + TT |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

5.2.3.1.6_1.3.3_1	Message exceptions for SA

Same as 5.2.2.1.6_1.3.3_1.

5.2.3.1.6_1.3.3_2	Message exceptions for SA

Same as 5.2.2.1.6_1.3.3_1.

5.2.3.1.6_1.3.4	Test requirement

Table 5.2.3.1.6.0-3 defines the primary level settings.

The target BLER for the downlink reference measurement channels specified in Annex FFS for each BLER test shall meet or exceed the specified value in Table 5.2.3.1.6_1.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.3.1.6_1.3.4-1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Target BLER | SNR (dB) |
| 1-1 | R.PDSCH.1-11.1 FDD | 10 / 15 | 16QAM, 0.54 | TDLA30-10 | 2x4, ULA Low | 1% (Note 1) | -2.3 + TT |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

5.2.3.2.6_1.3.4	Test requirement

Table 5.2.3.2.6.0-3 defines the primary level settings.

The target BLER percentage for the downlink reference measurement channels specified in Annex FFS for each BLER test shall meet or exceed the specified value in Table 5.2.2.2.6_1.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.3.2.6_1.3.4-1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Target BLER | SNR (dB) |
| 1-1 | R.PDSCH.1-16.1 TDD | 40 / 30 | 16QAM, 0.54 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 1% (Note 1) | -2.6 +TT |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

F.1.1.2	Measurement of Demod Performance requirements

This clause defines the maximum test system uncertainty for Demod Performance requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.2-1.

Table F.1.1.2-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | ±2.0 | Same as in LTE |
| Signal to noise ratio uncertainty | dB | ±0.3 | Same as in LTE |
| Signal to noise ratio variation | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 1Tx | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 2Tx | dB | ±0.7 | Same as in LTE |

The maximum test system uncertainty for test cases defined in section 5 is defined in Table F.1.1.2-2.

Table F.1.1.2-2: Maximum test system uncertainty for FR1 demodulation performance test cases

| Subclause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA |  | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler ± 0.6 dB for test 1-6 ± 0.9 dB for test 1-7 |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) + SNR uncertainty due to finite test time2 Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.3 dB for 10Hz Doppler, otherwise ±0.0 dB For test point 1-6, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB For test point 1-7, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.9 dB |  |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA |  | Same as 5.2.2.1.1_1 |  | Same as 5.2.2.1.1_1 |  |
| < Unchanged lines skipped > |  |  |  |  |  |
| 5.2.2.1.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  | [± 0.6 dB] |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB NOTE: MU factor due to finite test time is FFS |  |
| 5.2.2.1.6_1 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA |  | FFS |  | The method of calculationg the overall system uncertainty is the same as 5.2.2.1.1_1 except: SNR uncertainty due to finite test time for 1% residual BLER FFS dB for 10Hz Doppler, otherwise FFS dB Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) + SNR uncertainty due to finite test time2 = FFS dB |  |
| 5.2.2.1.7_1 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.2.1.1_1 |  | Same as 5.2.2.1.1_1 |  |
| < Unchanged lines skipped > |  |  |  |  |  |
| 5.2.2.2.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.2.1.5_1 |  | Same as 5.2.2.1.5_1 |  |
| 5.2.2.2.6_1 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.2.1.6_1 |  | Same as 5.2.2.1.6_1 |  |
| 5.2.2.2.7_1 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.2.1.1_1 |  | Same as 5.2.2.1.1_1 |  |
| < Unchanged lines skipped > |  |  |  |  |  |
| 5.2.3.1.5_1 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.2.1.5_1 |  | Same as 5.2.2.1.5_1 |  |
| 5.2.3.1.6_1 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.2.1.6_1 |  | Same as 5.2.2.1.6_1 |  |
| 5.2.3.1.7_1 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.3.1.1_1 |  | Same as 5.2.3.1.1_1 |  |
| < Unchanged lines skipped > |  |  |  |  |  |
| 5.2.3.2.5_1 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.2.1.5_1 |  | Same as 5.2.2.1.5_1 |  |
| 5.2.3.2.6_1 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.2.1.6_1 |  | Same as 5.2.2.1.6_1 |  |
| 5.2.3.2.7_1 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  | Same as 5.2.3.1.1_1 |  | Same as 5.2.3.1.1_1 |  |

<Unchanged Text Skipped>

F.1.3.2	Measurement of Demod Performance requirements

The derivation of the test requirements for the test cases in section 5 is defined in Table F.1.3.2-1.

Table F.1.3.2-1: Derivation of Test Requirements (FR1 demodulation performance tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.6 dB for test 1-6 0.9 dB for test 1-7 | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| < Unchanged lines skipped > |  |  |  |
| 5.2.2.1.4_1 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.1.6_1 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | FFS | Formula: SNR + TT |
| 5.2.2.2.11_1 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| < Unchanged lines skipped > |  |  |  |
| 5.2.2.2.4_1 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.2.2.6_1 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | FFS | Formula: SNR + TT |
| 5.2.2.2.9_1 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |
| < Unchanged lines skipped > |  |  |  |
| 5.2.3.1.4_1 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.1.6_1 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | FFS | Formula: SNR + TT |
| 5.2.3.1.11_1 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |
| < Unchanged lines skipped > |  |  |  |
| 5.2.3.2.3_1 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.6_1 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | FFS | Formula: SNR + TT |
| 5.2.3.2.9_1 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | TBD | Formula: SNR + TT T-put limit unchanged |
| 5.2.3.2.11_1 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |

<Unchanged Text Skipped>

G.1.5	Minimum Test time

Editor's Note: Simulation method to derive minimum test time for FR2 needs to be evaluated.

If a pass fail decision in clause G.1.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed.

The tables below contain the minimum number of slots for FDD and TDD.

By simulations the minimum number of active subframes (carrying DL payload) was derived (MNAS), then adding inactive subframes to the active ones. (for TDD additional subframes contain no DL payload), then rounding up to full thousand.

Simulation method to derive minimum test time:

With a level, corresponding a throughput at the test limit (here 30 % or 70 % of the max. throughput) the preliminary throughput versus time converges towards the final throughput. The allowance of ± 0.2 dB around the above mentioned level is predefined by RAN5 to find the minimum test time. The allowance of ±0.2 dB maps through the function "final throughput versus level" into a throughput corridor. The minimum test time is achieved when the preliminary throughput escapes the corridor the last time. The two functions "final throughput versus level" and "preliminary throughput versus time" are simulation results, which are done individual for each demodulation scenario.

Figure G.1.5-1: Simulation method to derive minimum test time

Table G.1.5-1: Minimum Test time for PDSCH demodulation

| TDD UL-DL pattern |  | Reference Channel |  | Propagation condition |  | Demodulation scenario (doppler speed) |  | Minimum number of active subframes (MNAS) |  | MNAS to MNS Scaling factor (Note 3) |  | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NA |  | R.PDSCH.1-8.1 FDD |  | HST-750 |  | 750 Hz |  | FFS |  | 1.0526 |  | FFS |  |
| NA |  | R.PDSCH.1-8.1 FDD |  | TDLC300-600 |  | 600 Hz |  | 8000 (Note 1) |  | 1.0526 |  | 9000 |  |
| NA |  | R.PDSCH.1-1.1 FDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.0526 |  | 11000 |  |
| NA |  | R.PDSCH.1-1.2 FDD, R.PDSCH.1-2.1 FDD, R.PDSCH.1-5.1 FDD, |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.0526 |  | 22000 |  |
| NA |  | R.PDSCH.1-1.3 FDD R.PDSCH.1-2.2 FDD, R.PDSCH.1-2.3 FDD, R.PDSCH.1-2.4 FDD, R.PDSCH.1-2.5 FDD, R.PDSCH.1-3.1 FDD, R.PDSCH.1-3.2 FDD, R.PDSCH.1-3.3 FDD, R.PDSCH.1-3.4 FDD, R.PDSCH.1-4.1 FDD, R.PDSCH.1-12.1 FDD, R.PDSCH.2-1.1 FDD, |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.0526 |  | 79000 |  |
| NA |  | R.PDSCH.1-7.1 FDD, R.PDSCH.1-7.2 FDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.25 |  | 94000 |  |
| FR1.15-1 |  | R.PDSCH.1-1.1 TDD R.PDSCH.1-1.2 TDD |  | TDLA30-10 |  | 10Hz |  | 75000 (Note 1) |  | 2.8571 |  | 215000 |  |
| FR1.30-1A |  | R.PDSCH.2-1.1 TDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-1 |  | R.PDSCH.2-1.2 TDD, R.PDSCH.2-2.1 TDD, R.PDSCH.2-7.1 TDD |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.2903 |  | 26000 |  |
| FR1.30-1 |  | R.PDSCH.2-2.2 TDD R.PDSCH.2-2.3 TDD R.PDSCH.2-2.4 TDD R.PDSCH.2-2.5 TDD R.PDSCH.2-3.1 TDD, R.PDSCH.2-3.2 TDD, R.PDSCH.2-3.3 TDD, R.PDSCH.2-3.4 TDD, R.PDSCH.2-4.1 TDD, |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-1 |  | R.PDSCH.2-1.3 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.4815 |  | 112000 |  |
| FR1.30-2 |  | R.PDSCH.2-5.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-2 |  | R.PDSCH.2-17.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 5 |  | 375000 |  |
| FR1.30-3 |  | R.PDSCH.2-6.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.4815 |  | 112000 |  |
| FR1.30-4 |  | R.PDSCH.2-9.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-5 |  | R.PDSCH.2-11.1 TDD |  | TDLB100-400 |  | 400Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-6 |  | R.PDSCH.2-12.1 TDD |  | TDLB100-400 |  | 400Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR2.60-1 |  | R.PDSCH.4-1.1 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.33 |  | 27000 |  |
| FR2.120-1A |  | R.PDSCH.5-1.1 TDD |  | TDLC60-300 |  | 300 Hz |  | 10000 (Note 2) |  | 1.25 |  | 13000 |  |
| FR2.120-1 |  | R.PDSCH.5-2.1 TDD R.PDSCH.5-2.2 TDD R.PDSCH.5-2.3 TDD R.PDSCH.5-3.1 TDD |  | TDLA30-300 |  | 300 Hz |  | 10000 (Note 2) |  | 1.25 |  | 13000 |  |
| FR2.120-1 |  | R.PDSCH.5-1.2 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.25 |  | 25000 |  |
| FR2.120-2 |  | R.PDSCH.5-4.1 TDD R.PDSCH.5-5.1 TDD R.PDSCH.5-5.2 TDD R.PDSCH.5-6.1 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.33 |  | 27000 |  |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table G.1.5-1a: Minimum Test time for PDSCH demodulation with 1% BLER

| TDD UL-DL pattern | Reference Channel | Propagation condition | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- | --- |
| NA | R.PDSCH.1-11.1 FDD R.PDSCH.1-11.2 FDD | TDLA30-10 | 10 Hz | FFS (Note 1) | 1.1111 | FFS |
| FR1.30-1 | R.PDSCH.2-16.1 TDD R.PDSCH.2-16.2 TDD | TDLA30-10 | 10 Hz | FFS (Note 1) | 1.6667 | FFS |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed). Note 3: MNS/MNAS ratio decided by scheduling pattern (how much time is required to collect required number of active DL SFs). |  |  |  |  |  |  |

Table G.1.5-2: Minimum Test time for PDCCH demodulation

| Reference Channel | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- |
| R.PDCCH 1-1.1 FDD, R.PDCCH.1-1.3 FDD, R.PDCCH.1-2.1 FDD, R.PDCCH.1-2.2 FDD, R.PDCCH.1-2.3 FDD, R.PDCCH.1-2.4 FDD, R.PDCCH.1-2.5 FDD, R.PDCCH.1-2.6 FDD | 10, 100, 400 Hz | 100000 (Note 1) | 1.0526 | 106000 |
| R.PDCCH.2-1.1 TDD, R.PDCCH.2-1.2 TDD, R.PDCCH.2-2.1 TDD, R.PDCCH.2-1.3 TDD | 10, 100, 400 Hz | 100000 (Note 1) | 1.2903 | 130000 |
| R.PDCCH.5-1.1 TDD R.PDCCH.5-1.2 TDD R.PDCCH.5-1.3 TDD R.PDCCH.5-2.1 TDD | 75, 300 Hz | 100000 (Note 2) | 1.25 | 130000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |

<End of Changes >