3GPP TSG-RAN5 Meeting #94-e	R5-221880

# Electronic Meeting,  21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1569 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of several CA combinations to Reference Sensitivity test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | WE Certification, DISH Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Several CA combinations are introduced in RAN4 but missing 38.521-1 reference sensitivity test |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added several CA combos to reference sensitivity case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The added CA combos reference sensitivity cannot be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.3.2.3, 7.3A.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Corrected the meeting year in the cover page |  |  |  |  |  |  |  |  |

##### 7.3A.0.3.2	ΔRIB,c for Inter-band CA

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in subclause 7.3A.0 shall be increased by the amount given by ΔRIB,c defined in subclause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied.

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14] for the applicable operating bands.

7.3A.0.3.2.1	ΔRIB,c for two bands

Table 7.3A.0.3.2.1-1: ΔRIB,c due to CA (two bands)

| Inter-band CA configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n77 | n1 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n78 | n78 | 0.5 |
| CA_n3-n77 | n3 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n79 | n79 | 0.5 |
| CA_n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n8-n79 | n79 | 0.5 |
| CA n28-n75 | n28 | 0.2 |
| CA_n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n79 | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n41-n781 | n78 | 0.5 |
| CA_n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n48-n66 | n48 | 0.5 |
|  | n66 | 0.2 |
| CA_n48-n70 | n48 | 0.5 |
|  | n70 | 0.2 |
| CA_n75-n78 | n78 | 0.5 |
| CA_n76-n78 | n78 | 0.5 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. |  |  |

7.3A.0.3.2.2	Void

7.3A.0.3.2.3	ΔRIB,c for three bands

Table 7.3A.0.3.2.3-1: ΔRIB,c due to CA (three bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n78-n79 | n78 | 0.5 |
| CA_n26-n70-n71A | n26 | 0 |
|  | n70 | 0 |
|  | n71 | 0 |
| CA_n48-n66-n70A | n48 | 0.5 |
|  | n66 | 0.2 |
|  | n70 | 0.2 |
| CA_n48-n66-n71A | n48 | 0.2 |
|  | n70 | 0.2 |
|  | n71 | 0.2 |
| CA_n48-n70-n71A | n48 | 0.2 |
|  | n70 | 0.2 |
|  | n71 | 0.2 |
| CA_n66-n70-n71 | n66 | 0 |
|  | n70 | 0 |
|  | n71 | 0 |

7.3A.0.3.2.4	ΔRIB,c for four bands

Table 7.3A.0.3.2.4-1: ΔRIB,c due to CA (four bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |

#### ------------Many unchanged sections skipped here-------------

7.3A.2.5	Test requirement

For 3DL carrier aggregation, test parameters are specified in table 7.3A.2.4.1-1. For the CA configurations listed in table 7.3A.2.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1 for each non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2 for each non-SDL carrier for 4 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous CA with one uplink carrier and two or more downlink sub-blocks, the test requirement for SCC(s) shall be increased by ΔRIBNC  given in Table 7.3A.0.2.2-1. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.5-1: Reference sensitivity requirement for 3DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
| Intra-band contiguous 3DL CA | CA_n77D | - |
|  | CA_n78D | - |
| Intra-band non-contiguous 3DL CA | CA_n48(3A) | - |
|  | CA_n1A-n78(2A) | - |
|  | CA_n1A-n78A-n79A | - |
|  | CA_n26A-n66-n70A | - |
|  | CA_n26A-n66(2A) | - |
|  | CA_n48A-n66(2A) | - |
|  | CA_n48A-n71(2A) | - |
|  | CA_n48B-n66A | - |
|  | CA_n48B-n70A | - |
|  | CA_n48B-n71A | - |
|  | CA_n48(2A)-n66A | - |
|  | CA_n48(2A)-n70A | - |
|  | CA_n48(2A)-n71A | - |
|  | CA_n48A-n66A-n70A | - |
|  | CA_n48A-n66A-n71A | - |
|  | CA_n48A-n70A-n71A | - |
|  | CA_n66A-n70A-n71A | - |
|  | CA_n66A-n71(2A) |  |
|  | CA_n66(2A)-n70A | - |
| Inter-band 3DL CA | CA_n66(2A)-n71A | - |
|  | CA_n66B-n70A | - |
|  | CA_n66B-n71A | - |
|  | CA_n70A-n71A(2A) |  |
|  | CA_n1A-n78C |  |
|  | CA_n29A-n66A-n70A | - |
| SDL configuration | CA_n29A-n66B | - |
|  | CA_n29A-n66(2A) | - |