3GPP TSG-RAN5 Meeting # 94-e	R5-221897

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1530 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MOP test cases for PC2 CA_n3A-n78A with UL CA_n3A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_CA_R17_2BDL_2BUL-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MOP test cases for CA_n3A-n78A with UL CA_n3A-n78A need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PC2 test requirements for MOP test case for PC2 CA_n3A-n78A with UL CA_n3A-n78A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: correct meeting date information on the coversheet |  |  |  |  |  |  |  |  |

<Start of change>

6.2A.1.1	UE maximum output power for CA (2UL CA)

<Unchanged Part Skipped>

6.2A.1.1.5	Test requirement

The maximum output power for CA, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.1.1.5-1 for Inter-band 2 UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.1.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n28A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n40A-n41A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2+TT/-32-TT |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n50A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: Void. NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refers to Table6.2A.1.1.5-2. |  |  |  |  |  |  |  |  |

Table 6.2A.1.1.5-1a: Maximum Output Power test requirement for Power Class except class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n41A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n3A-n78A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n28A-n41A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n28A-n79A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| CA_n40A-n41A |  |  | 267 | +2+TT/-32-TT |  |  |  |  |
| NOTE 1: Void. NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refer to Table6.2A.1.1.5-2. NOTE 7: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. |  |  |  |  |  |  |  |  |

<End of change>