3GPP TSG-RAN5 Meeting #94-e	R5-222039

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2205 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to cl 4.5.3 RRC_INACTIVE generic procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, pc_noOf_PDUsNewConnection is not handled in RRC_INACTIVE generic procedure specified in TS 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add steps to handle pc_noOf_PDUsNewConnection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail some TCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-220542r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.5.3	RRC_INACTIVE

4.5.3.1	Initiation

The SS shall:

1>	if connectivity is NR

2>	use 1 NR cell, default parameters;

2>	perform according to the table 4.5.3.2-1: NR RRC_INACTIVE;

4.5.3.2	Procedures

Table 4.5.3.2-1: NR RRC_INACTIVE

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-19a1 | Same as table 4.5.2.2-2, steps 1-19a1. | - | - | - | - |
| - | EXCEPTION: Steps 19Aa1-19Ab9 describes behaviour depending UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 19Aa1 | IF pc_noOf_PDUsNewConnection = 0 AND Test Loop Function=On, table 4.5.2.2-2 steps 19Aa1-19Aa2 take place. | - | - | - | - |
| 19 Ab1 | IF pc_noOf_PDUsNewConnection > 0 THEN the SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 19 Ab2-Ab9 | Steps 1-8 of the NR RRC_IDLE extension procedure specified in table 4.5.2.2-4, take place. | - | - | - | - |
| 20 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: RRCRelease | - | - |

<End of modified section 1>