3GPP TSG-RAN5 Meeting #94-e	R5-220048r1

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2168 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of NR CA configurations for Protocol testing with NR CA 3CC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, WE Certification, DISH Network, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IioT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is no clarification for the applicability of Inter-band NR CA configurations within FR1 (three bands). NR FR1 interband CA configurations table is missing several combinations having test requirements introduced to RF test specification 38.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add applicability of protocol testing with NR CA 3CC in Table 4.3.1.1.2.2-1. 2. Addition of NR CA combinations to NR interband configurations table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether Inter-band NR CA configurations within FR1 (three bands) can be used for protocol testing with NR CA 3CC. Several NR CA interband combination configurations are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: 1. Merge the changes of Table 4.3.1.1.2.2-1 in R5-220950. 2. Add WE Certification, DISH Network and Ericsson as the co-authors of this CR. 3. Add Note3 in Table 4.3.1.1.2.2-1. 4. Set the SIG applicability column to “No”, because no test case uses the combination for the time being. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.3.1.1.2.2	NR inter-band CA configurations in FR1 (three bands)

Table 4.3.1.1.2.2-1: Inter-band NR CA configurations within FR1 (three bands)

| NR CA Configuration (Note 3) |  | Uplink NR CA configuration |  | NR CA downlink configuration band 1 |  | NR CA downlink configuration band 2 |  | NR CA downlink configuration band 3 |  | NR CA uplink configuration band 1 |  | NR CA uplink configuration band 2 |  | NR CA uplink configuration band 3 |  | Applicable for protocol testing (Note 2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26A-n66A-n70A | CA_n26A-n66A | n26A | n66A | n70A | n26A | n66A | - | No |
|  | CA_n26A-n70A | n26A | n66A | n70A | n26A | - | n70A | No |
| CA_n29A-n66A-n70A |  | - |  | n29A (Note1) |  | n66A |  | n70A |  | - |  | - |  | - |  | No |  |
| CA_n29A-n66B-n70A |  | - |  | n29A (Note1) |  | CA_n66B |  | n70A |  | - |  | - |  | - |  | No |  |
| CA_n29A-n66(2A)-n70A |  | - |  | n29A (Note1) |  | CA_n66(2A) |  | n70A |  | - |  | - |  | - |  | No |  |
| CA_n48A-n66A-n70A | CA_n48A-n66A | n48A | n66A | n70A | n48A | n66A | - | No |
|  | CA_n48A-n70A | n48A | n66A | n70A | n48A | - | n70A | No |
| CA_n48A-n66A-n71A | CA_n48A-n66A | n48A | n66A | n71A | n48A | n66A | - | No |
|  | CA_n48A-n71A | n48A | n66A | n71A | n48A | - | n71A | No |
|  | CA_n66A-n71A | n48A | n66A | n71A | - | n66A | n71A | No |
| CA_n48A-n70A-n71A | CA_n48A-n70A | n48A | n70A | n71A | n48A | n70A | - | No |
|  | CA_n48A-n71A | n48A | n70A | n71A | n48A | - | n71A | No |
|  | CA_n70A-n71A | n48A | n70A | n71A | - | n70A | n71A | No |
| CA_n66A-n70A-n71A | CA_n66A-n71A | n66A | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | n66A | n70A | n71A | - | n70A | n71A |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A | CA_n66B | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | CA_n66B | n70A | n71A | - | n70A | n71A |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A | CA_n66(2A) | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | CA_n66(2A) | n70A | n71A | - | n70A | n71A |  |
| Note 1: This band cannot be used as PCell. Note 2: Protocol testing is limited to NR CA configurations with 3CC. Note 3: The band with the lowest UL frequency is used as PCell if nothing else is specified for in the table or in the test case for the specific configuration. |  |  |  |  |  |  |  |  |

<<< END OF CHANGES >>>