3GPP TSG-RAN5 Meeting #94-e	R5-220083r1

# Electronic Meeting, 21st February– 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1492 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new V2X test cases in 6.3E.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 6.3E.2 Transmit OFF power for V2X is still missing in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.3E.2 Transmit OFF power for V2X was introduced to the spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 6.3E.2 Transmit OFF power for V2X will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3E.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Extra space in sub clause numbering 6.3E.2.1.2 to 6.3E.2.1.5, 6.3E.2.1D.2 to 6.3E.2.1D.5, and 6.3E.2.2.2 to 6.3E.2.2.5 were removed. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.3E.1.1D.5	Test requirement

The minimum output power, derived in step 2 shall not exceed the values specified in Table 6.3E.1.1D.5-1.

Table 6.3E.1.1D.5-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -30+TT | 9.375 |
| 20 | -30+TT | 19.095 |
| 30 | -28.2+TT | 28.815 |
| 40 | -27+TT | 38.895 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3E.1.1D.5-2 |  |  |

Table 6.3E.1.1D.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | FFS | FFS |

6.3E.2	Transmit OFF power for V2X

6.3E.2.0	Minimum conformance requirements

6.3E.2.0.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 5.2E.1-1, the requirements specified in Table 6.3E.2.0.1-1 apply.

Table 6.3E.2.0.1-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -50 | 9.375 |
| 20 | -50 | 19.095 |
| 30 | -50 | 28.815 |
| 40 | -50 | 38.895 |

For NR V2X UE supporting SL MIMO, the transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3E.2.0.1-1 for single carrier. Transmit off power is defined as the mean power in at least one sub-frame 1 ms.

6.3E.2.0.2	Transmit OFF power for V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.3.2 shall apply for the uplink in licensed band and the requirements specified in Table 6.3E.2.0.1-1 shall apply for the sidelink in licensed band or Band n47.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3E.2.

6.3E.2.1	Transmit OFF power for V2X / non-concurrent operation

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- 6.3E.3.2.1 General time mask for V2X / non-concurrent operation is FFS.

6.3E.2.1.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

An excess Transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3E.2.1.2	Test applicability

The requirements of this test apply in test case 6.3E.3.2.1 General time mask for V2X / non-concurrent operation to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.3E.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3E.2.0.

6.3E.2.1.4	Test description

This test is covered by clause 6.3E.3.2.1 General time mask for V2X / non-concurrent operation.

6.3E.2.1.5	Test requirement

The requirement for the Transmit OFF power shall not exceed the values specified in Table 6.3E.2.1.5-1.

Table 6.3E.2.1.5-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -50+TT | 9.375 |
| 20 | -50+TT | 19.095 |
| 30 | -50+TT | 28.815 |
| 40 | -50+TT | 38.895 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3E.2.1.5-2 |  |  |

Table 6.3E.2.1.5-2: Test Tolerance (Transmit OFF power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |

6.3E.2.1D	Transmit OFF power for V2X / non-concurrent operation / SL-MIMO

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- 6.3E.3.2.1D General time mask for V2X / non-concurrent operation / SL-MIMO is FFS.

6.3E.2.1D.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

An excess Transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3E.2.1D.2	Test applicability

The requirements of this test apply in test case 6.3E.3.2.1D General time mask for V2X / non-concurrent operation / SL-MIMO to all types of NR UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.3E.2.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3E.2.0.

6.3E.2.1D.4	Test description

This test is covered by clause 6.3E.3.2.1D General time mask for V2X / non-concurrent operation / SL-MIMO.

6.3E.2.1D.5	Test requirement

The requirement for the Transmit OFF power shall not exceed the values specified in Table 6.3E.2.1D.5-1.

Table 6.3E.2.1D.5-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -50+TT | 9.375 |
| 20 | -50+TT | 19.095 |
| 30 | -50+TT | 28.815 |
| 40 | -50+TT | 38.895 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3E.2.1D.5-2 |  |  |

Table 6.3E.2.1D.5-2: Test Tolerance (Transmit OFF power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |

6.3E.2.2	Transmit OFF power for V2X / con-current operation

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- 6.3E.3.2.2 General time mask for V2X / con-current operation is FFS.

6.3E.2.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

An excess Transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3E.2.2.2	Test applicability

The requirements of this test apply in test case 6.3E.3.2.2 General time mask for V2X / con-current operation to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.3E.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3E.2.0.

6.3E.2.2.4	Test description

This test is covered by clause 6.3E.3.2.2 General time mask for V2X / con-current operation.

6.3E.2.2.5	Test requirement

The requirement for the Transmit OFF power shall not exceed the values specified in Table 6.3E.2.2.5-1.

Table 6.3E.2.2.5-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 10 | -50+TT | 9.375 |
| 20 | -50+TT | 19.095 |
| 30 | -50+TT | 28.815 |
| 40 | -50+TT | 38.895 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3E.2.2.5-2 |  |  |

Table 6.3E.2.2.5-2: Test Tolerance (Transmit OFF power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |

6.4	Transmit signal quality

In this clause a multitude of results are derived, all using one common algorithm returning these results: Global In-Channels TX-Test Annex E. Each sub clause of this clause contains a procedure and test requirements described for a specific measurement. If all relevant test parameters in different sub clauses are the same, then the results, returned by the Global In-Channel TX-Test, may be used across the applicable sub clauses.

<< End of changes >>