3GPP TSG-RAN5 Meeting #94-e	R5-220129r1

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5072 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of date for EPC TC 9.1.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Due to OS limitation, e.g. Andriod S, the current date “2010 12 April” is out of date and is not allowed to be set in UE when checking the NITZ by MMI. Therefore it is suggested to update the date to a new fixed date for conformance requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the date to a new value in step 1 and 3d1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformat UE will fail this case unfarily. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: update the date as <Current Year>/12/31. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

9.1.5.1	EMM information procedure

9.1.5.1.1	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED state and UE supporting the EMM information message }

ensure that {

when { UE receives an EMM Information message }

then { UE accepts the message and uses the contents to update appropriate information stored within the UE }

}

9.1.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clause 5.4.5.3.

[TS 24.301, clause 5.4.5.3]

When the UE (supporting the EMM information message) receives an EMM INFORMATION message, it shall accept the message and optionally use the contents to update appropriate information stored within the UE.

9.1.5.1.3	Test description

9.1.5.1.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 9.1.5.1.3.1-1

Table 9.1.5.1.3.1-1: USIM configuration

| USIM field | Priority | Value |
| --- | --- | --- |
| EFUST |  | Services 19 and 51 are not supported |

Preamble:

-	the UE is in state Generic RB established (state 3) on cell A according to TS 36.508 [18].

9.1.5.1.3.2	Test procedure sequence

Table 9.1.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Void | - | - | - | - |
| - | EXCEPTION: Steps 0Aa1 and 0Ab1 describe behaviour that depends on the UE release; the "lower case letter" identifies a step sequence that take place if the UE is of particular release. | - | - | - | - |
| 0Aa1 | IF the UE is (Rel-11 or higher) and (pc_DaylightSavingTime or pc_UniversalAndLocalTimeZone) THEN Switch on Time Zone reporting on the UE (Note 3) | - | - | - | - |
| 0Ab1 | IF the UE is lower than Rel-11 THEN configure the UE with automatic time zone update (Note 0) | - | - | - | - |
| 1 | The SS transmits an EMM INFORMATION message. | <-- | EMM INFORMATION | - | - |
| 2 | Check: Does the UE transmit in the next 5 seconds an EMM STATUS message with cause #97 "message type non-existent or not implemented"? | --> | EMM STATUS | 1 | F |
| - | EXCEPTION: Steps 2Aa1 to 3d2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 2Aa1 | IF ( pc_DaylightSavingTime AND pc_ProvideDST_inUse ) THEN Check: Does the UE assume that this daylight saving time applies to the tracking area of the current cell and is presented to the MS user at the earliest opportunity? (Note 8) Operator Action: The use of the supported Fields is checked: DaylightSavingTime: 1 (Note 7) | - | - | 1 | P |
| 2Aa2 | Void | - | - | - | - |
| 3a1 | IF pc_FullNameNetwork THEN Check: Does the UE associate the "full length name of the network" with the MCC and MNC contained in the last visited tracking area identification and is presented to the MS user at the earliest opportunity? Operator Action: “FullName12345678” is presented to the MS user. (Note 0, 1) | - | - | 1 | P |
| 3a2 | Void | - | - | - | - |
| 3b1 | IF pc_ShortNameNetwork THEN Check: Does the UE associate the "abbreviated name of the network" with the MCC and MNC contained in the last visited tracking area identification and is presented to the MS user at the earliest opportunity? Operator Action: "SName123" is presented to the MS user (Note 1) | - | - | 1 | P |
| 3b2 | Void | - | - | - | - |
| 3c1 | IF pc_LocalTimeZone THEN Check: Does the UE assume that this time zone applies to the tracking area of the current cell and is presented to the MS user at the earliest opportunity? Operator Action: The use of the supported Fields is checked: Timezone: GMT+1 (Note 2) | - | - | 1 | P |
| 3c2 | Void | - | - | - | - |
| 3d1 | IF pc_UniversalAndLocalTimeZone THEN Check: Does the UE assume that this time applies to the tracking area of the current cell and is presented to the MS user at the earliest opportunity? (Note 3, Note 8) Operator Action: The use of the supported Fields is checked: Local Time: Year: <Current Year> Month: December Day: 31st Hour: 14 Hours (Note 4) | - | - | 1 | P |
| 3d2 | Void | - | - | - | - |
| Note 0: AT command +CTZU is assumed to be used. Note 1: AT command +COPS is assumed to be used for check. Note 2: AT command +CCLK is assumed to be used for check. Note 3: AT command +CTZR is assumed to be used for check for Rel-11 onwards. Note 4: Current Year is derived by the SS and the minutes and seconds are not checked. Note 5: Void. Note 6: Void Note 7: If (Rel-11 or higher Release) and Time Zone reporting is enabled at Step 0, the UE returns the result code +CTZE. Note 8: In case of Rel-10 or lower releases: CTZU has been used to trigger UE report. A time zone report in the CTZE structure is expected. Triggering a time zone report in the CTZE structure is only required if pc_DaylightSavingTime or pc_UniversalAndLocalTimeZone are supported. |  |  |  |  |  |

9.1.5.1.3.3	Specific message contents

Table 9.1.5.1.3.3-1: EMM INFORMATION (step 1, Table 9.1.5.1.3.2-1)

| Derivation Path: 36.508 table 4.7.2-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Full name for network | "C63A9BED0CB7CB31D98C56B3DD70" O | "FullName12345678", Note 1 |  |
| Short name for network | "5367B85D8EC966" O | "SName123", Note 1 |  |
| Local time zone | "40" O | "GMT+1", Note 1, Note 2 |  |
| Universal time and local time zone | "xx211331832540" O | "<Current Year> 31 December 13:38:52 GMT+1", Note 1, Note 2 |  |
| Network daylight saving time | "01" O | "+1 hour adjustment for Daylight Saving Time", Note 1 |  |
| Note 1: Hard coded values have been chosen to allow for consistent/comparable SS behaviour. Note 2: Daylight Saving Time is included in the Local Time Zone. |  |  |  |

Table 9.1.5.1.3.3-2: Message EMM STATUS (step 2, Table 9.1.5.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EMM cause | '0110 0001'B | Message type non-existent or not implemented |  |

[End of change]