3GPP TSG-RAN5 Meeting #94-e	R5-220155r1

# Electronic Meeting, 21st Feb 2022 – 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0284 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 7.4a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update to use the generic procedure A.4.1a. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure A.4.1a has been used. The Pre-test conditions has been clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be aligned with the generic procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-5 CR 0285 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision R5-220155r1. The cover sheet is updated. The Specific message contents clarified. |  |  |  |  |  |  |  |  |

<Start of first modified section>

## 7.4a	MTSI MO Voice Call with preconditions at both originating UE and terminating UE / Default Configuration / 5GS

7.4a.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use preconditions and configured to use the EVS default configuration included first in the initial SDP offer }

ensure that {

when { UE is being made to initiate a voice call }

then { UE sends INVITE for voice call with preconditions and EVS default configuration }

}

(2)

with { UE having sent INVITE with preconditions }

ensure that {

when { UE receives 100 Trying followed by 183 Session Progress }

then { UE sends PRACK for 183 Session Progress }

}

(3)

with { UE having sent PRACK }

ensure that {

when { UE receives 200 OK for PRACK and resources are available }

then { UE sends UPDATE }

}

(4)

with { UE having sent UPDATE }

ensure that {

when { UE receives 200 OK for UPDATE followed by 180 Ringing sent reliably }

then { UE sends PRACK for 180 Ringing }

}

(5)

with { UE having sent PRACK for 180 Ringing }

ensure that {

when { UE receives 200 OK for PRACK followed by 200 OK for INVITE }

then { UE sends ACK }

}

7.4a.2	Conformance Requirements

Same as test case 7.4.

7.4a.2A Profile requirements (Informative)

Same as test case 7.4 with the following additions:

[NG.114 Version 1.0 and 2.0, clause 3.2.2.3]:

The configuration of the EVS payload type to be included first in the initial SDP offer for EVS is defined by the EVS/Br and EVS/Bw parameters as specified in Annex C.3, which must be configured to one of the five above EVS Configurations.

[NG.114 Version 1.0 and 2.0, clause C.3]:

Table C.3-1 contains the configuration parameters with their default values, that must be supported by the UE and the network. The UE must use the default value for each parameter in Table C.3-1 unless configured differently as described in Annex C.2.

…

| EVS/Br | 5.9-24.4 | Section 15.2 of 3GPP TS 26.114 [16] (/<X>/Speech/<X>/EVS/Br) and section 5 of 3GPP TS 26.441 [56] (Table 1) | 3.2.2.3 |
| --- | --- | --- | --- |
| EVS/Bw | nb-swb | Section 15.2 of 3GPP TS 26.114 [16] (/<X>/Speech/<X>/EVS/Bw) and section 5 of 3GPP TS 26.441 [56] (Table 1) | 3.2.2.3 |

7.4a.3	Test description

7.4a.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS

7.4a.3.2	Test procedure sequence

Table 7.4a.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call. | - | - |  |  |
| 1A-1F | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - |  |  |
| 2 | UE sends INVITE with first SDP offer (Step 1 of Annex A.4.1a) | --> | INVITE | 1 | P |
| 3 | SS sends a 100 Trying provisional response (Step 2 of Annex A.4.1a) | <-- | 100 Trying |  |  |
| 4 | SS sends an SDP answer (Step 3 of Annex A.4.1a) | <-- | 183 Session Progress |  |  |
| 5 | UE acks 183 Session Progress (Step 4 of Annex A.4.1a) | --> | PRACK | 2 | P |
| 6 | SS responds to PRACK (Step 5 of Annex A.4.1a) | <-- | 200 OK |  |  |
| 7 | UE sends UPDATE with second SDP offer (Step 6 of Annex A.4.1a) | --> | UPDATE | 3 | P |
| 8 | SS sends an SDP answer (Step 7 of Annex A.4.1a) | <-- | 200 OK |  |  |
| 9 | SS sends 180 Ringing reliably (Step 8 of Annex A.4.1a) | <-- | 180 Ringing |  |  |
| 10 | UE acks 180 Ringing (Step 9 of Annex A.4.1a) | --> | PRACK | 4 | P |
| 11 | SS responds to PRACK (Step 10 of Annex A.4.1a) | <-- | 200 OK |  |  |
| 12 | SS responds to INVITE (Step 11 of Annex A.4.1a) | <-- | 200 OK |  |  |
| 13 | UE acks 200 OK for INVITE (Step 12 of Annex A.4.1a) | --> | ACK | 5 | P |

7.4a.3.3	Specific message contents

None.

<End of modified section>