3GPP TSG-RAN5 Meeting #94-e	R5-220455r2

# Electronic Meeting, 21st February 2022 – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2367 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G Rel-15: Test Models updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. In clause 7.3.5.4, when preparing the prose CR for RAN5#93-e, the changes in steps 8a/b were missed, these changes were endorsed in R5w210304r1. When the RRC re-establishment procedure in the same cell is happening due to a handover failure, UE may not send any HARQ feedback causing SS to retransmit the RRCReconfiguration message. The SRBs/DRBs needs to be released and reconfigured earlier to stop SS doing retransmissions during the re-establishment procedure. 2. There is no model to test the NE-DC RRC and NE-DC PDCP test cases defined in the 5GS work plan. 3. Only 3 IP addresses are defined in the TTCN meaning it cannot support all of the DNNs defined in TS 38.508-1. 4. The execution guidelines need to be updated to take into account test prose updates and the new delivered test cases. 5. For the MAC NR CA Test Model, there is one instace of PSCell mentioned, whereas it should be PCell. 6. At RAN5 TTCN Workshop #56, it has been agreed to apply the usage of MacBearerRouting also for MAC CA test models. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Corrected steps 8a/b of the RRC Re-establishment procedure. Moved step 3 to step 0. 2. Introduced the NE-DC L3 and NE-DC PDCP test models. 3. PIXITs for 3 new IP addresses are added. 4. Updated clause 11. 5. PSCell corrected into PCell. 6. Details on usage of MacBearerRouting field has been added in clauses 5.1.2.3.2 and 5.2.2.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. The re-establishment sequence will not be fully documented 2. NE-DC test cases cannot be tested 3. Not all DNN/slices can be configured at the same time 4. Execution guideline will remain incomplete. 5. The specification will remain incorrect. 6. RAN5 TTCN Workshop #56 will not be documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.1.2.3.2, 5.2.1.5, 5.2.1.5A(new), 5.2.2.2.2, 5.2.2.2.3(new), 5.2.2.4.2, 7.3.5.4, 9.2, 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Further updates to the NR RRC connection re-establishment procedure in clause 7.3.5.4 has been found to be required. Revision2: Added changes related to TTCN Workshop document R5w220022. |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

ASP	Abstract Service Primitive

ATS	Abstract Test Suite

AWGN	Additive White Gaussian Noise

CA	Carrier Aggregation

CBRA	Contention Based Random Access

CCE	Control Channel Element

CFRA	Contention Free Random Access

CORESET	Control Resource Set

DAI	Downlink Assignment Index

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FR1	Frequency Range 1

FR2	Frequency Range 2

HO	Handover

H-SFN	Hyper SFN

ICS	Implementation Conformance Statement

IUT	Implementation Under Test

IXIT	Implementation eXtra Information for Testing

LSB	Least Significant Bit

MCG	Master Cell Group

MN	Master Node

MSB	Most Significant Bit

NE-DC	NR-E-UTRA Dual Connectivity

NR	NR Radio Access

NR-DC	NR-NR Dual Connectivity

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSCell	Primary SCG Cell

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RA	Random Access

RACH	Random Access Channel

RAR	Random Access Response

RAT	Radio Access Technology

RMSI	Remaining Minimum SI

SCG	Secondary Cell Group

SFN	System Frame Number

SN	Secondary Node

SRS	Sounding Reference Signal

SS	System Simulator

SSB	Synchronization Signal and PBCH block

SSS	Secondary Synchronisation Signal

TC	Test Case

UL	Uplink

UL-SCH	Uplink Shared Channel

UT	Upper Tester

VNG	Virtual Noise Generator

<End of modified section>

## <Start of next modified section>

##### 5.1.2.3.2	NR carrier aggregation

Figure 5.1.2.3.2-1: Test model for EN-DC MAC CA testing

The EN-DC MAC CA test model builds on top of the EN-DC MAC test model, with the differences specified hereafter.

On the SS NR side, in the SCG there is one PSCell and one SCell configured:

-	PSCell only: On DRBs the NR RLC is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no NR PDCP configured in the SS. The ports are directly above NR RLC.

-	PSCell / SCell: Layer 1 is configured in the normal way. NR MAC is configured in a special mode, where it does not add any MAC headers in DL and /or not remove any MAC headers in UL directions respectively at DRB port. In this case, the TTCN shall provide the final MAC PDU, including padding. Except for this, the MAC layer shall perform all of its other functions.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception in PSCell over system indication port, if configured. In a similar way the reception of PRACH preambles in PSCell / SCell is reported by SS over the same port, if configured.

The NR data routing between the RLC layer of PSCell and the lower layers of either PSCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

<End of modified section>

## <Start of next modified section>

#### 5.2.1.5	NR-NR dual connectivity

Figure 5.2.1.5-1: Test model for NR/5GC NR-DC Layer3: MCG and SCG

Figure 5.2.1.5-2: Test model for NR/5GC NR-DC Layer3 testing: MCG and split DRB

Figure 5.2.1.5-3: Test model for NR/5GC NR-DC Layer3 testing: Split SRB(s), DRBs removed for clarity

The NR/5GC Layer3 NR-DC test model builds on top of the NR/5GC Layer3 test model, with the differences specified hereafter:

-	SRB1 and SRB2 are configured only in the PCell,

-	SRB3 can be configured in the PSCell,

-	The NR PDCP can be configured in split for one or more SRBs or DRBs. In this case NR PDCP is configured in one cell and a proxy PDCP will be configured in the other cell. The SS shall route data to/from either cell, via the routing information provided.

#### 5.2.1.5A	NR-E-UTRA dual connectivity

Figure 5.2.1.5A-1: Test model for NR/5GC NE-DC Layer3: MCG and SCG

Figure 5.2.1.5A-2: Test model for NR/5GC NE-DC Layer3 testing: MCG and split DRB, E-UTRA terminated

Figure 5.2.1.5A-3: Test model for NR/5GC NE-DC Layer3 testing: MCG and split DRB, NR terminated

Figure 5.2.1.5A-4: Test model for NR/5GC NE-DC Layer3 testing: Split SRB(s), DRBs removed for clarity

The NR/5GC Layer3 NE-DC test model builds on top of the NR/5GC Layer3 test model, with the differences specified hereafter:

-	SRBs 0, 1 and 2 are configured only in NR.

-	On the SS side E-UTRA L1, MAC and RLC are configured in normal way and shall perform all their functions.

-	Only NR PDCP is configured in both NR and E-UTRA (E-UTRA PDCP is not used).  This can be configured in split for one or more SRBs or DRBs. In this case NR PDCP is configured in one cell and a proxy PDCP will be configured in the other cell. The SS shall route data to/from either cell, via the routing information provided.

-	SDAP can be configured in both NR and E-UTRA.

<End of modified section>

## <Start of next modified section>

##### 5.2.2.2.2	NR-NR dual connectivity

Figure 5.2.2.2.2-1: Test model for NR/5GC NR-DC PDCP testing (split DRB)

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer.

On UE side Ciphering is enabled as null algorithm and ROHC is not configured.

The SS configures in the First Cell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

The SS configures in the Second Cell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured as Proxy. DRB data will be sent between the NR PDCP Proxy and the NR PDCP.

Note that the First Cell refers to MCG PCell and the Second Cell to SCG PSCell, or vice versa.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.2.2.2.3	NR-E-UTRA dual connectivity

Figure 5.2.2.2.3-1: Test model for NE-DC PDCP testing (split DRB)

The UE is configured in Test Loop Mode, to loop back the user domain data above NR PDCP layer.

On UE side Ciphering is enabled as null algorithm and ROHC is not configured.

Test Loop Mode can be active on split DRB as shown in Figure 5.2.2.2.3-1.

On the SS NR, SRBs 0,1 and 2 are configured as per Layer3 test model. The Layer1, MAC and RLC of the MCG bearer are configured in normal operation. The PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

On the SS E-UTRA the Layer1, MAC and RLC of the SCG bearer are configured in normal operation. The PDCP layer of the SCG leg is configured as Proxy mode, TTCN shall configure E-UTRA for NE-DC PDCP testing only when a Test Loop Mode is active on a split DRB.

The SS shall route DL PDCP PDUs from TTCN to PCell and/or PSCell and SS shall indicate that the UL PDCP PDU is received from PCell or PSCell.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

<End of modified section>

##### 5.2.2.4.2	NR carrier aggregation

Figure 5.2.2.4.2-1: Test model for NR/5GC MAC CA testing

The NR/5GC MAC CA test model builds on top of the NR/5GC MAC test model, with the differences specified hereafter.

On the SS NR side, there is one PCell and one SCell configured:

-	PCell only: On DRBs the NR RLC is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no NR PDCP and SDAP configured in the SS. The ports are directly above NR RLC.

-	PCell / SCell: Layer 1 is configured in the normal way. NR MAC is configured in a special mode, where it does not add any MAC headers in DL and /or not remove any MAC headers in UL directions respectively at DRB port. In this case, the TTCN shall provide the final MAC PDU, including padding. Except for this, the MAC layer shall perform all of its other functions.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception in PCell over system indication port, if configured. In a similar way the reception of PRACH preambles in PCell / SCell is reported by SS over the same port, if configured.

The NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

<End of modified section>

## <Start of next modified section>

#### 7.3.5.4	RRC connection re-establishment

In general, the re-establishment is done without activation time, i.e. the timing information for configuration of the SS and sending of the RRCReestablishment is ‘Now'.

The Source Cell and Target cell can be the same cell.

0.	Source Cell:	If the Source Cell is the same as the Target Cell:

Release SRBs and DRBs.

(Re-)configure SRBs.

1.	Target Cell:		Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI.

NOTE 1:	The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

2.	Target Cell:	Receive RRCReestablismentRequest.

3.

Void

NOTE 2:	Void

4.	Target Cell:	Void

5.	Target Cell:	Activate Security.

6.	Target Cell:	Send RRCReestablishment.

7.	Target Cell:	Receive RRCReestablishmentComplete.

8.	Target Cell:	Reconfigure UL DCCH/DTCH DCI on USS.

8a. Target Cell	If the Source Cell is different from the Target Cell

Configure UL grant “OnSR” and stop periodic TA.

Else

Stop periodic TA.

8b	Target Cell	Reconfigure DRBs

9.	Target Cell:	Send RRCReconfiguration.

10.	Target Cell:	Receive RRCReconfigurationComplete.

11. Target Cell: 	Reconfigure DL DCCH/DTCH DCI on USS.

NOTE 3:	The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

12. Target Cell:	Re-configure RACH procedure as for initial access, enable TA transmissions.

NOTE 4:	The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

13.	Source Cell:	If the Source Cell is different from the Target Cell:

Release security.

Release  SRBs and DRBs.

(Re-)configure SRBs.

Stop periodic UL grant and periodic TA.

NOTE 5:	The FollowOnFlag is set to true in the ASP configuring the SRBs

<End of modified section>

<Start of next modified section>

## 9.2	E-UTRA and NR PIXIT

Table 9.2-1: E-UTRA and NR PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_ENDC_BandCombination | ENDC_BandCombination_Type | DC_1A_n28A |  | EN-DC Band Combination |
| px_ENDC_CA_BandCombination | ENDC_CA_BandCombination_Type | DC_1A_n28A_n78A |  | EN-DC CA Band Combination |
| px_ENDC_SecondaryBandCombination | ENDC_BandCombination_Type | DC_1A_n77A |  | Secondary EN-DC Band Combination |
| px_IPv4_Address_Other | charstring |  |  | Other IPv4 Address (see NOTE 4) |
| px_IPv6_Address_Other | charstring |  |  | Other IPv6 Address (see NOTE 4) |
| px_IPv4_Address4_NW | charstring |  |  | IPv4 Gateway Address in PDN4 |
| px_IPv6_Address4_NW | charstring |  |  | IPv6 Gateway Address in PDN4 |
| px_IPv4_Address4_UE | charstring |  |  | IPv4 Address connected to PDN4 |
| px_IPv6_Address4_UE | charstring |  |  | IPv6 Address connected to PDN4 |
| px_IPv4_Address5_NW | charstring |  |  | IPv4 Gateway Address in PDN5 |
| px_IPv6_Address5_NW | charstring |  |  | IPv6 Gateway Address in PDN5 |
| px_IPv4_Address5_UE | charstring |  |  | IPv4 Address connected to PDN5 |
| px_IPv6_Address5_UE | charstring |  |  | IPv6 Address connected to PDN5 |
| px_IPv4_Address6_NW | charstring |  |  | IPv4 Gateway Address in PDN6 |
| px_IPv6_Address6_NW | charstring |  |  | IPv6 Gateway Address in PDN6 |
| px_IPv4_Address6_UE | charstring |  |  | IPv4 Address connected to PDN6 |
| px_IPv6_Address6_UE | charstring |  |  | IPv6 Address connected to PDN6 |
| px_NEDC_BandCombination | NEDC_BandCombination_Type | DC_n1A_28A |  | NE-DC Band Combination |
| px_NR_CA_BandCombination | NR_CA_BandCombination_Type | CA_n3A_n77A |  | NR CA Band Combination |
| px_NR_DC_BandCombination | NR_DC_BandCombination_Type | DC_n78A_n257A |  | NR-DC Band Combination |
| px_NR_DC_CA_BandCombination | NR_DC_CA_BandCombination_Type | DC_n78A_n257G |  | NR-DC CA Band Combination |
| px_NR_CipheringAlgorithm | CipheringAlgorithm | nea2 |  | Ciphering Algorithm (see Note 1) |
| px_NR_IntegrityProtAlgorithm | IntegrityProtAlgorithm | nia2 |  | Integrity Algorithm (see Note 1) |
| px_NR_OverlappingNotSupportedBand_MFBI | integer | 1 |  | A not supported NR band that is overlapping with a supported band (px_NR_PrimaryBand). Applied to MFBI test case scenario. |
| px_NR_PrimaryBand | integer | 1 |  | NR primary band |
| px_NR_SecondaryBand | integer | 2 |  | NR secondary band. Applied to inter-band and SUL test cases. |
| px_NR_SidelinkBand | integer | 47 |  | NR sidelink band |
| NOTE 1: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. NOTE 2: Void. NOTE 3: Void. NOTE 4: IP address used for specific IP-address-related test requirements. The only requirements applying to this IP address is that its value shall be different from the values of all other IP address PIXITs (see TS 36.523-3 [12] clause 9) and it shall not be used within more than one PDN (PDU session) within a test case. In the present version of this specification, it is only used within PDN1 NW. |  |  |  |  |

<End of modified section>

<Start of next modified section>

# 11	Guidelines on test execution

## 11.1	Introduction

Clause 11 provides the guidelines on test executions.

The restriction on test case execution is due to the number of frequencies available for the specific band under test specified in TS 38.508-1[5] and the number of frequencies used by the test cases specified in TS 38.523-1[8].

## 11.2	EN-DC

### 11.2.1	Single NR carrier

This clause provides the guidelines for the EN-DC test cases.

A test case using more than one radio frequency on NR, i.e. using the radio frequencies NRf2 or NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band combination DC_1A_n51A

Band combination DC_20A_n51A

Band combination DC_28A_n51A

Band combination DC_3A_n51A

Band combination DC_41A_n41A

Band combination DC_42A_n51A

Band combination DC_7A_n51A

The list of test cases is given below:

8.2.3.6.1a, 8.2.3.7.1a, 8.2.3.8.1a, 8.2.3.10.1, 8.2.3.11.1, 8.2.3.11.2

## 11.3	NR/5GC

### 11.3.1	NR/5GC single RAT

#### 11.3.1.1	Single NR carrier

This clause provides the guidelines for the NR/5GC test cases.

A test case using more than one radio frequency, i.e. using the radio frequencies NRf2 or NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n51.

The list of test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.11, 6.1.2.13, 6.1.2.14, 6.1.2.16, 6.1.2.18, 6.1.2.20, 6.1.2.21, 6.1.2.22, 6.3.1.1, 6.3.1.2,  6.3.1.3,  6.3.1.4, 6.3.1.5, 6.3.1.8,  6.3.1.9, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.5.1.2, 6.5.2.4,

7.1.3.4.4

8.1.1.2.1, 8.1.1.3.1, 8.1.1.3.3, 8.1.1.3.7, 8.1.1.3.7b, 8.1.3.1.3, 8.1.3.1.6, 8.1.3.1.9, 8.1.3.1.11, 8.1.3.1.12, 8.1.3.1.14A, 8.1.3.1.15A, 8.1.3.1.20, 8.1.3.21, 8.1.4.1.2, 8.1.4.3.4, 8.1.4.3.5, 8.1.6.1.2.2, 8.1.6.1.2.10, 8.1.6.1.3.2, 8.1.6.1.3.3, 8.1.6.1.3.7, 8.1.6.1.4.3, 8.1.6.1.4.7,

9.1.4.1, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.4, 9.1.5.1.8, 9.1.5.1.10, 9.1.5.1.12, 9.1.5.1.14,

11.3.5, 11.3.6, 11.3.8, 11.3.9,

11.4.7, 11.4.8.

A test case using more than two radio frequencies, i.e. using the radio frequencies NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n14,

Band n24,

Band n30,

Band n53.

The list of test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.11, 6.1.2.20, 6.3.1.1, 6.3.1.2, 6.3.1.3, 6.3.1.5, 6.3.1.8, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.5.2.4,

8.1.1.3.3, 8.1.6.3.3,

9.1.5.1.2, 9.1.5.1.8.

#### 11.3.1.2	NR carrier aggregation

The restriction on NR CA test case execution as listed in this clause is due to the restriction of bandwidth of an NR CA configuration accommodating the necessary number of NR radio frequencies.

These test cases with switched allocation of PCell and SCell on SCell shall avoid to be executed on an NR CA configuration with a secondary band having no UL frequency:

CA_n28A_n75A,

CA_n29A_n66A,

CA_n29A_n70A,

CA_n75A_n78A,

CA_n76A_n78A,

CA_n8A_n75A

The list of test cases is given below:

8.1.4.1.7.2

<End of modified section>