3GPP TSG-RAN5 Meeting #94-e	R5-220505r2

# Electronic Meeting, 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2745 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove test case 8.1.6.1.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In step 2 the UE attempts to send RRCSetupRequest and starts timer T300. The SS does not respond to the random access attempt and T300 expires. As per TS 38.331 sub-clause 5.3.3.7: " The UE shall: 1> if timer T300 expires: ... 2> store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows: 3> set the plmn-Identity to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1; ..." In test purpose 1 and in step 7 the UE is expected to include rlf-Report-r16 but the UE has only VarConnEstFailReport available as per the core spec above. The only way to achieve RLF report with random access failure is handover failure that is already covered in TC 8.1.6.1.3.4. Also, connection establishment failure with RACH failure report is already covered by test case 8.1.6.1.4.8 and T300 expiry is covered by 8.1.6.1.4.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case is redundant and needs to be Voided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will fail conformant UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-220505->R5-220505r1: -Added the correct reson for change and summary for change to the coversheet. R5-220505r1->R5-220505r2: - Updated the reason for change - Added “Clauses affected” |  |  |  |  |  |  |  |  |

###### 8.1.6.1.3.6	Void