3GPP TSG-RAN5 Meeting #94-e	R5-220604r1

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2777 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 8.1.4.3.4-RRC DAPS HO Success Inter-frequency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, The testcase title and TPs are not aligned with TC 8.1.4.3.1, which is the intra-freq version of the same procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Correct the TC title and TPs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38523-2... CR 0204... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Update TPs to align with TC 8.1.4.3.1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.4.3.4	DAPS handover with key change / Success / Inter-frequency

8.1.4.3.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supporting DAPS handover }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for handover with key change and dapsConfig is configured for a DRB }
    then { UE performs DAPS handover to the target cell and keeps DL/UL reception/transmission with the source gNB for the DRB configured with dapsConfig }

}

(2)

with { UE in NR RRC_CONNECTED state and supporting DAPS handover and having received an RRCReconfiguration message including a reconfigurationWithSync for handover to the neighbour cell and a dapsConfig configured for any DRB }

ensure that {

when {  UE has performed random access procedure to the target cell successfully }
    then { UE transmits an RRCReconfigurationComplete message to the target gNB }

}

(3)

with { UE in NR RRC_CONNECTED state and supporting DAPS handover and having received an RRCReconfiguration message including a reconfigurationWithSync for handover to the neighbour cell and a dapsConfig configured for any DRB }

ensure that {

when { UE receives an RRCReconfiguration message including a daps-SourceRelease }
    then { UE releases the radio link with the source cell }

}

(4)

with { UE in NR RRC_CONNECTED state and supporting DAPS handover and having received an RRCReconfiguration message including a reconfigurationWithSync for handover to the neighbour cell and a dapsConfig configured for any DRB }

ensure that {

when { UE has performed random access procedure to the target cell successfully }
    then { UE continues the downlink user data reception from the source gNB until releasing the source cell }

}

8.1.4.3.4.2	Conformance requirements

Same as test case 8.1.4.3.1.

8.1.4.3.4.3	Test description

8.1.4.3.4.3.1	Pre-test conditions

Same as test case 8.1.4.3.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

8.1.4.3.4.3.2	Test procedure sequence

Same as test case 8.1.4.3.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.4.3.4.3.3	Specific message contents

Same as test case 8.1.4.3.1 with the following difference:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

<End of modified section 1>