z3GPP TSG-RAN5 Meeting #94-e	R5-220781r1

# Electronic Meeting, 21 Feb – 4 Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1541 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to statistical testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Existing table of early pass/fail limit is not aligned with the parameter set { df = 0.004, clp = 0.9975}, therefore corrections are needed to keep the consistency of the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating early pass/fail limit using parameter set 2 {df=0.006, clp=0.9945} according to discussion paper R5-220780. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains self-conflicted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.2, H.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Adding note explaing why the nsf is set to NA in the first few entries. R5-183902 is taken as reference. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

Annex H (normative):
Statistical Testing

H.1	General

This annex specifies mapping throughput to error ratio, pass fail limits and pass fail decision rules that are needed for measuring average throughput for a duration sufficient to achieve statistical significance for testing receiver characteristics.

H.2	Statistical testing of receiver characteristics

H.2.1	General

The test of receiver characteristics is twofold.

1.	A signal or a combination of signals is offered to the RX port(s) of the receiver.

2.	The ability of the receiver to demodulate /decode this signal is verified by measuring the throughput.

In (2) is the statistical aspect of the test and is treated here.

The minimum requirement for all receiver tests is >95% of the maximum throughput.

All receiver tests are performed in static propagation conditions. No fading conditions are applied.

H.2.2	Mapping throughput to error ratio

a)	The measured information bit throughput R is defined as the sum (in kilobits) of the information bit payloads successfully received during the test interval, divided by the duration of the test interval (in seconds).

b)	In measurement practice the UE indicates successfully received information bit payload by signalling an ACK to the SS.
If payload is received, but damaged and cannot be decoded, the UE signals a NACK.

c)	Only the ACK and NACK signals, not the data bits received, are accessible to the SS.
The number of bits is known in the SS from knowledge of what payload was sent.

d)	For the reference measurement channel, applied for testing, the number of bits is different in different slots, however in a radio frame it is fixed during one test.

e)	The time in the measurement interval is composed of successfully received slots (ACK), unsuccessfully received slots (NACK) and no reception at all (DTX-slots).

f)	DTX-slots may occur regularly according the applicable reference measurement channel (regDTX).
In real live networks this is the time when other UEs are served. In TDD these are the UL and special slots.
regDTX vary from test to test but are fixed within the test.

g)	Additional DTX-slots occur statistically when the UE is not responding ACK or NACK where it should. (statDTX)
This may happen when the UE was not expecting data or decided that the data were not intended for it.

The pass / fail decision is done by observing the:

-	number of NACKs

-	number of ACKs and

-	number of statDTXs (regDTX is implicitly known to the SS)

The ratio (NACK + statDTX)/(NACK+ statDTX + ACK)is the Error Ratio (ER). Taking into account the time consumed by the ACK, NACK, and DTX-TTIs (regular and statistical), ER can be mapped unambiguously to throughput for any single reference measurement channel test.

H.2.3	Design of the test

The test is defined by the following design principles (see clause H.2.6, Theory….):

1.	The early decision concept is applied.

2.	A second limit is introduced: Bad DUT factor M>1

3.	To decide the test pass:

Supplier risk is applied based on the Bad DUT quality

To decide the test fail

Customer Risk is applied based on the specified DUT quality

The test is defined by the following parameters:

1.	Limit ER = 0.05 (Throughput limit = 95%)

2.	Bad DUT factor M=1.5 (selectivity)

3.	Confidence level CL = 95% (for specified DUT and Bad DUT-quality)

H.2.4	Numerical definition of the pass fail limits

Table H.2.4-1: pass fail limits

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 67 | NA | 49 | 905 | 599 | 98 | 1653 | 1396 | 147 | 2379 | 2236 |
| 1 | 67 | NA | 50 | 920 | 614 | 99 | 1668 | 1413 | 148 | 2393 | 2253 |
| 2 | 95 | NA | 51 | 936 | 630 | 100 | 1683 | 1430 | 149 | 2408 | 2270 |
| 3 | 119 | NA | 52 | 952 | 645 | 101 | 1698 | 1446 | 150 | 2423 | 2288 |
| 4 | 141 | NA | 53 | 967 | 661 | 102 | 1713 | 1463 | 151 | 2437 | 2305 |
| 5 | 162 | NA | 54 | 983 | 677 | 103 | 1728 | 1480 | 152 | 2452 | 2323 |
| 6 | 183 | NA | 55 | 998 | 692 | 104 | 1742 | 1497 | 153 | 2467 | 2340 |
| 7 | 203 | NA | 56 | 1014 | 708 | 105 | 1757 | 1514 | 154 | 2481 | 2358 |
| 8 | 222 | NA | 57 | 1029 | 724 | 106 | 1772 | 1531 | 155 | 2496 | 2375 |
| 9 | 241 | NA | 58 | 1045 | 740 | 107 | 1787 | 1548 | 156 | 2511 | 2393 |
| 10 | 259 | 59 | 59 | 1060 | 756 | 108 | 1802 | 1565 | 157 | 2525 | 2410 |
| 11 | 278 | 69 | 60 | 1076 | 772 | 109 | 1817 | 1582 | 158 | 2540 | 2428 |
| 12 | 296 | 80 | 61 | 1091 | 788 | 110 | 1832 | 1599 | 159 | 2554 | 2445 |
| 13 | 314 | 92 | 62 | 1107 | 804 | 111 | 1847 | 1616 | 160 | 2569 | 2463 |
| 14 | 332 | 103 | 63 | 1122 | 820 | 112 | 1862 | 1633 | 161 | 2584 | 2480 |
| 15 | 349 | 115 | 64 | 1137 | 836 | 113 | 1877 | 1650 | 162 | 2598 | 2498 |
| 16 | 367 | 127 | 65 | 1153 | 852 | 114 | 1891 | 1667 | 163 | 2613 | 2515 |
| 17 | 384 | 139 | 66 | 1168 | 868 | 115 | 1906 | 1684 | 164 | 2628 | 2533 |
| 18 | 401 | 152 | 67 | 1184 | 884 | 116 | 1921 | 1701 | 165 | 2642 | 2550 |
| 19 | 419 | 165 | 68 | 1199 | 900 | 117 | 1936 | 1718 | 166 | 2657 | 2568 |
| 20 | 436 | 177 | 69 | 1214 | 916 | 118 | 1951 | 1735 | 167 | 2671 | 2585 |
| 21 | 453 | 191 | 70 | 1229 | 933 | 119 | 1966 | 1752 | 168 | 2686 | 2603 |
| 22 | 469 | 204 | 71 | 1245 | 949 | 120 | 1981 | 1769 | 169 | 2701 | 2620 |
| 23 | 486 | 217 | 72 | 1260 | 965 | 121 | 1995 | 1787 | 170 | 2715 | 2638 |
| 24 | 503 | 231 | 73 | 1275 | 981 | 122 | 2010 | 1804 | 171 | 2730 | 2656 |
| 25 | 520 | 244 | 74 | 1290 | 998 | 123 | 2025 | 1821 | 172 | 2744 | 2673 |
| 26 | 536 | 258 | 75 | 1306 | 1014 | 124 | 2040 | 1838 | 173 | 2759 | 2691 |
| 27 | 553 | 272 | 76 | 1321 | 1030 | 125 | 2055 | 1855 | 174 | 2773 | 2708 |
| 28 | 569 | 286 | 77 | 1336 | 1047 | 126 | 2069 | 1872 | 175 | 2788 | 2726 |
| 29 | 585 | 300 | 78 | 1351 | 1063 | 127 | 2084 | 1890 | 176 | 2803 | 2744 |
| 30 | 602 | 314 | 79 | 1366 | 1080 | 128 | 2099 | 1907 | 177 | 2817 | 2761 |
| 31 | 618 | 329 | 80 | 1382 | 1096 | 129 | 2114 | 1924 | 178 | 2832 | 2779 |
| 32 | 634 | 343 | 81 | 1397 | 1113 | 130 | 2128 | 1941 | 179 | 2846 | 2796 |
| 33 | 651 | 358 | 82 | 1412 | 1129 | 131 | 2143 | 1958 | 180 | 2861 | 2814 |
| 34 | 667 | 372 | 83 | 1427 | 1146 | 132 | 2158 | 1976 | 181 | 2875 | 2832 |
| 35 | 683 | 387 | 84 | 1442 | 1162 | 133 | 2173 | 1993 | 182 | 2890 | 2849 |
| 36 | 699 | 402 | 85 | 1457 | 1179 | 134 | 2187 | 2010 | 183 | 2904 | 2867 |
| 37 | 715 | 416 | 86 | 1472 | 1195 | 135 | 2202 | 2028 | 184 | 2919 | 2885 |
| 38 | 731 | 431 | 87 | 1487 | 1212 | 136 | 2217 | 2045 | 185 | 2933 | 2902 |
| 39 | 747 | 446 | 88 | 1503 | 1229 | 137 | 2232 | 2062 | 186 | 2948 | 2920 |
| 40 | 763 | 461 | 89 | 1518 | 1245 | 138 | 2246 | 2079 | 187 | 2963 | 2938 |
| 41 | 779 | 476 | 90 | 1533 | 1262 | 139 | 2261 | 2097 | 188 | 2977 | 2955 |
| 42 | 795 | 491 | 91 | 1548 | 1279 | 140 | 2276 | 2114 | 189 | 2992 | 2973 |
| 43 | 810 | 507 | 92 | 1563 | 1295 | 141 | 2291 | 2131 | 190 | 3006 | 2991 |
| 44 | 826 | 522 | 93 | 1578 | 1312 | 142 | 2305 | 2149 | 191 | 3021 | 3009 |
| 45 | 842 | 537 | 94 | 1593 | 1329 | 143 | 2320 | 2166 | 192 | 3035 | 3026 |
| 46 | 858 | 552 | 95 | 1608 | 1346 | 144 | 2335 | 2183 | 193 | 3050 | 3044 |
| 47 | 873 | 568 | 96 | 1623 | 1362 | 145 | 2349 | 2201 | 194 | 3064 | 3062 |
| 48 | 889 | 583 | 97 | 1638 | 1379 | 146 | 2364 | 2218 | *) note 2 in H.2.5 |  |  |

NOTE 1:	The first column is the number of errors (ne = number of NACK + statDTX)

NOTE 2:	The second column is the number of samples for the pass limit (nsp, ns=Number of Samples= number of NACK + statDTX + ACK)

NOTE 3:	The third column is the number of samples for the fail limit (nsf)

NOTE 4:	The UE could be decided as early pass/fail when at least 67 samples are receieved. The nsf is set to NA for ne less than 10.

H.2.5	Pass fail decision rules

The pass fail decision rules apply for a single measurement. A test case is passed only when all the measurements in the test case are passed.

Having observed   0  errors, pass the test at      67+    samples,                                                 otherwise continue

Having observed   1  error,   pass the test at     95+  otherwise continue

Having observed   2  errors,  pass the test at     119+  samples,                                                 otherwise continue

Etc. etc.

Having observed 193 errors,  pass the test at  3050+  samples, fail the test at 3044- samples,  otherwise continue

Having observed 194 errors,  pass the test at  3064+  samples, fail the test at 3062- samples.

Where x+ means: x or more,        x- means x or less

NOTE 1:	an ideal DUT passes after 67 samples. The maximum test time is 3062 samples.

NOTE 2:	It is allowed to deviate from the early decision concept  by  postponing  the decision (pass/fail or continue). Postponing the decision to or beyond the end of Table H.2.4-1 requires a pass fail decision against the test limit: pass the DUT for ER<0.0634, otherwise fail.

H.2.6	Theory to derive the pass fail limits (Informative)

Editor's note:	This clause of the Annex G is for information only and it describes the background theory and information for statistical testing.

H.2.6.1	Numerical definition of the pass-fail limits

A statistical test is characterized by test time, selectivity and confidence level. The outcome of the statistical test is a decision. This decision may be correct, i.e., DUTs whose throughput is less than 95% maximum throughput being declared to fail, and DUTs whose throughput is higher or equal to 95% being declared to pass, or in-correct with oppsite decision. The Confidence Level (CL) describes the probability that the decision is a correct one. The complement is the wrong decision probability (risk) D = 1-CL.

As described in H.2.2, the measurement of throughput could be mapped to ER (Error Ratio). When testing ER, transport blocks or "samples" are observed and the number of correctly and erroneously received blocks are recorded. For a "standard" test, a pre-defined number of samples are observed, and a pass/fail decision is made based on the number of observed errors being above/below a threshold. This threshold is based on the targeted throughput or BLER and the design target CL. There is always some risk of a statistical variation leading to an incorrect pass/fail decision. The greater the number of samples that are recorded, the lower is the risk of such an error. The number of samples that are observed in a standard test is dimensioned to achieve an acceptable low risk of error (i.e., an acceptable high confidence level) for DUTs that just meet the specified limit.

The standard test works well where the target ER level is relatively high and confidence level relatively low (both are chosen to be on a comparable order of magnitude). However, for relatively low ER testing the length of time required for observing sufficient samples to achieve a 95% confidence level is excessive. In many cases, the DUTs will in fact have a much lower true ER level than the target ER level, (in which case, the number of samples needed to achieve high confidence that the true ER level is lower than the limit is much smaller). On the other hand, a bad DUT which is expected to fail the requirement might have a much higher true ER level (in which case, errors occur more frequently and it can be demonstrated that the DUT is above the target ER limit with fewer samples).

To avoid long test time, an alternative test method called early pass/fail is adopted. With the early pass/fail, each time a block error is encountered, a decision is made on whether the DUT can be passed/failed with 95% CL or the test needs to continue until another error is encountered. In the case of very good DUTs, the test can also be passed, when the number of samples permissible for one error event is reached and no error event is recorded. Pass/Fail is decided based on the total number of observed samples and errors, and a statistical calculation based on an inverse binomial cumulative distribution. The calculation involves one parameter, one variable and the result:

-	Parameter: d (per step decision probability).

-	Variable: ne (number of observed errors).

-	Result: ns (number of expected samples for pass/fail, depending on which one is calculated).

The per step decision probability risk, d, expresses the probability of making an incorrect pass/fail decision in the current step (i.e., for the current decision coordinate). d is determined by simulation such that the overall risk of making a wrong decision over all steps of each test of a large number of tests on a large number of DUTs that exactly meet the target ER limit is D=5% (and hence the CL 95%).

It should be noted that d is determined separately considering early pass and early fail testing.

For a marginal DUT (i.e., a DUT almost exactly meeting the target ER level), the unmodified early pass/early fail approach is unable to distinguish whether the DUT has just passed or just failed the BLER (ε→0), and can thus terminate with an "undecided" result. To avoid this undecided result and provide selectivity, a so-called "bad device factor" (M) is introduced into the early pass calculation. This factor biases the decision towards avoiding failing good DUT.

H.2.6.2	Simulation to derive the pass-fail limits for testing 95% throughput

As per the description in H.2.2, the 95% throughput measurement is mapped to ER=0.05, where ER is (NACK + statDTX)/(NACK+ statDTX + ACK).

The binomial distribution and its inverse are used to design the pass and fail limits. Note that this method is not unique and that other methods exist.

Where

-	fail(..) is the error ratio for the fail limit.

-	pass(..) is the error ratio for the pass limit.

-	ER is the specified error ratio 5%.

-	ne is the number of bad results. This is the variable in both equations.

-	M is the Bad DUT factor M=1.5.

-	df is the wrong decision probability of a single (ne, ns) co-ordinate for the fail limit.
It is found by simulation to be df = [0.0006].

-	clp is the confidence level of a single (ne, ns) co-ordinate for the pass limit.
It is found by simulation to be clp = [0.9945].

-	qnbinom(..): The inverse cumulative function of the negative binomial distribution.

The simulation works as follows:

-	A large population of limit DUTs with true ER = 0.05 is decided against the pass and fail limits.

-	clp and df are tuned such that CL (95 %) of the population passes and D (5 %) of the population fails.

-	A population of Bad DUTs with true ER = M*0.05 is decided against the same pass and fail limits.

-	clp and df are tuned such that CL (95 %) of the population fails and D (5 %) of the population passes.

-	The number of DUTs decrease during the simulation, as the decided DUTs leave the population. That number decreases with an approximately exponential characteristics. After 195 bad results all DUTs of the population are decided.

NOTE:	The exponential decrease of the population is an optimal design goal for the decision co-ordinates (ne, ns), which can be achieved with other formulas or methods as well.

H.2A	Statistical testing of receiver characteristics with CA

H.2A.1	General

H.2.1 applies.

H.2A.2	Mapping throughput to error ratio

H.2.2 applies for each component carrier.

H.2A.3	Design of the test

The test is defined by the following design principles (see clause H.x, Theory….):

1.	The standard concept is applied. (not the early decision concept).

2.	A second limit is introduced, defining the Bad DUT.

3.	To decide the test pass:

-	Supplier risk is applied based on the Bad DUT quality.

-	To decide the test fail.

-	Customer Risk is applied based on the specified DUT quality.

The test is defined by the following parameters:

1)	Limit Error Ratio = 0.05 (95% throughput is tested).

2)	Bad DUT factor M=1.5 (selectivity).

3)	Confidence level CL = 95% (for specified DUT and Bad DUT-quality).

H.2A.4	Pass fail decision rules

Apply 978 samples to the DUT per CC.

Decide pass per CC in case of ≤ 62 errors, otherwise fail.

NOTE 1:	The pass fail decision is done individually for each CC. The pass fail decision for one measurement is as follows: pass if all CCs or SCC only according to the test cases pass, otherwise fail. A test case is passed only when all the measurements in the test case are passed.

NOTE 2:	It is allowed to apply more samples to the DUT, common for all CCs, (e.g. up to an integer number of frames). Use the ratio (62/978) for the pass fail decision.

NOTE 3:	62/978 = 0.0634, the same test limit is used at the end of Table H.2.4-1

<End of Changes >