Title: 	Discussion on test point selection for MPR, NR ACLR and SEM in FR2

Source: 	Ericsson, Keysight Technologies, Rohde & Schwarz, CAICT, Sporton

Introduction

A default baseline for test point selection for TRx test cases was presented in [1] at RAN5#1 5G NR Adhoc meeting, where some proposals for environmental conditions, test frequencies, channel bandwidths, subcarrier spacing and RB allocation for FR1 were based on LTE. Finally, it was stated that each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Maximum Power Reduction (MPR), Spectrum Emission mask (SEM) and Adjacent Channel Leakage Ratio (ACLR) tests in FR2. Firstly, single transmission case is done, then ACLR UL-MIMO case is provided with exception test points analysis based on the outcome from comparing the minimum requirements and waveforms allowed.

This paper tries to follow endorsed proposals in R5-197586 [6] in RAN5#84:

The baseline for this combined MPR, ACLR and SEM analysis is, as per 38.905 v16.5.0 [7]:

38.521-2_TPanalysis_6.2.2_MPR_v2.zip

38.521-2_TPanalysis_6.5.2.1_SEM.zip

38.521-2_TPanalysis_6.5.2.3_ACLR.zip

The above 3 papers were replaced by this combined analysis:

38.521-2_TPanalysis_6.2A.2_MPR_6.5.2.1_SEM_6.5.2.3_NR_ACLR.zip

And subsequent revisions.

Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

Test Environment

Test environment for SEM measurement for LTE and FR1 in [1] is Normal Conditions. This test environment can be also leveraged for FR2.

Test environments MPR and ACLR for LTE and FR1 in [1] are Normal and Extreme Conditions. This could be leveraged for FR2. Similar as FR1, SEM could be only tested in NC.

Summary:

Proposal 1: Define Test Environment for SEM, ACLR and MPR in FR2 as per the table below

Table 2.1-1 Test environment configuration summary

| Test Case | Test Environment |
| --- | --- |
| MPR | NC, TH, TL |
| ACLR | NC, TH, TL |
| SEM | NC |

Test Subcarrier Spacing

The goal of this section is to expose technical reasons to select the SCS to be tested in NR ACLR, MPR and SEM in FR2. This study is focused to detect the most critical use cases, based on SCS selection, when the nominal operation mode of the UE is compromised in this test.

MPR aspects:

In NR FR1, Test Subcarrier Spacing for MPR test case is Lowest and Highest supported subcarrier spacing. This can be leveraged for FR2.

ACLR and SEM aspects:

The following aspects have been considered:

Spectrum utilization and Minimum GuardBand

The maximum transmission bandwidth configuration NRB for each channel bandwidth and subcarrier spacing is specified in Table 2.2-1 (obtained from Table 5.3.2-1 in [2]).

Table 2.2-1 Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

The maximum transmission bandwidth (in MHz) per each channel bandwidth and SCS is calculated using the values of the maximum number of Resource block and subcarrier spacing information (assuming 12 subcarriers per resource block), these values are shown in Table 2.2-2.

Table 2.2-2 Maximum transmission bandwidth configuration in MHz

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 47.52 | 95.04 | 190.08 | N/A |
| 120 | 46.08 | 95.04 | 190.08 | 380.16 |

Observation 1: Maximum transmission bandwidth for each channel bandwidth is not always achieved with lowest SCS and it keeps similar for different SCS.

Besides Table 2.2-3 (taken from Table 5.3.3-1 in [2]) depicts the minimum guardband for a UE channel bandwidth in each subcarrier spacing. This guardband is the frequency separation between the channel edge and the transmission bandwidth edge.

Table 2.2-3 Minimum Guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 |

Observation 2: Lower guardbands are achieved with lower SCS for 50 MHz channel bandwidth. For 100 MHz and 200 MHz channel bandwidth, similar guardband values are achieved.

In [1], maximum transmission bandwidth and minimum guardband were important to select the SCS to test in FR1. However, for FR2, these parameters are not decisive for selecting the SCS for all channel bandwidths, as several SCS have similar values for the same channel bandwidth.

Secondary lobes from subcarriers

Continuing the purpose to find the most critical situations for the UE related to the SCS selection other observation might be done.

In Table 2.2-3 the frequency range for the guardband is shown for each SCS and channel bandwith. However, if frequency response of each subcarrier is assumed as non-ideal, then this guardband might be studied in terms of how many secondary lobs would keep inside this guardband. This information gives us an estimation on how many power might go out the channel bandwidth due to the non-linearity of the transmitter.

Table 2.2-4 Number of secondary lobs are included in Guardband for each UE channel bandwidth and SCS (kHz) from last subcarrier

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 20.1 | 40.3 | 82.1 | N/A |
| 120 | 15.8 | 20.1 | 40.8 | 82.1 |

Table 2.2-4 depicts the number of secondary lobs inside the guardband. The most of cases show that the highest SCS will have lower number of secondary lobs inside the guardband so, if they are not filtered by channel bandwidth filter, this power would be an unwanted spurious emission out of the channel.

Observation 3: Based on the number of secondary lobs in guardband, it seems to be reasonable to test the Highest SCS supported by UE.

Phase Noise Impact

In FR2, whose bands operates in high frequencies, one of main problems is related to Phase Noise affecting to local oscillators. OFDM subcarrier generation is a non-ideal process involving local oscillators to generate the subcarriers. Local oscillators are affected by phase noise, which increases with carrier frequency. The effect of the phase noise in the frequency domain is a non-ideal, wider pulse for each subcarrier. This phase noise provokes that subcarrier is wider than really it is, even increasing the transmission bandwidth.

To achieve efficient communications in high frequency by protect it from this phase noise is why SCS highest are defined since it is known that SCS highest are more robust against phase noise. So, lowest SCS might be more affected by this problem and each subcarrier can be wider and the transmission bandwidth might be lightly wider.

Observation 4: Based on comments about phase noise, the worst situation is test in conditions at high frequency and lower SCS, which seems not to be so robust with this issue.

Measurement Method:

In SEM test case, power is measured using TRP what takes significant larger time than EIRP measurements used in MPR and ACLR test cases. For that reason, in order to optimize testing time, is proposed to test SEM only with Highest SCS.

Summary:

Proposal 2: Select SCS for MPR, ACLR and SEM in FR2 as per the table below

Table 2.2-5 Test subcarrier spacing summary

| Test Case | Test Subcarrier Spacing |
| --- | --- |
| MPR | Lowest, Highest SCS |
| ACLR | Lowest, Highest SCS |
| SEM | Highest SCS |

  - Test Frequencies

MPR Aspects

In NR FR1, Test Frequencies for MPR test case are Low range and High range test frequencies. This can be leveraged for FR2.

ACLR Aspects

In LTE, Adjacent Channel Leakage Ratio measurement is tested for Low, Mid and High frequencies. This criteria is reasonable to verify different behaviours of the UE transmitter for different frequencies due to non-linearity based on the frequency.

With the purpose to reduce testing time the following analysis is based on detecting the most critical situations, related with the non-linearity with frequency.

The UE filter design, which operates in a specific band, usually is based on a reference frequency to design the active components in its transmitter. In general, the most critical situations to operate are when the transmitter filter in UE is centered in frequencies far from the reference frequency.

The most critical cases based on the reference frequency for UE filters and the frequency to be tested are:

- UE implementation based on the Mid Frequency  The most far frequency would be Low and High, so this option would be covered in case to test Low or High frequency.

- UE implementation based on any extreme frequency (Low or High)  If testing is indicated for both Low and High, then the worst case would be tested as well. For instance: if the reference frequency is Low then the testing on High would be the most far frequency distant.

Observation 5: Low and High frequencies can cover the most critical situations for ACLR with the frequency variation obtaining a timing reduction in test as well. This criteria is even applied for this kind of bands which are really wide.

SEM Aspects

Spectrum Emission Mask is included inside the section Out of band emissions where it is pretended to evaluate the unwanted emissions immediately outside the nominal channel resulting from the modulation process and non-linearity in the transmitter.

In [8] it was discussed that Low/High frequencies need to be tested mainly because of the front-end architecture of the UEs, utilizing band specific RF filters.

The following quote was stated in [8]:

“The UE filter design, which operates in a specific band, usually is based on a reference frequency to design the active components in its transmitter. In general, the most critical situations to operate are when the transmitter filter in UE is centered in frequencies far from the reference frequency.

The most critical cases based on the reference frequency for UE filters and the frequency to be tested are:”

This is certainly true for FR1 where band specific filters are used in the RF front-ends of the UEs. For FR2 the discussion came up whether those devices actually use specific front-end filters for each band or at all.

RAN4 has discussed some reference architectures for devices operating in FR2 in the context of defining assumptions to later derive UE requirements. In Figure 1 a reference architecture from [10] can be seen, which was approved by RAN4 in [11]. As can be seen from the drawing, the FR1 bands utilize RF filters. However for the FR2 bands some filters are only implemented in front of the receiver LNA and not in the transmit chain of the device.

Figure 1 NSA reference architecture according to [4]

Taking this information into account it can be concluded that it is not necessary to test the Low/High frequencies because of the filter implementation of the UE. It would be sufficient to use a single test point of the band under test so only Mid frequency range will be tested. In order to ensure that SEM test points are a subset of the MPR test points, that will imply that Mid frequency range will also need to be tested in MPR and for consistency should be added to ACLR. In order to optimize testing time, please refer to section 2.5 about which RB allocations will be tested in Mid frequency.

Summary:

Proposal 3: Select Test frequencies for MPR, ACLR and SEM in FR2 as per the table below

Table 2.3-1 Test frequencies summary

| Test Case | Test Frequencies |
| --- | --- |
| MPR | Low, Mid, High |
| ACLR | Low, Mid, High |
| SEM | Mid |

Test Channel Bandwidth

MPR Aspects

In NR FR2, MPRWT for Channel Bandwidth 50/100/200 MHz are same, while MPRWT for Channel Bandwidth 400 MHz are different with Channel Bandwidth 50/100/200 MHz. MPR test should cover all the different requirements. In NR FR1, Test Channel Bandwidth for MPR test case is Lowest and Highest test channel bandwidth for different MPR requirements. This can be leveraged for MPR test in FR2.

Observation 6: Test the Lowest and Highest supported channel bandwidth seems to cover the most critical situations for MPR requirement in FR2.

SEM Aspects

In LTE, Spectrum Emission Mask is tested for Lowest, 5 MHz, 10 MHz and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE transmitter related to the non-linearity with different channel bandwidth.

Based on it, NR Spectrum Emission Mask for FR2 might be tested for 50, 100, 200 and 400 MHz, corresponding to Lowest, Highest and two middle channel bandwidths.

Observation 7: Test the Lowest, 100 MHz, 200 MHz and Highest would cover all situations defined in LTE.

The lowest channel bandwidth makes UE to conform its filter with narrow band-pass and with a very high slope, which it is difficult to obtain without increasing losses in the bandpass of the filter and keeping this band-pass flat. This is why the narrowest filters operating in high frequency are design usually with a not really abrupt ramp-up/ramp-down to avoid adding extra losses. Based on this, it would make reasonable to test the Lowest channel bandwidths.

However, the channel bandwidth with Higher band pass are especially sensible to obtain ramp-down with high slope with filters whose size is small, so it is interesting to test this option as well.

Although the most critical scenarios are covered by Lowest and Highest channel bandwidths, the LTE approach can be reasonable by adding one additional Mid bandwidth to cover more non-linearity cases with channel bandwidths. Based on it, NR Spectrum Emission Mask might be tested for Lower, Mid and Highest.

Observation 8: Test the Lowest. Mid and Highest seems to cover the most critical situations for SEM.

ACLR Aspects:

In LTE, Adjacent Channel Leakage Ratio is tested for Lowest, 5 MHz, 10 MHz and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE transmitter related to the non-linearity with different channel bandwidth.

Based on it, NR Adjacent Channel Leakage Ratio for FR2 might be tested for 50, 100, 200 and 400 MHz, corresponding to Lowest, Highest and two middle channel bandwidths.

Observation 9: Test the Lowest, 100 MHz, 200 MHz and Highest would cover all situations defined in LTE.

The lowest channel bandwidth makes UE to conform its filter with narrow band-pass and with a very high slope, which it is difficult to obtain without increasing losses in the bandpass of the filter and keeping this band-pass flat. This is why the narrowest filters operating in high frequency are design usually with a not really abrupt ramp-up/ramp-down to avoid adding extra losses. Based on this, it would make reasonable to test the Lowest channel bandwidths.

However, the channel bandwidth with Higher band pass are especially sensible to obtain ramp-down with high slope with filters whose size is small, so it is interesting to test this option as well.

Observation 10: Test the Lowest and Highest seems to cover the most critical situations.

Based on the observations above and in order to ensure test points for SEM and ACLR tests are a sub-set of the MPR test points, it is proposed then to test only lowest and Highest channel bandwidth only also for SEM and ACLR tests.

Summary:

Proposal 4: Select Test channel bandwidth for MPR, ACLR and SEM in FR2 as per the table below

Table 2.4.1-1 Test channel bandwidth summary

| Test Case | Test Channel Bandwidth |
| --- | --- |
| MPR | Lowest, Highest |
| ACLR | Lowest, Highest |
| SEM | Lowest, Highest |

UL Modulation and RB Allocation

As it has been introduced this test cases needs to evaluate the UE behaviour in MPR, ACLR and SEM considering modulation and RB allocation.

MPR Aspects

The selection of test points for Test Channel Bandwidth, Uplink Modulation and RB allocation should be based on the Minimum conformance requirements for MPR in [2], which are extracted as bellow:

#### 6.2.2	UE maximum output power reduction

#### 6.2.2.0	General

The requirements in section 6.2.2 only apply when both UL and DL of a UE are configured for single CC operation, and they are of the same bandwidth. A UE may reduce its maximum output power due to modulation orders, transmit bandwidth configurations, waveform types and narrow allocations. This Maximum Power Reduction (MPR) is defined in subsections below. When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2.4 apply.

For a UE that is configured for single CC operation with different channel bandwidths in UL and DL, the requirements in section 6.2A.2 apply.

For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 KHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

#### 6.2.2.1	UE maximum output power reduction for power class 1

For power class 1, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB is less than or equal to 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in table 5.3.2-1, and waveform types. MPRWT is defined in Tables 6.2.2.1-1 and 6.2.2.1-2.

Table 6.2.2.1-1 MPRWT for power class 1, BWchannel ≤ 200 M

| Modulation |  | MPRWT (dB), BWchannel ≤ 200 M |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) | RBstart < Ceil(1/3 NRB) OR RBend > Ceil(2/3 NRB) |
| DFT-s-OFDM | PI/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 5.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |

Table 6.2.2.1-2 MPRWT for power class 1, BWchannel = 400 M

| Modulation |  | MPRWT (dB), BWchannel = 400 M |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | [RBstart ≥ Ceil(1/4 NRB) AND RBend ≤ Ceil(3/4 NRB) AND LCRB≤Ceil(1/4 NRB)] | [RBstart < Ceil(1/4 NRB) OR RBend > Ceil(3/4 NRB) OR LCRB>Ceil(1/4 NRB)] |
| DFT-s-OFDM | PI/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.0 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 | ≤ 9.0 |

Where RBend = RBStart + LCRB, and the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High,

and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

#### 6.2.2.2	UE maximum output power reduction for power class 2

For power class 2, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

-	MPRnarrow = 2.5 dB, when the allocated RB size is less than or equal to 1.44MHz, and 0≤RBstart≤Ceil(1/3 NRB) or Ceil(2/3NRB)≤RBstart≤NRB-LCRB

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.2-1.

Table 6.2.2.2-1 MPRWT for power class 2

|  |  | Channel Bandwidth / MPRWT |  |
| --- | --- | --- | --- |
|  |  | 50 / 100 / 200 MHz | 400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 3.0 |
|  | QPSK | 1.5 | 3.0 |
|  | 16QAM | 3 | 4.5 |
|  | 64QAM | 5 | 6.5 |
| CP-OFDM | QPSK | 3.5 | 5.0 |
|  | 16QAM | 5 | 6.5 |
|  | 64QAM | 7.5 | 9.0 |

#### 6.2.2.3	UE maximum output power reduction for power class 3

For power class 3, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 2.5 dB, when the allocated RB size is less than or equal to 1.44 MHz, and 0 ≤ RBstart ≤ Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤NRB-LCRB

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.3-1 and Table 6.2.2.3-2.

Table 6.2.2.3-1 MPRWT for power class 3, BWchannel ≤ 200 MHz

|  |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) | RBstart < Ceil(1/3 NRB) OR RBend > Ceil(2/3 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 2.0 |
|  | QPSK | 0.0 | ≤ 2.0 |
|  | 16QAM | ≤ 3.0 | ≤ 3.5 |
|  | 64QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.0 |
|  | 16QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64QAM | ≤ 7.5 | ≤ 7.5 |

Table 6.2.2.3-2 MPRWT for power class 3, BWchannel = 400 MHz

|  |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/4 NRB) AND RBend ≤ Ceil(3/4 NRB) AND LCRB≤Ceil(1/4 NRB) | RBstart < Ceil(1/4 NRB) OR RBend > Ceil(3/4 NRB) OR LCRB>Ceil(1/4 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 3.0 |
|  | QPSK | 0.0 | ≤ 3.0 |
|  | 16QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64QAM | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64QAM | ≤ 9.0 | ≤ 9.0 |

Where RBend = RBStart + LCRB

#### 6.2.2.4	UE maximum output power reduction for power class 4

For power class 4, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 2.5 dB, when the allocated RB size is less than or equal to 1.44 MHz, and 0 ≤ RBstart ≤ Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤NRB-LCRB

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.4-1.

Table 6.2.2.4-1 MPRWT for power class 4

|  |  | Channel Bandwidth / MPRWT |  |
| --- | --- | --- | --- |
|  |  | 50 / 100 / 200 MHz | 400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 3.0 |
|  | QPSK | 1.5 | 3.0 |
|  | 16QAM | 3 | 4.5 |
|  | 64QAM | 5 | 6.5 |
| CP-OFDM | QPSK | 3.5 | 5.0 |
|  | 16QAM | 5 | 6.5 |
|  | 64QAM | 7.5 | 9.0 |

According to the above MPR requirement and the consideration of Crest Factors, when the waveform is same, the highest modulation order is the most critical case, and when modulation order is same, CP-OFDM is more critical than DFT-s-OFDM for MPR in FR2. Since FR2 testing is more time consuming it is proposed to only select the most critical waveform and modulation order for the same MPR requirement.

Observation 11: To cover the most critical situations, select the most critical waveform and modulation order for the test of the same MPR requirement, which is, if the waveform is same, select the max modulation order, if the modulation order is same, select CP-OFDM waveform. For DFT-s-OFDM with high modulation order and CP-OFDM with low modulation order, since it is hard to say which is more critical, both should be tested.

In LTE and NR FR1, MPR is tested for the largest RB allocation and the Outer 1RB allocation for each MPR minimum conformance requirement. Basically FR2 MPR testing should follow that and test the largest RB allocation and Outer 1RB allocation(or the narrowest Outer RB allocation) for each MPR minimum conformance requirement. The narrow RB allocation at left edge for Lowest Frequency and narrow RB allocation at right edge for Highest Frequency are considered as the most critical case.

Observation 12: To cover the most critical situations, select the largest RB allocation and the Outer 1RB allocation(or the narrowest Outer RB allocation) for each MPR minimum conformance requirement. Select the narrow Uplink RB allocation at left edge for Lowest Frequency and narrow RB allocation at right edge for Highest Frequency for different requirement for MPR in FR2.

Based on the two observations above, in order to save testing time, it is proposed to test Outer Full RB allocations and Inner full RB allocations only at mid frequency range, given low and high frequency ranges will be well covered by edge partial allocations.

SEM and ACLR Aspects:

Based on out of band emission requirement dependency with modulation process, it seems to be reasonable to test all UL modulations.

Table 2.5-1 Uplink Modulations

UL Modulations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

DFT-s-OFDM 256 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

CP-OFDM 256 QAM

If section 4.5.3 in [4] is analysed, the spectrum utilisation and the transmission bandwidth are different depending on the type of OFDM modulation (CP-OFDM or DFT-s-OFDM) as shown in Tables below:

CP-OFDM (Tables obtained from section 4.5.3.1 in [4])

Table 4.5.3.1-1: Range 1 NR UE and BS maximum RB allocation for CP-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

Table 4.5.3.1-2: Range 1 NR UE and BS transmission bandwidths in MHz for CP-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 47.52 | 95.04 | 190.08 | N/A |
| 120 | 46.08 | 95.04 | 190.08 | 380.16 |

Table 4.5.3.1-3: Range 1 NR UE and BS spectrum utilization for CP-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 95.0% | 95.0% | 95.0% | N/A |
| 120 | 92.2% | 95.0% | 95.0% | 95.0% |

Table 4.5.2.3-4: Range 1 NR UE and BS minimum guard band sizes (kHz) for CP-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 1210 | 2450 | 4930 | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 |

DFT-s-OFDM (Tables obtained from section 4.5.3.2 in [8])

Table 4.5.3.2-1: Range 1 NR UE maximum RB allocation for DFT-s-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 64 | 128 | 264 | N/A |
| 120 | 32 | 64 | 128 | 264 |

Table 4.5.3.2-2: Range 1 NR UE transmission bandwidths in MHz for DFT-s-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 46.08 | 92.16 | 190.08 | N/A |
| 120 | 46.08 | 92.16 | 184.32 | 380.16 |

Table 4.5.3.2-3: Range 1 NR UE spectrum utilization for DFT-s-OFDM

| SCS [kHz] | BS / UE Channel bandwidths [MHz] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 92.2% | 92.2% | 95.0% | N/A |
| 120 | 92.2% | 92.2% | 92.2% | 95.0% |

If each table in CP-OFDM modulation is compared with its corresponding table in DFT-s-OFDM modulation:

- For a specific SCS and ChBw the number of resource blocks for DFT-s modulations is equal or lower than the used by CP modulations.

- UE spectrum utilization can be smaller for DFT-s-OFDM than for CP-OFDM.

Observation 13: Due to the different behaviour between DFT-s-OFDM and CP-OFDM modulations the testing for both types of modulations would be required (DFT-s and CP modulations).

Observation 14: According to the knowledge about the OFDM modulations behaviour, it is known that the highest OFDM modulation orders tend to present higher non-linearity, and higher PAPR (Peak to Average Power Ratio), than lower modulations orders. Although the modulations with less non-linearity are based on pi/2-BPSK and QPSK modulations, it would be necessary to test them to discard any problem with this modulations in UE.

Considering the analysis of CCDFs for all the NR signals done in [9], the following reduction about modulation can be used:

- Consider limiting of testing to only QPSK CP-OFDM from among CP-OFDM modulations,

- Consider limiting of testing to only one of 64QAM or 256QAM DFT-s-OFDM.

Observation 15: To cover the most critical situations, test the following UL modulations supported by UE: DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM, DFT-s-OFDM 64 QAM, CP-OFDM QPSK,) for Spectrum Emission Mask measurement in FR2.

Summary:

Proposal 5: Select the most critical waveform and modulation order for the test of the same MPR requirement, which is, if the waveform is same, select the max modulation order, if the modulation order is same, select CP-OFDM waveform. For DFT-s-OFDM with high modulation order and CP-OFDM with low modulation order, since it is hard to say which is more critical, both should be tested for MPR in FR2.

Proposal 6: For ACLR and MPR, select the largest RB allocation and the Outer 1RB allocation(or the narrowest Outer RB allocation) for each MPR minimum conformance requirement. Select the narrow Uplink RB allocation at left edge for Lowest Frequency, narrow RB allocation at right edge for Highest Frequency and largest RB allocation for Mid frequency range for different requirements for MPR in FR2.

Proposal 7: For SEM, given only Mid frequency range will be tested and given its larger testing time due to TRP measurement method, select only the largest RB allocation.

Proposal 8: Select UL modulation and RB allocation for ACLR and SEM in FR2 as per the table below

Table 2.5.1-1 UL modulation and RB allocation summary

| Test Case | UL Modulation | RB Allocation |
| --- | --- | --- |
| ACLR (Power Class 1) | DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM, DFT-s-OFDM 64 QAM CP-OFDM QPSK | Outer 8RB, Outer full Outer 8RB, Outer full Outer 8RB, Outer full Outer 8RB, Outer full Outer 8RB, Outer full |
| ACLR (Power Class 2, 3 and 4) | DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM, DFT-s-OFDM 64 QAM CP-OFDM QPSK | Outer 1RB, Outer full Outer 1RB, Outer full Outer 1RB, Outer full Outer 1RB, Outer full Outer 1RB, Outer full |
| SEM | DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM, DFT-s-OFDM 64 QAM CP-OFDM QPSK | Outer full Outer full Outer full Outer full Outer full |

Number of Test Points

| Test Case | Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2.2 MPR | 1 | 1(Note 1) | 2 | 2 | TBD | TBD |
| 6.5.2.1 SEM | 1 | 1(Note 1) | 2 | 1 | 5 | 10 |
| 6.5.2.3 ACLR | 1 | 1(Note 1) | 2 | 2 | 15 | 60 |
| Note 1: Outer xRB left will be tested only with Low frequency range. Outer xRB right will be tested only with High frequency range. Other allocations will be tested only with Mid frequency range. |  |  |  |  |  |  |

Exceptions for UL MIMO test

Extracted from Clause 6.5D.2 [6]

6.5D.2	Out of band emission for UL-MIMO

For UE supporting UL-MIMO, the out of band emissions requirements in clause 6.5.2 apply. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.0-1.

6.2D.2.3	UE maximum output power reduction for modulation / channel bandwidth for UL MIMO for power class 3

For UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.3-1 is specified in sub-clause 6.2.2.3. The requirements shall be met with configurations specified in sub-clause 6.2D.1.0.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

Observation 16: Same out of band emission requirements as specified for single layer transmission apply for UE supporting UL-MIMO.

According to [6], CP-OFDM is mandatory to UL-MIMO transmission with two layer data, therefore DFT-s-OFDM is precluded from the test points for UL-MIMO measurement.

Proposal 9: Precluding DFT-s-OFDM, test CP-OFDM QPSK, CP-OFDM 16 QAM, CP-OFDM 64 QAM for NR MPR, ACLR and SEM UL-MIMO measurement in FR2, while the other test parameters are the same as single transmission case.

For Rel-16 UEs supporting uplink full power transmission (ULFPTx), the requirements specified in TS 38.101-2 clause 6.2D.2 and 6.5D.2 shall be met with the PUSCH configurations specified in Table 6.2D.1.0, based upon UE’s support of uplink full power transmission mode. The PUSCH configuration in Table 6.2D.1.0-2 for ULFPTx includes both DFT-s-OFDM and CP-OFDM.

Proposal 10: Test points for Rel-16 ULFPTx UE can be the same as single transmission case without exception.

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Adjacent Channel Leakage Ratio test case definition:

Proposal 1: Define Test Environment for SEM, ACLR and MPR in FR2 as per the table below

| Test Case | Test Environment |
| --- | --- |
| MPR | NC, TH, TL |
| ACLR | NC,TH, TL |
| SEM | NC |

Proposal 2: Select SCS for MPR, ACLR and SEM in FR2 as per the table below

| Test Case | Test Subcarrier Spacing |
| --- | --- |
| MPR | Lowest, Highest SCS |
| ACLR | Lowest, Highest SCS |
| SEM | Highest SCS |

Proposal 3: Select Test frequencies for MPR, ACLR and SEM in FR2 as per the table below

| Test Case | Test Frequencies |
| --- | --- |
| MPR | Low, Mid, High |
| ACLR | Low, Mid, High |
| SEM | Mid |

Proposal 4: Select Test channel bandwidth for MPR, ACLR and SEM in FR2 as per the table below

| Test Case | Test Channel Bandwidth |
| --- | --- |
| MPR | Lowest, Highest |
| ACLR | Lowest, Highest |
| SEM | Lowest, Highest |

Proposal 5: Select the most critical waveform and modulation order for the test of the same MPR requirement, which is, if the waveform is same, select the max modulation order, if the modulation order is same, select CP-OFDM waveform. For DFT-s-OFDM with high modulation order and CP-OFDM with low modulation order, since it is hard to say which is more critical, both should be tested for MPR in FR2.

Proposal 6: For ACLR and MPR, select the largest RB allocation and the Outer 1RB allocation(or the narrowest Outer RB allocation) for each MPR minimum conformance requirement. Select the narrow Uplink RB allocation at left edge for Lowest Frequency, narrow RB allocation at right edge for Highest Frequency and largest RB allocation for Mid frequency range for different requirement for MPR in FR2.

Proposal 7: For SEM, given only Mid frequency range will be tested and given its larger testing time due to TRP measurement method, select only the largest RB allocation.

Proposal 8: Select UL modulation and RB allocation for ACLR and SEM in FR2 as per the table below

| Test Case | UL Modulation | RB Allocation |
| --- | --- | --- |
| ACLR/SEM (Power Class 1) | DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM, DFT-s-OFDM 64 QAM CP-OFDM QPSK | Outer 8RB, Outer full Outer 8RB, Outer full Outer 8RB, Outer full Outer 8RB, Outer full Outer 8RB, Outer full |
| ACLR/SEM (Power Class 2, 3 and 4) | DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM, DFT-s-OFDM 64 QAM CP-OFDM QPSK | Outer 1RB, Outer full Outer 1RB, Outer full Outer 1RB, Outer full Outer 1RB, Outer full Outer 1RB, Outer full |
| SEM | DFT-s-OFDM PI/2 BPSK, DFT-s-OFDM QPSK, DFT-s-OFDM 16 QAM, DFT-s-OFDM 64 QAM CP-OFDM QPSK | Outer full Outer full Outer full Outer full Outer full |

Proposal 9: Precluding DFT-s-OFDM, test CP-OFDM QPSK, CP-OFDM 16 QAM, CP-OFDM 64 QAM for NR MPR, ACLR and SEM UL-MIMO measurement in FR2, while the other test parameters are the same as single transmission case.

Proposal 10: Test points for Rel-16 ULFPTx UE can be the same as single transmission case without exception.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-2 v15.5.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] 3GPP TS 38.521-2 v16.5.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[4] 3GPP TR 38.817-01 v0.3.0 General aspects for UE RF for NR (Release 15)

[5] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019

[6] R5-197586, “Discussion on the relation between MPR and SEM/ACLR test cases”, Ericsson. RAN5#84, Ljubljana Aug. 2020

[7] 3GPP TR 38.905 v16.3.0, Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases

[8] R5-182028, “Discussion on test point selection for NR SEM in FR2”, Keysight Technologies, RAN5#2-5G-NR Adhoc, Taoyuan, Taiwan, April 2018.

[9] R5-183012, “Discussion on test frequencies for SEM in FR2”, Rohde & Schwarz, RAN5#79, Busan, South Korea, May 2018.

[10] R4-1708161: Reference UE RF architectures in both range 1 and range 2; LG Electronics; RAN4#84; Berlin August 2018

[11] R4-1710710: TP on UE RF architectures for NSA DC UE; LG Electronics; RAN4#84bis; Dubrovnik October 2018