3GPP TSG-RAN WG5 Meeting # 94-e	R5-220853r3

# Electronic meeting, February 21 - March 4, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1388 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of 4.1.1 on lower humidity limit in temperature test environment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN5#90-e meeting, a CR R5-211127 on test environment conditions revised the normal temperature test environment with the lower humidity limit to 25%. However, this issue has been discussed in previous GERAN (GP-080557) and RAN5 meetings many years ago and reached the agreements that the lower limit for humidity in normal conditions could be removed. It is noted that there is no performance dependency on the lower limit for the humidity level, so for testing purposes it doesn’t make any sense to keep it. To avoid having to declare a lower limit value in UE test, CR R5-106417 removed the lower limit from 34.121-1 in WCDMA requirements and CR R5-110534 did the same changes in LTE requirements. To be consistent with the previous agreements, there is no need in R5-211127 to set the lower limit for humidity as 25%. Therefore, it is suggested to remove the lower humidity limit of 25% and keep the upper limit unchanged. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Editor’s note below section 4.1.1 to indicate lower humidity limit is under discussion for normal test condition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary declarations for lower humidity limit for LTE test will be needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 – Correcting the order of companies listed in Source to WG matching to 3GU. Correction to WIC by removing LTE-UEConTest_RF and updated TEI16_Test to TEI8_Test. Removed reference to “RAN5” before the CR#. Added option 2 as suggested by Ericsson during RF session meeting. R2 : (1) Remove the change marks in the coversheet. (2) Remove the sentence “Option 2 is to add a note below Table 4.1.1-1” in the field of “Reason for change” in coversheet. (3) Change the field of “Summary of change” in coversheet to “Add a note below Table 4.1.1-1 to indicate no lower humidity limit is used for normal test condition.” (4) Undo the changes in Table 4.1.1-1 and add a note to indicate no lower humidity limit is used. R3 : (1) Add an Editor’s note in section 4.1.1 to indicate lower humidity limit is under discussion. (2) Change the field of “Summary of change” in coversheet to remove the reference to adding Note 1 and update to adding Editor’s note in section 4.1.1 (3) Undo the changes in Table 4.1.1-1 for Note 1. |  |  |  |  |  |  |  |  |

### << start of changes  >>

### << Unchanged sections omitted >>

# 4	Common test environment

## 4.1	Environmental conditions

The requirements in this clause apply to all types of UE(s).

### 4.1.1	Temperature

Editor's note: Lower humidity limit is under discussion as it is not aligned with previous technologies.

The UE shall fulfil all the requirements in the full temperature range of:

Table 4.1.1-1: Temperature Test Environment

| +15C to +35C | For normal conditions (with relative humidity of 25% to 75 %) |
| --- | --- |
| -10C to +55C | For extreme conditions (see IEC publications 68-2-1 and 68-2-2) |

Outside this temperature range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 36.101 [27] clause 6.2 for extreme operation.

The normative reference for this requirement is TS 36.101 [27] Annex E.2	.

Some tests are performed also in extreme temperature conditions. These test conditions are denoted as TL (temperature low, -10C) and TH (temperature high, +55C).

### 4.1.2	Voltage

The UE shall fulfil all the requirements in the full voltage range, i.e. the voltage range between the extreme voltages.

The manufacturer shall declare the lower and higher extreme voltages and the approximate shutdown voltage. For the equipment that can be operated from one or more of the power sources listed below, the lower extreme voltage shall not be higher, and the higher extreme voltage shall not be lower than that specified below.

Table 4.1.2-1: Voltage Test Environment

| Power source | Lower extreme voltage | Higher extreme voltage | Normal conditions voltage |
| --- | --- | --- | --- |
| AC mains | 0.9 * nominal | 1.1 * nominal | nominal |
| Regulated lead acid battery | 0.9 * nominal | 1.3 * nominal | 1.1 * nominal |
| Non regulated batteries: Leclanché Lithium Mercury/nickel & cadmium | 0.85 * nominal 0.95 * nominal 0.90 * nominal | nominal 1.1 * nominal | nominal 1.1 * nominal nominal |

Outside this voltage range the UE if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 36.101 [27] for extreme operation. In particular, the UE shall inhibit all RF transmissions when the power supply voltage is below the manufacturer declared shutdown voltage.

The normative reference for this requirement is TS 36.101 [27] Annex E.2.

Some tests are performed also in extreme voltage conditions. These test conditions are denoted as VL (lower extreme voltage) and VH (higher extreme voltage).

==============================================================

### << End of changes >>