3GPP TSG-RAN5 Meeting #94-e	R5-220888r3

# Online,  , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0697 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to FR2 absolute power tolerance MU and TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The values of p0-NominalWithGrant must be even numbers, but the current values in 6.3.4.2 are odd numbers. The current MU & TT values are incorrect, because DL MU is not considered in the MU calculation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the values of p0-NominalWithGrant from odd numbers to even numbers in Table 6.3.4.2.4.3-1, 6.3.4.2.4.3-2, 6.3.4.2.4.3-3, and correct the expected measured power in Table 6.3.4.2.5-1, 6.3.4.2.5-2, 6.3.4.2.5-3. - Remove Correct Note 2 in Table 6.3.4.2.5-1 and 6.3.4.2.5-2 according to the Proposal 3 of the discussion paper. - Correct MU & TT values according to the Proposal 1 and 2 of the discussion paper. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4.2, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | To be updated as per the outcome of the MU discussion of R5-220887. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: According to the outcome of the MU discussion of R5-220887r1: - Note 2 in Table 6.3.4.2.5-1 and 6.3.4.2.5-2 are corrected - TT values are correct in Table 6.3.4.2.5-5 and F.3.2-1 r2: - For test point 1 , FR2b, 50MHz, p0-NominalWithGrant value is midified from -114 to -112 in Table 6.3.4.2.4.3-1, and expected measured power is modified from 5.0dBm to 7.0dBm in Table 6.3.4.2.5-1. - Correction of Note 2 in Table 6.3.4.2.5-1 and 6.3.4.2.5-2 are removed, because the correction is caused by double-counting DL MU. r3: - Added editor’s note “The reduction of the impact of DL MU by choosing alpha < 1 is FFS.” - The expected measured power for test point 1 is modified to be close to minimum measureable power in Table 6.3.4.2.5-1, and corresponding p0-NominalWithGrant value is corrected in Table 6.3.4.2.4.3-1 - Notes in Table 6.3.4.2.5-1 are corrected to not test the lower limit. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.3.4.2	Absolute power tolerance

Editor’s Note: The following aspects are either missing or not yet determined:

- Testing of extreme conditions for FR2 is FFS.

- UE transmitted power for PC 1, 2 and 4 are FFS

- The reduction of the impact of DL MU by choosing alpha < 1 is FFS.

6.3.4.2.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

To verify the ability of the UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e. transmission gap is larger than 20 ms.

6.3.4.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.3.4.2.3	Minimum conformance requirements

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error RSRP estimate.

The minimum requirements specified in Table 6.3.4.2.3-1 apply in the power range bounded by the minimum output power as specified in sub-clause 6.3.1 (Pmin) and the maximum output power as specified in sub-clause 6.2.1.1 as minimum peak EIRP (‘Pmax’). The intermediate power point 'Pint' is defined in table 6.3.4.2.3-2.

Table 6.3.4.2.3-1: Absolute power tolerance

| Power Range | Tolerance |
| --- | --- |
| Pint ≥ P ≥ Pmin | ± 14.0 dB |
| Pmax ≥ P > Pint | ± 12.0 dB |

Table 6.3.4.2.3-2: Intermediate power point

| Power Parameter | Value |
| --- | --- |
| Pint | Pmax – 12.0 dB |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.3.4.2.

6.3.4.2.4	Test description

6.3.4.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.3.4.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.4.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | 50 MHz, 100 MHz, 200 MHz, 400 MHz (NOTE 2) |  |
| Test SCS as specified in Table 5.3.5-1. |  | Highest |  |
| Test Parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | N/A for absolute power tolerance test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Test is required only for CBWs supported by the UE. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement Channel is set according to Table 6.3.4.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3.4.2.4.3.

6.3.4.2.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 0_1 with TPC command 0dB for C_RNTI to schedule the UL RMC according to Table 6.3.4.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Configure the UE transmitted output power to test point 1 in section 6.3.4.2.4.3. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP of the first subframe in the Tx beam peak direction in the measurement bandwidth specified in Table 6.3.1.5-1 and Table 6.3.1.5-2 for the specific channel bandwidth under test. EIRP test procedure is defined in Annex K. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi. For TDD slots with transient periods are not under test.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

7.	Repeat test steps 1~6 for measurement of test point 2~3. The timing of the execution between the two test points shall be larger than 20ms.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.3.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config and with following exceptions:

Table 6.3.4.2.4.3-1: PUSCH-ConfigCommon (Test point 1) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -122 |  | FR2a, 50MHz |
|  | -126 |  | FR2a, 100MHz |
|  | -128 |  | FR2a, 200MHz |
|  | -130 |  | FR2a, 400MHz |
|  | -118 |  | FR2b, 50MHz |
|  | -120 |  | FR2b, 100MHz |
|  | -124 |  | FR2b, 200MHz |
|  | -126 |  | FR2b, 400MHz |
| } |  |  |  |

Table 6.3.4.2.4.3-2: PUSCH-ConfigCommon (Test point 2) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -108 |  | FR2a, 50MHz |
|  | -110 |  | FR2a, 100MHz |
|  | -114 |  | FR2a, 200MHz |
|  | -116 |  | FR2a, 400MHz |
|  | -110 |  | FR2b, 50MHz |
|  | -112 |  | FR2b, 100MHz |
|  | -116 |  | FR2b, 200MHz |
|  | -118 |  | FR2b, 400MHz |
| } |  |  |  |

Table 6.3.4.2.4.3-3: PUSCH-PowerControl (Test point 3) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -98 |  | FR2a, 50MHz |
|  | -102 |  | FR2a, 100MHz |
|  | -104 |  | FR2a, 200MHz |
|  | -106 |  | FR2a, 400MHz |
|  | -100 |  | FR2b, 50MHz |
|  | -104 |  | FR2b, 100MHz |
|  | -106 |  | FR2b, 200MHz |
|  | -108 |  | FR2b, 400MHz |
| } |  |  |  |

Table 6.3.4.2.4.3-4: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 4 |  | SCS_120kHz |
|  | 7 |  | SCS_240kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_120kHz | SCS=120kHz for SS/PBCH block |
| SCS_240kHz | SCS=240kHz for SS/PBCH block |

Table 6.3.4.2.4.3-5: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| tpc-Accumulation | disabled |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.3.4.2.5	Test requirement

The measured EIRP in step 5 and 7 shall not to exceed the values specified in Table 6.3.4.2.5-1 to 6.3.4.2.5-3.

Table 6.3.4.2.5-1: Absolute power tolerance: test point 1 for power class 3

|  | Frequency range | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | FR2a | -3.0 | -4.0 | -2.9 | -2.8 |
| power | FR2b | 1.0 | 2.0 | 1.1 | 1.2 |
| Power tolerance (Note 2) |  | ± (14+TT)dB |  |  |  |
| Note 1: The higher power limit shall not exceed the Max EIRP defined in sub-clause 6.2.1.1.5. Note 2: Do not test lower limit. |  |  |  |  |  |

Table 6.3.4.2.5-2: Absolute power tolerance: test point 2 for power class 3

|  | Frequency range | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | FR2a | 11.0 | 12.0 | 11.1 | 11.2 |
| power | FR2b | 9.0 | 10.0 | 9.1 | 9.2 |
| Power tolerance (Note 2) |  | ± (12+TT)dB |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.5, and the higher power limit shall not exceed the Max EIRP defined in sub-clause 6.2.1.1.5. Note 2: Do not test lower limit at CBW ≥ 200 MHz for FR2b |  |  |  |  |  |

Table 6.3.4.2.5-3: Absolute power tolerance: test point 3 for power class 3

|  | Frequency range | Channel bandwidth / expected output power (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Expected Measured | FR2a | 21.0 | 20.0 | 21.1 | 21.2 |
| power | FR2b | 19.0 | 18.0 | 19.1 | 19.2 |
| Power tolerance |  | ± (12+TT)dB |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.5, and the higher power limit shall not exceed the Max EIRP defined in sub-clause 6.2.1.1.5. |  |  |  |  |  |

Table 6.3.4.2.5-4: Test Tolerance for power class 1, 2, 4

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| IFF (Max device size ≤ 30 cm) | Same as Table 6.3.1.5-3 | Same as Table 6.3.1.5-3 |

Table 6.3.4.2.5-5: Test Tolerance for power class 3

| Test Metric | NTC testing | ETC testing |
| --- | --- | --- |
| IFF (Max device size ≤ 30 cm) | ±8.05 dB | ±8.42 dB |

<<Unchanged sections skipped>>

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| <<Unchanged rows omitted>> |  |  |
| 6.3.1 Minimum output power | PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm FFS (FR2a) FFS dB (FR2b) PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±6.15 dB (FR2a & FR2b, NTC testing ±6.41 dB (FR2a & FR2b, ETC testing)) |  |
| 6.3.2 Transmit OFF power | PC3: Max Device size ≤ 30 cm ±5.49 dB (FR2a) | MTSU = 1.00 x MU (from Table B.8-2-2 in TR 38.903) |
| 6.3.3.2 General ON/OFF time mask | PC3: ON power: TBD OFF power: Max Device size ≤ 30 cm ±6.15 dB (FR2a & FR2b, NTC testing) ±6.41 dB (FR2a & FR2b, ETC testing) |  |
| 6.3.3.4 PRACH time mask | PC3: PRACH power: TBD OFF power: Max Device size ≤ 30 cm ±6.15 dB (FR2a & FR2b, NTC testing) ±6.41 dB (FR2a & FR2b, ETC testing) |  |
| 6.3.3.6 SRS time mask | TBD |  |
| 6.3.4.2 Absolute power tolerance | PC3 Max Device size ≤ 30 cm ±8.05 dB (FR2a & FR2b, NTC testing) ±8.42 dB (FR2a & FR2b, ETC testing) | MTSU = SQRT (UL Meas Uncer2 + DL Meas Uncer2) UL Meas Uncer: Same as 6.3.1 DL Meas Uncer: Same as 7.3.2 |
| 6.3.4.3 Relative power tolerance | PC3 Max Device size ≤ 30 cm [±1.7 dB] (FR2a) [±1.7 dB] (FR2b) | MTSU = 1.00 x MU (from Table B.9a.2.2-2 in TR 38.903) |
| 6.3.4.4 Aggregate power tolerance | PC3 Max Device size ≤ 30 cm ±1.4 dB (FR2a) ±1.4 dB (FR2b) | MTSU = 1.00 x MU (from Table B.9a.3.2-2 in TR 38.903) |
| <<Unchanged rows omitted>> |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

<<Unchanged sections skipped>>

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| <<Unchanged rows omitted>> |  |  |
| 6.3.1 Minimum output power | PC3 Minimum EIRP IFF (Max Device size ≤ 30 cm) NTC 4.21 dB (FR2a 50 MHz) 2.52 dB (FR2a 100 MHz) 0.66 dB (FR2a 200 MHz) 0 dB (FR2a 400 MHz) 1.17 dB (FR2b 50 MHz) 0 dB (FR2b 100 MHz) 0 dB (FR2b 200 MHz) 0 dB (FR2b 400 MHz) ETC 4.37 dB (FR2a 50 MHz) 2.68 dB (FR2a 100 MHz) 0.82 dB (FR2a 200 MHz) 0 dB (FR2a 400 MHz) 1.33 dB (FR2b 50 MHz) 0 dB (FR2b 100 MHz) 0 dB (FR2b 200 MHz) 0 dB (FR2b 400 MHz) | Minimum EIRP TT = max(R, ΔSNRmr + 0.65 x (MTSUIFF – 1.0)) -R R: Relaxation needed to limit influence of TE noise to 1 dB (specified in clause 6.3.1.5) ΔSNRmr: Systematic offset due to noise when measuring at minimum requirement level (-13 dBm) FR2a 50 MHz: ΔSNRmr = 0.86 dB FR2a 100 MHz: ΔSNRmr = 1.57 dB FR2a 200 MHz: ΔSNRmr = 2.71 dB FR2a 400 MHz: ΔSNRmr = 4.35 dB FR2b 50 MHz: ΔSNRmr = 2.32 dB FR2b 100 MHz: ΔSNRmr = 3.82 dB FR2b 200 MHz: ΔSNRmr = 5.82 dB FR2b 400 MHz: ΔSNRmr = 8.21 dB |
| 6.3.2 Transmit OFF power | 0 dB |  |
| 6.3.3.2 General ON/OFF time mask | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b) | ON Power TBD |
| 6.3.3.4 PRACH time mask | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD | ON Power TBD |
| 6.3.4.2 Absolute power tolerance | PC3 Max Device size ≤ 30 cm ±8.05 dB (FR2a & FR2b, NTC testing) ±8.42 dB (FR2a & FR2b, ETC testing) | TT = MTSU |
| 6.3.4.3 Relative power tolerance | PC3 IFF (Max Device size ≤ 30 cm) [0.46 dB] (FR2a) [0.46 dB] (FR2b) | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |
| 6.3.4.4 Aggregate power tolerance | PC3 IFF (Max Device size ≤ 30 cm) 0.26 dB (FR2a) 0.26 dB (FR2b) | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |
| <<Unchanged rows omitted>> |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

<<End of change>>