3GPP TSG-RAN5 Meeting #94-e	R5-220894r1

# Online,  , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1294 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of MTSU and TT for Intra-band EN-DC additional spurious emissions test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, DOCOMO Communications Lab. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MTSU and TT for 6.5B.4.1 and 6.5B.4.2 are not defined in Annex F. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the editor’s note in 6.5B.4.2. - Define MTSU for 6.5B.4.1 and 6.5B.4.2 in Table F.1.2-1. - Define TT for 6.5B.4.1 and 6.5B.4.2 in Table F.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.4.2, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Undo the removal of editor’s note to solve the overlap with R5-221317 |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.5B.4.2	Additional Spurious Emissions for Intra-band non-contiguous EN-DC

Editor's note:	MU and TT is missing in annex F for this test case

6.5B.4.2.1	Test purpose

Same minimum conformance requirements as in clause 6.5B.4.1.1.

6.5B.4.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.4.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5B.4.1.3.

6.5B.4.2.4	Test description

6.5B.4.2.4.1	Initial conditions

Same initial conditions as described in clause 6.5B.4.1.4.1 for both E-UTRA and NR carriers with the following exception:

1.	For each EN-DC combination specified in Table 5.3B.1.3-1, channel spacing between NR and E-UTRA is specified according to clause 5.4B.1.

2.	Set up the NR and E-UTRA test frequencies so that NR carrier is located at the lower frequency side as specified in Table 5.3B.1.3-1. Repeat each testing with E-UTRA carrier frequency is located at the lower side as specified in Table 5.3B.1.3-1.

6.5B.4.2.4.2	Test Procedure

Same test procedure as described in clause 6.5B.4.1.4.2.

6.5B.4.2.4.3	Message Contents

Message contents are according to TS 38.508-1 [6] clause 4.6 with the following exceptions for each network signalled value.

6.5B.4.2.4.3.1 	Message contents exceptions for network signalled value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.5B.4.2.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [6] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.5B.4.2.5	Test Requirement

Test requirements for Spurious Emissions for intra-band non-contiguous EN-DC are the same as the minimum requirements described in 6.5B.4.2.3 and are not repeated in this clause.

<<Unchanged sections skipped>>

F.1	Acceptable uncertainty of Test System (normative)

TBD

F.1.1	Measurement of test environments

TBD

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |
| 6.5B.1.1 Occupied bandwidth for Intra-Band Contiguous EN-DC | 1.5% of aggregated channel bandwidth |  |
| 6.5B.1.2 Occupied bandwidth for Intra-Band Non-Contiguous EN-DC | Same as clause 6.5.1 in TS 38.521-1 [8] |  |
| 6.5B.1.3 Occupied bandwidth for Inter-Band EN-DC within FR1 | Same as clause 6.5.1 in TS 38.521-1 [8] |  |
| 6.5B.1.4 Occupied bandwidth for Inter-Band EN-DC including FR2 | Same as clause 6.5.1 in TS 38.521-2 [9] |  |
| 6.5B.1.4_1.1 Occupied bandwidth for Inter-band EN-DC including FR2 (3 CCs) | TBD |  |
| 6.5B.1.4_1.2 Occupied bandwidth for Inter-band EN-DC including FR2 (4 CCs) | TBD |  |
| 6.5B.1.4_1.3 Occupied bandwidth for Inter-band EN-DC including FR2 (5 CCs) | TBD |  |
| 6.5B.1.4D Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO | Same as clause 6.5D.1 in TS 38.521-2 [9] |  |
| 6.5B.2.1.1 Spectrum emissions mask for intra-band contiguous EN-DC | Same as clause 6.5.2.2 in TS 38.521-1 [8] |  |
| 6.5B.2.1.2 Additional spectrum emissions mask for intra-band contiguous EN-DC | Same as clause 6.5.2.3 in TS 38.521-1 [8] |  |
| 6.5B.2.1.3 Adjacent channel leakage ratio for intra-band contiguous EN-DC | Same as clause 6.5.2.4.1 in TS 38.521-1 [8] |  |
| 6.5B.2.2.1 Spectrum emissions mask for intra-band non-contiguous EN-DC | Same as clause 6.5.2.2 in TS 38.521-1 [8] |  |
| 6.5B.2.2.2 Additional Spectrum emissions mask for intra-band non-contiguous EN-DC | Same as clause 6.5.2.3 in TS 38.521-1 [8] |  |
| 6.5B.2.2.3 Adjacent channel leakage ratio for intra-band non-contiguous EN-DC | TBD |  |
| 6.5B.2.3.1 Spectrum emissions mask for Inter-band EN-DC within FR1 | Same as clause 6.5.2.2 in TS 38.521-1 [8] |  |
| 6.5B.2.3.2 Additional Spectrum emissions mask for Inter-band EN-DC within FR1 | Same as clause 6.5.2.3 in TS 38.521-1 [8] |  |
| 6.5B.2.3.3 Adjacent channel leakage ratio for inter-band EN-DC within FR1 | Same as clause 6.5.2.4.1 in TS 38.521-1 [8] |  |
| 6.5B.2.4.1 Spectrum emissions mask for Inter-band EN-DC including FR2 | Same as clause 6.5.2.1 in TS 38.521-2 [9] |  |
| 6.5B.2.4.1_1.1 Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs) | Same as clause 6.5A.2.1.1 in TS 38.521-2 [9] |  |
| 6.5B.2.4.1_1.2 Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs) | Same as clause 6.5A.2.1.2 in TS 38.521-2 [9] |  |
| 6.5B.2.4.1_1.3 Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs) | Same as clause 6.5A.2.3 in TS 38.521-2 [9] |  |
| 6.5B.2.4.1_1.1 Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs) | Same as clause 6.5A.2.1.1 in TS 38.521-2 [9] |  |
| 6.5B.2.4.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 | Same as clause 6.5.2.3 in TS 38.521-2 [9] |  |
| 6.5B.2.4.3_1.1 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 CCs) | Same as clause 6.5A.2.2.1 in TS 38.521-2 [9] |  |
| 6.5B.2.4.3_1.2 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 CCs) | Same as clause 6.5A.2.2.2 in TS 38.521-2 [9] |  |
| 6.5B.2.4.3_1.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 CCs) | Same as clause 6.5A.2.2.3 in TS 38.521-2 [9] |  |
| 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.2.2 Spurious Emission band UE co-existence for intra-band non-contiguous EN-DC | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.4.1 General Spurious Emissions for Inter-band including FR2 (2 CCs) | Same as clause 6.5.3.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2 Spurious emission band UE co-existence for Inter-band including FR2 | Same as clause 6.5.3.2 in TS 38.521-2 [9] |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 | Same as clause 6.5.4.3 in TS 38.521-1 [8] |  |
| 6.6B.4 Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Same as clause 6.6.1 in TS 38.521-2 [9] |  |

<<Unchanged sections skipped>>

F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

F.3.1	Measurement of test environments

TBD

F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |
| 6.5B.1.1 Occupied bandwidth for Intra-Band Contiguous EN-DC | Same as 6.5.1 in TS 38.521-1 [8] |  |
| 6.5B.1.2 Occupied bandwidth for Intra-Band Non-Contiguous EN-DC | Same as 6.5.1 in TS 38.521-1 [8] |  |
| 6.5B.1.3 Occupied bandwidth for Inter-Band EN-DC within FR1 | Same as 6.5.1 in TS 38.521-1 [8] |  |
| 6.5B.1.4 Occupied bandwidth for Inter-Band EN-DC including FR2 | Same as 6.5.1 in TS 38.521-2 [9] |  |
| 6.5B.1.4_1.1 Occupied bandwidth for Inter-band EN-DC including FR2 (3 CCs) | TBD |  |
| 6.5B.1.4_1.2 Occupied bandwidth for Inter-band EN-DC including FR2 (4 CCs) | TBD |  |
| 6.5B.1.4_1.3 Occupied bandwidth for Inter-band EN-DC including FR2 (5 CCs) | TBD |  |
| 6.5B.1.4D Occupied bandwidth for inter-band EN-DC including FR2 for UL MIMO | Same as 6.5D.1 in TS 38.521-2 [9] |  |
| 6.5B.2.1.1 Spectrum emissions mask for intra-band contiguous EN-DC | Same as 6.5.2.2 in TS 38.521-1 [8] |  |
| 6.5B.2.1.2 Additional spectrum emissions mask for intra-band contiguous EN-DC | Same as 6.5.2.3 in TS 38.521-1 [8] |  |
| 6.5B.2.1.3 Adjacent channel leakage ratio for intra-band contiguous EN-DC | Same as 6.5.2.4.1 in TS 38.521-1 [8] |  |
| 6.5B.2.2.1 Spectrum emissions mask for intra-band non-contiguous EN-DC | Same as 6.5.2.2 in TS 38.521-1 [8] |  |
| 6.5B.2.2.2 Additional Spectrum emissions mask for intra-band non-contiguous EN-DC | Same as 6.5.2.3 in TS 38.521-1 [8] |  |
| 6.5B.2.2.3 Adjacent channel leakage ratio for intra-band non-contiguous EN-DC | Same as 6.5.2.4.1 in TS 38.521-1 [8] |  |
| 6.5B.2.3.1 Spectrum emissions mask for Inter-band EN-DC within FR1 | Same as 6.5.2.2 in TS 38.521-1 [8] |  |
| 6.5B.2.3.2 Additional Spectrum emissions mask for Inter-band EN-DC within FR1 | Same as 6.5.2.3 in TS 38.521-1 [8] |  |
| 6.5B.2.3.3 Adjacent channel leakage ratio for inter-band EN-DC within FR1 | Same as 6.5.2.4.1 in TS 38.521-1 [8] |  |
| 6.5B.2.4.1 Spectrum emissions mask for Inter-band EN-DC including FR2 | Same as 6.5.2.1 in TS 38.521-2 [9] |  |
| 6.5B.2.4.1_1.1 Spectrum emissions mask for Inter-band EN-DC including FR2 (2 NR CCs) | Same as clause 6.5A.2.1.1 in TS 38.521-2 [9] |  |
| 6.5B.2.4.1_1.2 Spectrum emissions mask for Inter-band EN-DC including FR2 (3 NR CCs) | Same as clause 6.5A.2.1.2 in TS 38.521-2 [9] |  |
| 6.5B.2.4.1_1.3 Spectrum emissions mask for Inter-band EN-DC including FR2 (4 NR CCs) | Same as clause 6.5A.2.3 in TS 38.521-2 [9] |  |
| 6.5B.2.4.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 | Same as 6.5.2.3 in TS 38.521-2 [9] |  |
| 6.5B.2.4.3_1.1 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (3 CCs) | Same as clause 6.5A.2.2.1 in TS 38.521-2 [9] |  |
| 6.5B.2.4.3_1.2 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (4 CCs) | Same as clause 6.5A.2.2.2 in TS 38.521-2 [9] |  |
| 6.5B.2.4.3_1.3 Adjacent channel leakage ratio for Inter-band EN-DC including FR2 (5 CCs) | Same as clause 6.5A.2.2.3 in TS 38.521-2 [9] |  |
| 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC | Same as 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC | Same as 6.5.3.2 in TS 38.521-1 [8] |  |
| 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC | Same as 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.2.2 Spurious Emission band UE co-existence for intra-band non-contiguous EN-DC | Same as 6.5.3.2 in TS 38.521-1 [8] |  |
| 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 | Same as 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 | Same as 6.5.3.2 in TS 38.521-1 [8] |  |
| 6.5B.3.4.1 General Spurious Emissions for Inter-band including FR2 (2 CCs) | Same as 6.5.3.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2 Spurious emission band UE co-existence for Inter-band including FR2 | Same as 6.5.3.2 in TS 38.521-2 [9] |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 | Same as 6.5.4 in TS 38.521-1 [8] |  |

<<End of change>>