3GPP TSG-RAN5 Meeting #94-e	R5-220903r1

# Online,  , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2258 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test frequency range for n14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The DL range for n14 is only 10MHz. Therefore, the Low frequency range, Mid frequency range, and High frequency range will be the same test point for CBW=10MHz. When a test case requires test frequencies as Low range, Mid range, High range, the same test will be tested three times at the same frequency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that only Low frequency range is tested |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicate test points maybe created. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Corrected sentence |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

4.3.1.1.1.14	Reference test frequencies for NR operating band n14

Table 4.3.1.1.1.14-1: Test frequencies for NR operating band n14 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 760.5 | 152100 | 758.25 | 151650 | 0 | 15 | 1902 | 152190 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 763 | 152600 | 742.39 | 148478 | 102 |  | 1909 | 152690 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 765.5 | 153100 | 672.53 | 134506 | 504 |  | 1913 | 153130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 790.5 | 158100 | 788.25 | 157650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 793 | 158600 | 700.03 | 140006 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 795.5 | 159100 | 792.17 | 158434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 763 | 152600 | 758.32 | 151664 | 0 | 15 | 1903 | 152210 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 793 | 158600 | 788.32 | 157664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcchConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.14-2: Test frequencies for NR operating band n14 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 763 | 152600 | 758.68 | 151736 | 0 | 15 | 1909 | 152690 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 793 | 158600 | 788.68 | 157736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcchConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<End of change>>