3GPP TSG-RAN5 Meeting #94-e	R5-220907r1

# Online,  , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1565 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to SUL test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are editorial errors in 6.2.1, 6.4C.1, 6.5C.3.1, 6.5C.3.3, 7.3C.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the editorial errors in 6.2.1, 6.4C.1, 6.5C.3.1, 6.5C.3.3, 7.3C.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The editorial errors remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.4C.1, 6.5C.3.1, 6.5C.3.3, 7.3C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Undo the changes in 6.4C.1 to solve the overlap with R5-220249 |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.2.1	UE maximum output power

Editor’s Note: The following aspects are not yet determined:

- PC1 requirements for NR operating bands other than Band n14 are not defined in RAN4 Rel-15 and Rel-16 specifications.

6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1, 1.5, 2 and 3 UE release 15 and forward.

6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | ± 2 |
| n2 |  |  |  |  |  |  | 23 | ± 23 |
| n3 |  |  |  |  |  |  | 23 | ± 23 |
| n5 |  |  |  |  |  |  | 23 | ± 2 |
| n7 |  |  |  |  |  |  | 23 | ± 23 |
| n8 |  |  |  |  |  |  | 23 | ± 23 |
| n12 |  |  |  |  |  |  | 23 | ± 23 |
| n14 | 316 | +2/-3 |  |  |  |  | 23 | ± 2 |
| n20 |  |  |  |  |  |  | 23 | ± 23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 |  |  |  |  |  |  | 23 | ± 23 |
| n26 |  |  |  |  |  |  | 23 | ± 23 |
| n28 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ± 2 |
| n34 |  |  |  |  | 26 | +2/-3 | 23 | ± 2 |
| n38 |  |  |  |  |  |  | 23 | ± 2 |
| n39 |  |  |  |  | 26 | +2/-3 | 23 | ± 2 |
| n40 |  |  |  |  |  |  | 23 | ± 2 |
| n41 |  |  | 297 | 2/-33 | 26 | +2/-33 | 23 | ± 23 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ± 2 |
| n51 |  |  |  |  |  |  | 23 | ± 2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  |  |  | 23 | ± 2 |
| n70 |  |  |  |  |  |  | 23 | ± 2 |
| n71 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  |  |  | 23 | ± 2 |
| n77 |  |  | 297 | 2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  | 297 | 2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 297 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  |  |  | 23 | ± 2 |
| n81 |  |  |  |  |  |  | 23 | ± 2 |
| n82 |  |  |  |  |  |  | 23 | ± 2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  |  |  | 23 | ± 2 |
| n86 |  |  |  |  |  |  | 23 | ± 2 |
| n95 |  |  |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  |  |  | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-33 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: FFS NOTE 5: FFS NOTE 6: Generally, PC1 UE for Band n14 is not targeted for smartphone form factor. The UE power class 1. requirements for Band n14 are applicable for public safety scenario only. NOTE 7: Achieved via dual Tx. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.4C.1	Frequency error for SUL

6.4C.1.1	Test purpose

Same test purpose as in clause 6.4.1.1

6.4C.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.1.

6.4C.1.4	Test description

Same test description as specified in clause 6.4.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.1.4-1  use Table 6.4C.1.4-1

Table 6.4C.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for both SUL carrier and Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration |  | UL Configuration | SUL Configuration |  |
|  | Modulation | RB allocation | N/A | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

Table 6.4C.1.4-1: Void

6.4C.1.5	Test requirement

The 10 frequency error Δf results measured on the SUL carrier must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

<<Unchanged sections skipped>>

6.5C.3.1	General spurious emissions for SUL

6.5C.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5C.3.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.1.3	Minimum conformance requirements

The general spurious emission requirement specified in clause 6.5.3.1.3 applies to the UE that support SUL operating on the SUL bands.

The normative reference for this requirement is TS 38.101-1 [2] subclauses 6.5.3.1

6.5C.3.1.4	Test description

Same test description as specified in clause 6.5.3.1.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	Instead of table 6.5.3.1.4.1-1 use Table 6.5C.3.1.4-1

Table 6.5C.3.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for both SUL carrier and Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | CP-OFDM QPSK | OuterFull |
| 2 |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: Void |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

-	Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Tables 4.6.1-28 with condition SUL AND (RF OR RRM), 4.6.3-14 with condition SUL_SUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

6.5C.3.1.5	Test requirement

The measured average power of spurious emission on the SUL carrier, derived in step 3, shall not exceed the described value in Table 6.5C.3.1.5-1.

The spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1from the edge of the channel bandwidth.

Table 6.5C.3.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz |  |  |  |

<<Unchanged sections skipped>>

6.5C.3.3	Additional spurious emissions for SUL

6.5C.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5C.3.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.3.3	Minimum conformance requirements

The additional spurious emission requirements specified in 6.5.3.3.3 apply to the UE operating on SUL bands.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.3.3

6.5C.3.3.4	Test description

Same test description as specified in clause 6.5.3.3.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	For NS_05, instead of table 6.5.3.3.4.1-4 use Table 6.5C.3.3.4-1

-	For NS_43, instead of table 6.5.3.3.4.1-5 use Table 6.5C.3.3.4-2

-	For NS_18, instead of table 6.5.3.3.4.1-3 use Table 6.5C.3.3.4-3

-	For NS_48, instead of table 6.5.3.3.4.1-22 use Table 6.5C.3.3.4-4

-	For NS_49, instead of table 6.5.3.3.4.1-23 use Table 6.5C.3.3.4-5

-	For NS_56, instead of table 6.5.3.3.4.1-27 use Table 6.5C.3.3.4-6

<<Unchanged sections skipped>>

7.3C.2	Reference sensitivity power level for SUL

Editor’s Note:  The following aspects are either missing or not yet determined:

-	Exceptional test points for configurations except SUL_n78-n80 is FFS

<<Unchanged sections skipped>>

7.3C.2.5	Test requirement

The throughput measured in step 4 shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 for REFSENS without exception testing with receive power level specified in Tables 7.3C.2.5-1 for 2Rx antenna port and Tables 7.3C.2.5-2 for 4 Rx antenna port, and parameters specified in table 7.3C.2.4.1-1.

Table 7.3C.2.5-0: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Table 7.3C.2.5-1: Reference sensitivity QPSK PREFSENS for 2Rx

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT | -90.6 +TT | -89.6 +TT |  |  |  |  | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT | -90.7 +TT | -89.7 +TT |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT | -90.9 +TT | -89.7 +TT |  |  |  |  |  |
| n2 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |  |  |  |  |  |  |  |  |
| n3 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | -89.7 +TT | -88.9 +TT | -87.6 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -91.0 +TT | -89.8 +TT | -89.0 +TT | -87.7 +TT |  |  |  |  |  |  |
|  | 60 |  | -94.5 +TT | -92.4 +TT | -91.2 +TT | -90.0 +TT | -89.1 +TT | -87.9 +TT |  |  |  |  |  |  |
| n5 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -90.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -91.0 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |  |  |  |  |  |  |  |  |
| n8 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.0 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -90.2 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -93.6 +TT | -91.6 +TT | -90.5 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -94.0 +TT | -91.9 +TT | -90.7 +TT |  |  |  |  |  |  |  |  |  |
| n26 | 15 | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT |  |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |  |  |  |  |  |  |  |  |
| n28 | 15 | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT |  | -78.5 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  | -78.6 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | -99.0 +TT | -95.8 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.1 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT |  |  | -90.6 +TT |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  |  | -90.7 +TT |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |  | -90.9 +TT |  |  |  |  |  |  |
| n39 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT | -90.6 +TT |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT | -90.7 +TT |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT | -90.9 +TT |  |  |  |  |  |  |
| n40 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT | -90.6 +TT | -89.6 +TT |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT | -90.7 +TT | -89.7 +TT | -88.9 +TT | -87.6 +TT |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT | -90.9 +TT | -89.8 +TT | -89.1 +TT | -87.6 +TT |  |  |  |
| n411 | 15 |  | -94.8 +TT | -93.0 +TT | -91.8 +TT |  | -89.9 +TT | -88.6 +TT | -87.6 +TT |  |  |  |  | TDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  | -90.0 +TT | -88.7 +TT | -87.7 +TT | -86.9 +TT | -85.6 +TT | -85.1 +TT | -84.7 +TT |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  | -90.1 +TT | -88.9 +TT | -87.8 +TT | -87.1 +TT | -85.6 +TT | -85.1 +TT | -84.7 +TT |  |
| n481 | 15 | -99.0 +TT | -95.8 +TT | -94.0 +TT | -92.7 +TT |  |  | -89.6 +TT | -88.65 +TT |  |  |  |  | TDD |
|  | 30 |  | -96.1 +TT | -94.1 +TT | -92.9 +TT |  |  | -89.7 +TT | -88.75 +TT | -87.95 +TT | -86.65 +TT | -86.15 +TT | -85.65 +TT |  |
|  | 60 |  | -96.5 +TT | -94.4 +TT | -93.1 +TT |  |  | -89.9 +TT | -88.85 +TT | -88.05 +TT | -86.75 +TT | -86.25 +TT | -85.75 +TT |  |
| n50 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT |  | -91.9 +TT | -90.6 +TT | -89.6 +TT |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  | -92.0 +TT | -90.7 +TT | -89.7 +TT | -88.9 +TT | -87.6 +TT |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  | -92.1 +TT | -90.9 +TT | -89.8 +TT | -89.1 +TT | -87.6 +TT |  |  |  |
| n51 | 15 | -100.0 +TT |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 | -100.0 +TT | -96.8 +TT |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.6+TT | -94.6+TT | -93.5+TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.0+TT | -94.9+TT | -93.7+TT |  |  |  |  |  |  |  |  |  |
| n66 | 15 | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | -92.2 +TT | -91.4 +TT | -90.1 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -96.6 +TT | -94.6 +TT | -93.5 +TT | -92.3 +TT | -91.5 +TT | -90.2 +TT |  |  |  |  |  |  |
|  | 60 |  | -97.0 +TT | -94.9 +TT | -93.7 +TT | -92.5 +TT | -91.6 +TT | -90.4 +TT |  |  |  |  |  |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.3 +TT | -91.9 +TT | -87.4 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |  |  |  |  |  |  |  |  |
| n771,4 | 15 |  | -95.3 +TT | -93.5 +TT | -92.2 +TT |  |  | -89.1 +TT | -88.1 +TT |  |  |  |  | TDD |
|  | 30 |  | -95.6 +TT | -93.6 +TT | -92.4 +TT |  |  | -89.2 +TT | -88.2 +TT | -87.4 +TT | -86.1 +TT | -85.6 +TT | -85.1 +TT |  |
|  | 60 | - | -96.0 +TT | -93.9 +TT | -92.6 +TT |  |  | -89.4 +TT | -88.3 +TT | -87.5 +TT | -86.2 +TT | -85.7 +TT | -85.2 +TT |  |
| n781 | 15 |  | -95.8 +TT | -94.0 +TT | -92.7 +TT |  |  | -89.6 +TT | -88.6 +TT |  |  |  |  | TDD |
|  | 30 |  | -96.1 +TT | -94.1 +TT | -92.9 +TT |  |  | -89.7 +TT | -88.7 +TT | -87.9 +TT | -86.6 +TT | -86.1 +TT | -85.6 +TT |  |
|  | 60 |  | -96.5 +TT | -94.4 +TT | -93.1 +TT |  |  | -89.9 +TT | -88.8 +TT | -88.0 +TT | -86.7 +TT | -86.2 +TT | -85.7 +TT |  |
| n791 | 15 |  |  |  |  |  |  | -89.6 +TT | -88.6 +TT |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  | -89.7 +TT | -88.7 +TT | -87.9 +TT | -86.6 +TT |  | -85.6 +TT |  |
|  | 60 |  |  |  |  |  |  | -89.9 +TT | -88.8 +TT | -88.0 +TT | -86.7 +TT |  | -85.7 +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2C.1 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: TT for each frequency and channel bandwidth is specified in Table 7.3C.2.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<End of change>>