3GPP TSG-RAN5 Meeting #94-e	R5-221034r1

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1724 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update report config for iRAT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For EN-DC event-triggered reporting test cases, Table H.3.1-4 updates the reportConfig in order to include SSB-Index reporting. The correspondig iRAT event-triggered reporting test cases with SSB-Index (8.4.2.3, 8.4.2.4) do not have a similar reference to activate the SSB-Index reporting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new table H.3.4-8 referring to Table 4.6.6-8A of TS 36.508 and updating the necessary IEs for SSB-Index reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SSB-Index reporting will not be configured for iRAT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: added reportRS-IndexResultsNR to the new Table |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

## H.3.4	E-UTRA RRC messages and information elements contents exceptions for NR measurement configuration

RRCConnectionReconfiguration

Includes the nr-SecondaryCellGroupConfig-r15 to convey NR RRCReconfiguration message as specified in TS 38.331 [13].

Table H.3.4-1: RRCConnectionReconfiguration: NR RRC Reconfiguration in EN-DC

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MCG_and_SCG |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 |  | RRCReconfiguration |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table H.3.4-1a: RRCConnectionReconfiguration for measurement configuration

For measurement configuration provided by NR PSCell under EN-DC.

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MEAS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| measConfig |  | Not present |  |  |  |  |  |  |
|  |  | MeasConfig-DEFAULT |  | Table H.3.4-4 |  | GAP_NEEDED |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 |  | RRCReconfiguration |  | Table H.3.1-1 |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| GAP_NEEDED | Measurement gap is needed in Test |

ULInformationTransferMRDC

uplink transfer of MR DC information for transferring the NR RRC Measurement Report message in EN-DC.

Table H.3.4-2: ULInformationTransferMRDC: uplink transfer of MR DC information

| Derivation Path: 36.508 [25], Table 4.6.1-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULInformationTransferMRDC ::= SEQUENCE { |  |  |  |
| ul-DCCH-MessageNR-r15 | OCTET STRING including the MeasurementReport |  |  |
| } |  |  |  |

RRCConnectionReconfigurationComplete

Includes the scg-ConfigResponseNR to convey NR RRCReconfigurationComplete message as specified in TS 38.331 [13].

Table H.3.4-3: RRCConnectionReconfigurationComplete: NR RRC Reconfiguration Complete in EN-DC

| Derivation Path: 36.508 [25], Table 4.6.1-9: with condition MCG_and_SCG |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| scg-ConfigResponseNR-r15 |  | OCTET STRING including the RRCReconfigurationComplete message according TS 38.508-1, table 4.6.1-14. |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

MeasConfig-DEFAULT

Configures measurement gap that applies to FR1 only in EN-DC as specified in TS 38.331 [13].

Table H.3.4-4: MeasConfig-DEFAULT

| Derivation Path: 36.508 [25],Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) | Freq is the frequency of E-UTRA PCell |  |
| } |  |  |  |
| } |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | INTER-RAT NR |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) | Freq is the frequency of E-UTRA PCell |  |
| } |  |  |  |
| measObjectId[2] | 2 |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR-r15 | MeasObjectNR | Table H.3.4-6 |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  | INTER-RAT NR |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigEUTRA | ReportConfigEUTRA-A4(Thres) | Set Thres according to test parameters specified in test cases | EVENT A4 |
| reportConfigInterRAT | ReportConfigInterRAT-B2-NR(EUTRA-Thres, NR-Thres) | Set EUTRA-Thres and NR-Thres according to test parameters specified in test cases | EVENT B2 |
|  | ReportConfigInterRAT-B1-NR(EUTRA-Thres, NR-Thres) | Set EUTRA-Thres and NR-Thres according to test parameters specified in test cases | EVENT B1 |
|  | ReportConfigInterRAT-SFTD with Condition INTER-RAT | TS 36.508 [25] Table 4.6.6-8B | SFTD |
|  | ReportConfigInterRAT-PERIODICAL | Table 4.6.6-9 in TS 36.508 [25] | PERIODICAL |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  | INTER-RAT NR |
| measId[1] | 1 |  |  |
| measObjectId[1] | 2 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-FR1 | Table H.3.4-5 |  |
|  | Not present |  | GAPLESS |
| fr1-Gap-r15 | false |  | gapUE |
|  | true |  | gapFR1 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| gapUE | fr1-Gap-r15 set to false indicates the gap is applicable for measurements on FR1 and FR2. E-UTRAN includes this field only when the UE is configured with (NG)EN-DC. |
| gapFR1 | fr1-Gap-r15 set to true indicates the gap is only applicable for measurements on FR1. E-UTRAN includes this field only when the UE is configured with (NG)EN-DC. |
| INTER-RAT NR | Measurement configuration for inter-RAT NR measurements |
| EVENT B2 | For event B2 triggered measurement reporting test cases |
| EVENT B1 | For event B1 triggered measurement reporting test cases |
| EVENT A4 | For event A4 triggered measurement reporting |
| SFTD | For inter-RAT SFTD measurement test cases |
| GAPLESS | MG is not needed in test |
| PERIODICAL | For periodic measurement reporting test cases |

MeasGapConfig-FR1

Configures FR1 measurement gap by LTE RRC in EN-DC as specified in TS 38.331 [13].

Table H.3.4-5: MeasGapConfig-FR1

| Derivation Path: 36.508 [25],Table 4.6.6-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 39 | TGRP = 40 ms | Pattern #0 |
|  | 0 |  | BFD |
| gp4-r15 | 19 | TGRP = 20 ms | Pattern #4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Pattern #0 | gp0 corresponds to gap offset of Gap Pattern Id "0" with MGRP = 40ms |
| Pattern #4 | gp4-r15 corresponds to gap offset of Gap Pattern Id "4" with MGRP = 40ms (see TS 38.133, Table 9.1.2-1). It can be applied for (NG)EN-DC, see TS 38.133, Table 9.1.2-2. |
| BFD | Measurement gap pattern for Beam Failure Dectection tests |

Table H.3.4-6: MeasObjectNR

| Derivation Path: TS 36.508 [25] Table 4.6.6-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| rs-ConfigSSB-r15 ::= SEQUENCE { |  |  |  |
| measTimingConfig-r15 ::= SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20-r15 | 0 |  | SMTC.1 or SMTC.2 |
|  | 10 |  | SMTC.4 or SMTC.5 |
|  | 17 |  | SMTC.6 |
| sf160-r15 | 0 |  | SMTC.3 |
| } |  |  |  |
| ssb-Duration-r15 | sf1 |  | SMTC.1, SMTC.3 or SMTC.4 |
|  | sf5 |  | SMTC.2, SMTC.5 or SMTC.6 |
| } |  |  |  |
| subcarrierSpacingSSB-r15 | kHz15 |  | SSB.1 FR1, SSB.3 FR1 or SSB.5 FR1 |
|  | kHz30 |  | SSB.2 FR1, SSB.4 FR1 or SSB.6 FR1 |
|  | kHz120 |  | SSB.1 FR2, SSB.3 FR2, SSB.5 FR2 or SSB.7 FR2 |
|  | KHz240 |  | SSB.2 FR2, SSB.4 FR2, SSB.6 FR2 or SSB.8 FR2 |
| } |  |  |  |
| deriveSSB-IndexFromCell-r15 | true |  | Synchronous cells |
|  | false |  | Asynchronous cells |
| } |  |  |  |

Table H.3.4-7: RRCConnectionReconfiguration for iRAT measurement configuration

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT with Conditions INTER RAT NR and PERIODICAL | Table H.3.4-4 |  |
|  | MeasConfig-DEFAULT with Conditions INTER RAT NR and SFTD | Table H.3.4-4 | INTER-RAT and SFTD |
|  | MeasConfig-DEFAULT with Conditions INTER RAT NR and EVENT B2 | Table H.3.4-4 | INTER-RAT and EVENT B2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table H.3.4-8: ReportConfigInterRAT-B2-NR(EUTRA-Thres, NR-Thres)

| Derivation Path: 36.508 [25], Table 4.6.6-8A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-B2-NR ::= SEQUENCE { |  |  |  |
| maxReportRS-Index-r15 | 2 |  | SSB-Index |
| reportQuantityRS-IndexNR-r15 SEQUENCE { |  |  | SSB-Index |
| ss-rsrp | TRUE |  |  |
| ss-rsrq | FALSE |  |  |
| ss-sinr | FALSE |  |  |
| } |  |  |  |
| reportRS-IndexResultsNR | TRUE |  | SSB-Index |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB-Index | To include SSB Index |

< End of changes >