3GPP TSG-RAN5 Meeting #94-e	R5-221039r3

# Electronic Meeting, 21st Feb. – 04th Mar. 2022

Title:	Discussion on handling TxD test cases

Source:	Huawei, Hisilicon

Agenda item:	5.3.38.6

Document for:	Discussion

Introduction

In RAN plenary 94e, new WID on UE conformance for Transparent Tx Diversity (TxD) for NR was approved.

The purpose of this document is to analyze how to verify TxD requirements in RAN5 based on the latest RAN4 progress. Moreover, the handling TxD cases and PC 1.5 cases is also further discussed.

Discussion

RAN4 progress

In RAN4#101-bis-e, it was agreed to move 2Tx PC2 and PC1.5MPR tables to clause 6.2D, and TxD requirements in suffix G refers to 6.2D [4].

- 6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3 and UE power class 1 in Band n14, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-2, Table 6.2.2-1 and Table 6.2.2-5, respectively for channel bandwidths  ≤ 100 MHz.

If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

If the relative channel bandwidth > 4% for TDD bands or > 3% for FDD bands, the ∆MPR is defined in Table 6.2.2-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high)

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | ≤ 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2.2-3: ∆MPR

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 and n83 | Power class 3 | 30 MHz | 0.5 |
| n40 | Power class 3 and power class 2 | 100 MHz | 1 |

Table 6.2.2-4 Void

Table 6.2.2-4a Void

- 6.2D.2	UE maximum output power reduction for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1 for PC3, Table 6.2D.2-1 for PC2, Table 6.2D.2-2 and Table 6.2D.2-3 for PC1.5 respectively. For UE power class 1.5, the allowed maximum power reduction (MPR) defined in Table 6.2D.2-3 is in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the allowed MPR for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1 for PC3, Table 6.2D.2-1 for PC2, Table 6.2D.2-2 and Table 6.2D.2-3 for PC1.5 respectively, and the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode.

The same MPR requirements shall be applicable to UE with 1-layer UL MIMO transmission (either with or without ULPFTx) as with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.2 apply for the power class as indicated by the ue-PowerClass field in capability signaling.

Table 6.2D.2-1 Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | [≤ 3.5] | [≤ 1] | [0] |
|  | QPSK | [≤ 3.5] | [≤ 2] | [0.5] |
|  | 16 QAM | [≤ 3.5] | [≤ 2.5] | [≤ 1.5] |
|  | 64 QAM | [≤ 3.5] | [≤ 3] |  |
|  | 256 QAM | [≤ 5.5] |  |  |
| CP-OFDM | QPSK | [≤ 3.5] | [≤ 3.5] | [≤ 2] |
|  | 16 QAM | [≤ 3.5] | [≤ 3.5] | [≤ 2.5] |
|  | 64 QAM | [≤ 4.5] |  |  |
|  | 256 QAM | [≤ 8.5] |  |  |

Table 6.2D.2-2 Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Table 6.2D.2-3 Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

- 6.2G.2	UE maximum output power reduction for Tx Diversity

For UE supporting Tx diversity, the allowed MPR for the maximum output power in Table 6.2.1-1 is specified in Table 6.2.2-1, Table 6.2D.2-1, Table 6.2D.2-2 and Table 6.2D.2-3 for UE power class 3, 2 and 1.5 respectively. For UE power class 1.5, the allowed maximum power reduction (MPR) defined in Table 6.2D.2-3 is in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The maximum output power is defined as the sum of the maximum output power at each UE antenna connector.

Observation 1: In RAN4 endorsed CR [4], it was agreed to remove 2Tx PC2 and PC1.5 MPR requirements from clause 6.2.2.

Observation 2: In RAN4 endorsed CR [4], general TxD requirments are specified in suffix G and refer to detailed 2Tx requirements in suffix D.

TxD test case

Based on 38.101-1 v17.4.0, RAN4 has introduced clause 6.2G to separately define TxD requirements. Correspondingly in RAN5, test cases with suffix G will be specified for verifying TxD requirments under NR_RF_TxD WI.

Proposal 1: For TxD, test cases with suffix G are required to specify the measurement procedures for TxD requirements.

According to RAN4 agreement in [3]

- Leave TxD as implementation aspect and assume that UE that does not declare TxD meets 1Tx requirements and has at least one full power PA

  - Only UE supporting 23+23 for PC2 and UE supporting 26+26 for PC1.5 are allowed to report TxD

For UE supporting 23+23dBm for PC2 and 26+26dBm for PC1.5, UE couldn’t achieve the maximum out power by single PA since there is no full power PA. Therefore, UE have to report TxD support and implement TxD via 2Tx to achieve MOP, which is applicable to TxD requirements in suffix G in 38.101-1.

For UE not supporting TxD, at least one full power PA is assumed. It’s indicated that maximum output power can be achieved by single PA, which is applicable to 1Tx requirements in clause 6.2 in 38.101-1.

To sum up, it can be concluded that for PC 2 UEs reporting TxD or PC 1.5 UEs, TxD test case with suffix G will be applicable. For PC 2 UEs not reporting TxD or PC 3 UEs,test cases defined in clause 6 (clauses with no suffix G) will be applicable.

Proposal 2: For PC 2 UEs reporting TxD or PC 1.5 UEs, test cases with suffix G will be applicable when the test cases are completed. For PC 2 UEs not reporting TxD or PC 3 UEs, test cases defined in clause 6 (clauses with no suffix G) will be applicable.

PC 1.5 test cases re-arrangement

In RAN5#93e discussion R5-218209 [1], the following observations were made.

Observation 5: For PC1.5, a few test cases are quite in shape.

Observation 6: For TxD, new test cases with suffix G will be specified under NR_RF_TxD WI.

Observation 7: It needs further discussion how to re-arrange PC1.5 test cases.

In last RAN5 meeting, the re-arrangement of PC 1.5 test cases are pending since MPR requirements for PC 1.5 with dual Tx are still left in clause 6.2.2.

As per Observation 1 above, it was agreed to remove PC1.5 MPR requirements from 38.101-1 clause 6.2.2. Considering PC1.5 requirements were derived on the premise that UE indicates TxD support, it’s reasonable to merge PC1.5 test cases to TxD test cases with suffix G after contents of general TxD test cases are stable.

Proposal 3: Merge PC1.5 test cases to TxD test cases with suffix G after contents of general TxD test cases are stable.

If companies see the urgent need to move PC1.5 to clause G in the same meeting when new test cases in clause G is introduced, there would be two possible solutions of documents preparation:

- Option 1:

1.	After TxD WP is agreed (post RAN5#94e), PC1.5 WP to be updated accordingly before RAN5#95-e to move PC1.5 test cases to Clause G, respectively (PC1.5 n41, n77, n78, n79).

2.	In RAN5#95e, TxD CRs adding full structure of clause G test cases are submitted under TxD WI. Only PC2 requirements are covered.

3.	In RAN5#95e, PC1.5 CRs adding PC1.5 specific requirements are submitted under PC1.5 WIs for each band. The CRs must use corresponding TxD CRs as baseline, accepting all changes, and only PC1.5 related changes are under change mark.

4.	In the coversheet of TxD CRs and PC 1.5 CRs, implementation guideline shall be included indicating that TxD CRs shall be implemented before PC1.5 CRs

- Option 2:

- After TxD WP is agreed (post RAN5#94e), PC1.5 WP to be updated accordingly before RAN5#95-e to move PC1.5 test cases to Clause G, respectively (PC1.5 n41, n77, n78, n79).

- In RAN5#95e, TxD CRs adding full structure of clause G test cases are submitted under TxD WI. Both PC1.5 and PC2 requirements are covered. In the ‘other comment’ of coversheet, impacted PC1.5 WIs shall be listed for information.

- No CRs under PC1.5 WIs are needed. PC1.5 WP could include the TxD CRs for tracking purpose.

For Option 1, TxD cases for PC2 reporting TxD and PC 1.5 re-arrangement are handled under different WIs respectively. For Option 2, TxD cases for PC2 reporting TxD and PC 1.5 re-arrangement are handled under TxD WI.

Observation 3: If there is a need to introducing TxD cases and moving PC 1.5 cases to clause G in the same meeting, the CR handling process could be further discussed based on Option 1 or Option 2.

Proposal

Proposal 1: For TxD, test cases with suffix G are required to specify the measurement procedures for TxD requirements.

Proposal 2: For PC 2 UEs reporting TxD or PC 1.5 UEs, test cases with suffix G will be applicable when the test cases are completed. For PC 2 UEs not reporting TxD or PC 3 UEs, test cases defined in clause 6 (clauses with no suffix G) will be applicable.

Proposal 3: Merge PC1.5 test cases to TxD test cases with suffix G after contents of general TxD test cases are stable.

References

[1]	R5-218209, Discussion on Handling TxD WI and PC1.5 WIs, Huawei, HiSilicon, RAN5#93e

[2]	RP-211940, Revision of WID on UE RF requirements for Transparent Tx Diversity (TxD) for NR, Qualcomm Incorporated, RAN#93e

[3]	R4-2119970, WF on PC2 TxD implementations with 26+23 and 26+26 PA’s

[4] R4-2202349, Draft CR TS 38.101-1: moving 2Tx MPR to clause 6.2D and amending PC2 2TX MPR