3GPP TSG-RAN5 Meeting # 94-e	R5-221042r2

# Electronic Meeting,  21th February - 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2814 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Inter-System MDT test cases 8.1.6.3.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, MediaTek Inc., Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test cases of the group 8.1.6.3.2 have similar conformance requirements and procedural sequence. It is proposed to refer to the definitions in 8.1.6.3.2.1 in other test cases to keep the prose concise. Message contents for the test cases are not completely defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added references to 8.1.6.3.2.1 where appropriate in 8.1.6.3.2.2 and 8.1.6.3.2.3. Updated message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Maintenance of the prose and implemented test cases won’t be simlplified Some messge contents will remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st Revision: Merged changes from R5-220192(MediaTek Inc).and R5-220616(Huawei, HiSilicon). Added corresponding companies as co-source. In step 3 for all test cases decreased timer value to 8s. Harmonised new changes across all test cases. Added missing IEs. Reverted back few changes in 8.1.6.3.2.3 to avoid collisions with R5-220617 2nd Revision Moved 1st revision description from “Other comments” clause. Few minor editorial updates changing font colour. |  |  |  |  |  |  |  |  |

##### 8.1.6.3.2	Inter-System MDT / Logged MDT

###### 8.1.6.3.2.1	Inter-System MDT / Logged MDT / Logging and reporting / Bluetooth measurement collection

8.1.6.3.2.1.1	Test Purpose (TP)

(1)

with { UE received RRCSetup message }

ensure that {

when { UE has Bluetooth logged measurements available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE transmits the logMeasAvailableBT in the RRCSetupComplete message }

}

(2)

with { UE in NR RRC_CONNECTED state and UE has Bluetooth logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 including logMeasResultListBT }

}

(3)

with { UE has Bluetooth logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and a Bluetooth beacon is not included in BT-NameList }

ensure that {

when { UE in NR RRC_CONNECTED state and receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 not including the measurement result of the Bluetooth beacon not included in BT-NameList }

}

8.1.6.3.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.5a.1.3, 5.5a.3.2, 5.3.3.7. Unless otherwise stated these are Rel-16 requirements

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-NameList, if included, in VarLogMeasConfig;

1>	store the received sensor-NameList, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause 5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]); or

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

…

2>	when performing the logging:

…

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

8.1.6.3.2.1.3	Test description

8.1.6.3.2.1.3.1	Pre-test conditions

System Simulator:

- 	NR Cell 1, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41)

- 	Cell 40 and Cell 41 are configured as per TS 38.508-1 [4] cl 4.4.1.3.2

UE:

None

Preamble:

- 	The UE is in state 3N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.2.1.3.2	Test procedure sequence

Table 8.1.6.3.2.1.3.2-1/2 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.2.1.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 |  |  |  |
|  | BeaconRSSI | dBm |  | -80 | -50 |  |

Table 8.1.6.3.2.1.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 |  |  |  |
|  | BeaconRSSI | dBm |  | -80 | -50 |  |

Table 8.1.6.3.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 3 | Wait 8s to allow UE to activate logging | - | - | - | - |
| 4 | The SS transmits a Paging message on Cell 1. | <-- | Paging | - | - |
| 5 | The UE transmits an RRCRequest message on Cell 1. | --> | RRCRequest | - | - |
| 6 | SS transmit an RRCSetup message on Cell 1. | <-- | RRCSetup | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message with logMeasAvailableBT on Cell 1? | --> | RRCSetupComplete | 1 | P |
| 8-11 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfo-r16 with one entry (Bluetooth beacon 1) measurement result in the IE logMeasResultListBT. | --> | UEInformationResponse | 2,3 | P |

8.1.6.3.2.1.3.3	Specific message contents

Table 8.1.6.3.2.1.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.2.1.3.2-3)

| Derivation path: 38.508 table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16-IEs SEQUENCE { |  |  |  |
| bt-NameList-r16 SEQUENCE { SIZE (1..maxBT-Name-r16)) OF | 1 entry | Bluetooth beacon 1 |  |
| BT-Name-r16[1] | Set as per Table 4.4.9-1 of 3GPP TS 36.508 | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| Periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24s |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.2.1.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.3.2.1.3.2-3)

| Derivation path: 38.508 table 4.6.1-22 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  | Not checked |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| iab-NodeIndication-r16 |  | Not present |  |  |  |  |  |
| idleMeasAvailable-r16 |  | Not present |  |  |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |  |  |  |  |
| logMeasAvailable-r16 |  | true |  |  |  |  |  |
| logMeasAvailableBT-r16 |  | true |  |  |  |  |  |
| logMeasAvailableWLAN-r16 |  | Not present |  |  |  |  |  |
| connEstFailInfoAvailable-r16 |  | Not present |  |  |  |  |  |
| rlf-InfoAvailable-r16 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.6.3.2.1.3.3-3: UEInformationRequest (step 12, Table 8.1.6.3.2.1.3.2-3)

Derivation Path: 38.508-1 table 4.6.1-32A with Condition LOG

Table 8.1.6.3.2.1.3.3-4: UEInformationResponse (step 13, Table 8.1.6.3.2.1.3.2-3)

| Derivation Path: 38.508-1 table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceRecordingSessionRef-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 { |  |  |  |  |
| LogMeasInfo-r16[1] SEQUENCE { |  |  | 1 entry |  |
| locationInfo-r16 SEQUENCE { |  |  |  |  |
| commonLocationInfo-r16 |  | Not checked |  |  |
| bt-LocationInfo-r16 SEQUENCE { |  |  |  |  |
| LogMeasResultListBT-r16 ::= SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF LogMeasResultBT-r16 { |  | 1 entry |  |  |
| LogMeasResultBT-r16[1] SEQUENCE { |  |  | Report Bluetooth beacon 1 |  |
| bt-Addr-r16 |  | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r16 |  | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| wlan-LocationInfo-r16 |  | Not present |  |  |
| sensor-LocationInfo-r16 |  | Not present |  |  |
| } |  |  |  |  |
| relativeTimeStamp-r16 |  | Any allowed value | INTEGER (0..7200) |  |
| servCellIdentity-r16 |  | Same as Cell 1 |  |  |
| measResultServingCell-r16 |  | Not checked |  |  |
| measResultNeighCells-r16 |  | Not present |  |  |
| anyCellSelectionDetected-r16 |  | Not checkedt |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logMeasAvailable-r16 |  | Not present |  |  |
| logMeasAvailableBT-r16 |  | Not present |  |  |
| logMeasAvailableWLAN-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

###### 8.1.6.3.2.2	Inter-System MDT / Logged MDT / Logging and reporting / WLAN measurement collection

8.1.6.3.2.2.1	Test Purpose (TP)

(1)

with { UE received RRCSetup message }

ensure that {

when { UE has WLAN logged measurements available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE transmits the RRCSetupComplete message with logMeasAvailable-r16 and logMeasAvailableWLAN-r16 included }

}

(2)

with { UE in NR RRC_CONNECTED state and UE has WLAN logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 including logMeasResultListWLAN }

}

(3)

with { UE has WLAN logged measurements available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and a WLAN AP is not included in wlan-NameList }

ensure that {

when { UE in NR RRC_CONNECTED state and receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 not including the measurement result of the WLAN AP not included in wlan-NameList }

}

8.1.6.3.2.2.2	Conformance requirements

Same as test case 8.1.6.3.2.1.

8.1.6.3.2.2.3	Test description

8.1.6.3.2.2.3.1	Pre-test conditions

System Simulator:

- 	NR Cell 1, WLAN AP 1 (Cell 27) and WLAN AP 2 (Cell 27b)

- Cell 27 and Cell 27b are configured as per TS 38.508-1 [4] cl 4.4.1.3 with the OFFLOAD condition

UE:

None

Preamble:

- The UE is in state 3N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.2.2.3.2	Test procedure sequence

Table 8.1.6.3.2.2.3.2-1/2 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.2.2.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | Cell 27 (WLAN AP 1) | Cell 27b (WLAN AP 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 |  |  |  |
|  | BeaconRSSI | dBm |  | -60 | -50 |  |

Table 8.1.6.3.2.2.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | Cell 27 (WLAN AP 1) | Cell 27b (WLAN AP 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 |  |  |  |
|  | BeaconRSSI | dBm |  | -60 | -50 |  |

Table 8.1.6.3.2.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 3 | Wait 8s to allow UE to activate logging. | - | - | - | - |
| 4 | The SS transmits a Paging message on NR Cell 1. | <-- | Paging | - | - |
| 5 | The UE transmits an RRCRequest message on NR Cell 1. | --> | RRCRequest | - | - |
| 6 | SS transmit an RRCSetup message on NR Cell 1. | <-- | RRCSetup | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message with logMeasAvailable-r16 and logMeasAvailableWLAN-r16 on NR Cell 1? | --> | RRCSetupComplete | 1 | P |
| 8-11 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message on NR Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfo-r16 with one entry (WLAN AP 1) measurement result in the IE logMeasResultListWLAN. | --> | UEInformationResponse | 2, 3 | P |

8.1.6.3.2.2.3.3	Specific message contents

Same as test case 8.1.6.3.1.1 with the following difference:

Table 8.1.6.3.2.2.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.2.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16-IEs SEQUENCE { |  |  |  |
| wlan-NameList-r16 CHOICE { |  |  |  |
| Setup | WLAN-NameList-r16 |  |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24s |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.2.2.3.3-1A: WLAN-NameList-r16 (Table 8.1.6.3.2.2.3.3-1)

| Derivation path: TS 38.508-1 [4], table 4.6.5-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-NameList-r16 ::= SEQUENCE (SIZE (1..maxWLAN-Name-r16)) OF { | 1 entry | WLAN AP1 |  |
| WLAN-Name-r16[1] | Set as per Table 4.4.8-1 of TS 36.508 | OCTET STRING (SIZE (1..32)) Refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| } |  |  |  |

Table 8.1.6.3.2.2.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.3.2.2.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| iab-NodeIndication-r16 | Not present |  |  |  |
| idleMeasAvailable-r16 | Not present |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |  |
| logMeasAvailable-r16 | true |  |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |  |
| logMeasAvailableWLAN-r16 | true |  |  |  |
| connEstFailInfoAvailable-r16 | Not present |  |  |  |
| rlf-InfoAvailable-r16 | Not present |  |  |  |
| } |  |  |  |  |
| mobilityHistoryAvail-r16 | Not present |  |  |  |
| mobilityState-r16 | Not present |  |  |  |
| nonCriticalExtension | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.6.3.2.2.3.3-3:

Void

Table 8.1.6.3.2.2.3.3-4: UEInformationResponse (step 13, Table 8.1.6.3.2.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 { |  |  |  |
| LogMeasInfo-r16[1] SEQUENCE { |  | entry 1 |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| commonLocationInfo-r16 | Not checked |  |  |
| bt-LocationInfo-r1 | Not present |  |  |
| wlan-LocationInfo-r16 SEQUENCE (SIZE (1..maxWLAN-Id-Report-r16)) OF LogMeasResultWLAN-r16 { | 1 entry | Report WLAN AP 1 |  |
| LogMeasResultWLAN-r16[1] SEQUENCE { |  |  |  |
| wlan-Identifiers-r16 SEQUENCE { |  |  |  |
| ssid-r12 | ssid for WLAN AP 1 |  |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| rssiWLAN-r16 | Any allowed value |  |  |
| rtt-WLAN-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| sensor-LocationInfo-r16 | Not present |  |  |
| } |  |  |  |
| relativeTimeStamp-r16 | Any allowed value | INTEGER (0..7200) |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 | Not present |  |  |
| anyCellSelectionDetected-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| …….logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.3.2.3	Inter-System MDT / Logged MDT / Logging and reporting / Sensor measurement collection

8.1.6.3.2.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and UE has logged Sensor information available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 including Sensor-LocationInfo-r16 }

}

(2)

with { UE has logged Sensor information available and the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and a sensor is not included in sensor-NameList }

ensure that {

when { UE in NR RRC_CONNECTED state and receiving UEInformationRequest message with logMeasReportReq set to true }

then { UE transmits UEInformationResponse messages with LogMeasInfo-r16 not including the information of the sensor not included in sensor-NameList }

}

8.1.6.3.2.3.2	Conformance requirements

Same as test case 8.1.6.3.2.1.

8.1.6.3.2.3.3	Test description

8.1.6.3.2.3.3.1	Pre-test conditions

System Simulator:

- 	NR Cell 1

UE:

- 	The UE at least contains one sensor that supports UE orientation measurement, and optionally contains other sensors to do barometeric pressure, UE motion speed etc.

Preamble:

- 	The UE is in state 1N-A on NR cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.2.3.3.2	Test procedure sequence

Table 8.1.6.3.2.3.3.2-1/2 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.2.3.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 |  |

Table 8.1.6.3.2.3.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 |  |

Table 8.1.6.3.2.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 3 | Wait 8s to allow UE to activate logging | - | - | - | - |
| 4-11 | Steps 1 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message on Cell 1. | <-- | UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfo-r16 with one entry (sensor 1) measurement result in the IE logMeasResultListBT. | --> | UEInformationResponse | 1,2 | P |

8.1.6.3.2.3.3.3	Specific message contents

Same as test case 8.1.6.3.2.1 with the following difference:

Table 8.1.6.3.2.3.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.3.2.3.3.2-3)

| Derivation path: 38.508 table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16-IEs SEQUENCE { |  |  |  |
| traceReference-r16 | Not checked |  |  |
| traceRecordingSessionRef-r16 | Not checked |  |  |
| tce-Id-r16 | Not checked |  |  |
| absoluteTimeInfo-r16 | Not checked |  |  |
| sensor-NameList-r16 SEQUENCE { |  |  |  |
| measUeOrientation | true | UE location sensor in use |  |
| reportType CHOICE { |  |  |  |
| Periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24s |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.2.3.3.3-2:

Void

Table 8.1.6.3.2.3.3.3-3: UEInformationResponse (step 13, Table 8.1.6.3.2.3.3.2-3)

| Derivation Path: 38.508-1 table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 { |  |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| commonLocationInfo-r16 | Not checked |  |  |
| bt-LocationInfo-r16 | Not present |  |  |
| wlan-LocationInfo-r16 | Not present |  |  |
| sensor-LocationInfo-r16 ::= SEQUENCE { |  | Report sensor measurement information |  |
| sensor-MeasurementInformation-r16 | Not present |  |  |
| sensor-MotionInformation-r16 | UE motion measurement in octet string | This field provides motion sensor measurements as Sensor-MotionInformation defined in TS 37.355 |  |
| } |  |  |  |
| relativeTimeStamp-r16 | Any allowed value | INTEGER (0..7200) |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 | Not present |  |  |
| anyCellSelectionDetected-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |