3GPP TSG-RAN5 Meeting #94-e	R5-221069r1

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2818 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case name of TC 9.1.10.3 and TC 9.1.10.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tejet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case names of TC 9.1.10.3 and TC 9.1.10.4 are not correct and this CR is to correct the mistake to make them align with WP and -2 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the TC names of TC 9.1.10.3 and TC 9.1.10.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TC name will still be incorrect and user of spec will be confused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.3 , 9.1.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Missing clauses affected in coversheet are fixed. |  |  |  |  |  |  |  |  |

<Start of Change>

9.1.10.3	NSSAA / Initial registration / Rejected NSSAI, pending NSSAI

9.1.10.3.1	Test Purpose (TP)

(1)

with { UE has sent REGISTRATION REQUEST message with NSSAA bit and Request NSSAI }

ensure that {

when { SS sends REGISTRATION ACCEPT message contains rejected NSSAI with rejection cause of "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" }

then { UE stores the rejected S-NSSAI(s) in the rejected NSSAI and shall not attempt to use this S-NSSAI in the current PLMN over any access }

}

(2)

with { UE receives rejected NSSAI in the REGISTRATION ACCEPT message with rejection cause of "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" }

ensure that {

when { UE is switched off and switched on }

then { UE deletes the stored Rejected NSAAI for the current PLMN }

}

(3)

with { UE receives rejected NSSAI in the REGISTRATION ACCEPT message with rejection cause of "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" }

ensure that {

when { The UICC containing the USIM is removed }

then { UE deletes the stored Rejected NSAAI for the current PLMN }

}

(4)

with { UE stores pending NSSAI }

ensure that {

when { SS sends REGISTRATION ACCEPT message with the pending NSSAI is not included and "NSSAA to be performed" indicator is not set to "Network slice-specific authentication and authorization is to be performed"  }

then { deletes the stored pending NSSAI for the current PLMN }

<Unchange Parts omitted>

<Start of Change>

9.1.10.4	NSSAA / Initial registration / Reject

9.1.10.4.1	Test Purpose (TP)

(1)

with { UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { UE receives a REGISTRATION REJECT with cause #62 "No network slices available" }

then { UE abort the initial registration procedure, set the 5GS update status to 5U2 NOT UPDATED, enter state 5GMM-DEREGISTERED.NORMAL-SERVICE or 5GMM-DEREGISTERED.PLMN-SEARCH and reset the registration attempt counter }

}

(2)

with { UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { UE receives the rejected NSSAI in the REGISTRATION REJECT message with rejection cause of "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" }

then { UE stores the rejected NSSAI and doesn't attempt to use this S-NSSAI in the current PLMN over any access }

<End of Change>