3GPP TSG-RAN5 Meeting #94-e	R5-221077r1

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2819 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MDT TC 8.1.6.3.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TP description needs to be update according to case title and test purpose. Release information is missing in the case. The conformance requirement should be updated as this case is for RLF, not for logged measurement. The pre-test condition should be updated according to the defined NR SI Comb and BT Beacon in TS 38.508-1. UE state in the preamble shall be 3N-A (RRC_Connected). The parameter and unit in Power level tables should be for NR, not LTE. Specificc message contents should be updated according to TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TP description was updated for RLF. Release information was added. Conformance requirements were updated according to the TPs. NR SI Comb and BT Beacon were updated according to TS 38.508-1. UE state was replaced by 3N-A in the preamble. The power level tables were updated. Specific message contents updated according to TS 38.508-1 and 38.331. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformat UE will fail this case unfarily. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: LogMeasResultBT-r16 was added for bt-LocationInfo-r16. Overlaps with R5-221043 were removed. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

8.1.6.3.3.1	Inter-System MDT / Radio Link Failure / Logging and reporting / Bluetooth measurement collection

8.1.6.3.3.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state detecting radio link failure }

ensure that {

when { UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable included }

}

(2)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report-r16 including LocationInfo-r16 which includes logMeasResultListBT }

}

(3)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq and a Bluetooth beacon is not included in bt-NameList }

then { UE sends the UEInformationResponse message not including the measurement result of the Bluetooth beacon not included in bt-NameList }

}

8.1.6.3.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.3.5.9, 5.3.10.5, 5.3.3.7, 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.9]

The UE shall:

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

…

;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

…

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

2>	if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure:

3>	if the last RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover:

4>	include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.6.3.3.1.3	Test description

8.1.6.3.3.1.3.1	Pre-test conditions

System Simulator:

- NR Cell 1, NR Cell 2, Bluetooth beacon 1 (Cell 40) and Bluetooth beacon 2 (Cell 41).

- System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

- Cell 40 and Cell 41 are configured as per 38.508-1 [4] cl 4.4.1.3.

UE:

None

Preamble:

- The UE is in state 1N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.3.1.3.2	Test procedure sequence

Table 8.1.6.3.3.1.3.2-1/2 illustrate the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.3.1.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 |  |  | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -79 |  |  | Only Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 [4], Table 6.2.2.1-3. |  |  |  |  |  |  |  |

Table 8.1.6.3.3.1.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Cell 40 (Bluetooth beacon 1) | Cell 41 (Bluetooth beacon 2) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS |  |  | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | FFS |  |  | Only Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  | -80 | -50 |  |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 [4], Table 6.2.2.2.2-2. |  |  |  |  |  |  |  |

Table 8.1.6.3.3.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 2 parameter according to the row "T1" in Table 8.1.6.3.3.1.3.2-1/2. | - | - | - | - |
| 4 | The UE transmits an RRCReestablishmentRequest message on Cell 2. | --> | RRCReestablishmentRequest | - | - |
| 5 | The SS transmits an RRCReestablishment message on Cell 2. | <-- | RRCReestablishment | - | - |
| 6 | Check: Does the UE transmit an RRCReestablishmentComplete message on Cell 2 with logMeasAvailableBT included? | --> | RRCReestablishmentComplete | 1 | P |
| 7 | The SS transmits an RRCReconfiguration message on Cell 2. | <-- | RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message on Cell 2. | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS transmits a UEInformationRequest message on Cell 2. | <-- | UEInformationRequest | - | - |
| 10 | Check: Does the UE transmit a UEInformationResponse message with rlf-Report-r9 with logMeasResultListBT including one entry (Bluetooth beacon 1) measurement result? | --> | UEInformationResponse | 2, 3 | P |

8.1.6.3.3.1.3.3	Specific message contents

Table 8.1.6.3.3.1.3.3-1: RRCReconfiguration (step 1, Table 8.1.6.3.3.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| RRC-TransactionIdentifier |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| RRCReconfiguration-IEs SEQUENCE { |  |  |  |
| RRCReconfiguration-v1530-IEs SEQUENCE { |  |  |  |
| OtherConfig-v1610 SEQUENCE { |  |  |  |
| Setup BT-NameList-r16 SEQUENCE (SIZE (1..maxBT-Name-r16)) OF BT-Name-r16 SEQUENCE { | 1 entry |  |  |
| BT-Name-r16[1] | Set as per Table 4.4.9-1 of 36.508 | OCTET STRING (SIZE (1..248)) Refers to LOCAL NAME defined in Bluetooth specification |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.3.1.3.3-2: RRCReestablishmentComplete (step 6, Table 8.1.6.3.3.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-11 RRCSetupComplete |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| logMeasAvailable-r16 | TRUE |  |  |
| logMeasAvailableBT-r16 | TRUE |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| connEstFailInfoAvailable-r16 | Not present |  |  |
| rlf-InfoAvailable-r16 | true |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.3.1.3.3-2A: UEInformationRequest (step 9, Table 8.1.6.3.3.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with Condition RLF

Table 8.1.6.3.3.1.3.3-3: UEInformationResponse (step 10, Table 8.1.6.3.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 | Present | Specified in Table 8.1.6.3.3.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.3.1.3.3-4: rlf-Report-r16 (Table 8.1.6.3.3.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 | Not checked |  |  |
| measResultNeighCells-r16 | Not checked |  |  |
| c-RNTI-r16 | Not checked |  |  |
| previousPCellId-r16 | Not checked |  |  |
| failedPCellId-r16 | Not checked |  |  |
| reconnectCellId-r16 | Not checked |  |  |
| timeUntilReconnection-r16 | Not checked |  |  |
| reestablishmentCellId-r16 | Not checked |  |  |
| timeConnFailure-r16 | Not checked |  |  |
| timeSinceFailure-r16 | Not checked |  |  |
| connectionFailureType-r16 | rlf |  |  |
| rlf-Cause-r16 | Not checked |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| commonLocationInfo-r16 | Not checked |  |  |
| bt-LocationInfo-r16 SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF LogMeasResultBT-r16 SEQUENCE { | 1 entry |  |  |
| LogMeasResultBT-r16[1] SEQUENCE { |  | Report Bluetooth beacon 1 |  |
| bt-Addr-r16 | Bluetooth public address of Bluetooth beacon 1 | BIT STRING (SIZE (48)) |  |
| rssi-BT-r16 | Any allowed value | INTEGER (-128..127) |  |
| } |  |  |  |
| } |  |  |  |
| wlan-LocationInfo-r16 | Not checked |  |  |
| sensor-LocationInfo-r16 | Not checked |  |  |
| } |  |  |  |
| noSuitableCellFound-r16 | Not checked |  |  |
| ra-InformationCommon-r16 | Not checked |  |  |
| csi-rsRLMConfigBitmap-v1650 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]