3GPP TSG-RAN5 Meeting #94-e	R5-221095r2

# Electronic Meeting, 21st Feb – 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2826 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new SNPN test case for EAP based primary authentication and key agreement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New test case SNPN for EAP based primary authentication and key agreement needs to be added as per work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case added ##### 9.1.11.3 SNPN / EAP based primary authentication and key agreement / EAP-AKA' related procedures Revision 1: CR Title alignment with 3GU Revision 2: Corrected Year on CR cover page |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE may fail to interoperate with network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.11.3 [New] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... 38.523-2CR ... 0211 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 9.1.11.2.3.3-2: REGISTRATION REQUEST (step 18, Table 9.1.11.2.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ngKSI |  |  |  |
| NAS key set identifier | '111'B | no key is available |  |
| TSC | Any allowed value | TSC does not apply for NAS key set identifier value "111" |  |
| 5GS mobile identity | The valid SUCI | Only SUCI is available. |  |
| Last visited registered TAI | Not present |  |  |

#### 9.1.11.3	SNPN / EAP based primary authentication and key agreement / EAP-AKA' related procedures

9.1.11.3.1	Test Purpose (TP)

(1)

with { the UE in 5GMM-REGISTERED-INITIATED state }

ensure that {

when { the SS sends an EAP-request/AKA'-challenge message within AUTHENTICATION REQUEST }

then { the UE sends an EAP-response/AKA'-challenge message within AUTHENTICATION RESPONSE }

}

(2)

with { the UE in 5GMM-REGISTERED-INITIATED state and SS initiates an EAP based primary authentication and key agreement procedure }

ensure that {

when { the SS sends an EAP-success message within AUTHENTICATION RESULT }

then { the UE considers the procedure complete and authentication procedure succeed }

}

(3)

with { the UE in 5GMM-REGISTERED-INITIATED state and SS initiates an EAP based primary authentication and key agreement procedure}

ensure that {

when { the SS sends an EAP-failure message within AUTHENTICATION REJECT }

then { the UE shall consider the entry of the "list of subscriber data" with/without the SNPN identity of the current SNPN as invalid for 3GPP access until the UE is switched off or the entry is updated, and the USIM is considered invalid until switching off the UE }

}

9.1.11.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501 clauses 5.4.1.2.2.3, 5.4.1.2.2.8, 5.4.1.2.2.11 , 9.12.1

[TS 24.501, clause 5.4.1.2.2.3 ]

If a USIM is present and the SNN check is successful, the UE shall handle the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [40]. The USIM shall derive CK and IK and compute the authentication response (RES) using the 5G authentication challenge data received from the ME, and pass RES to the ME. The ME shall derive CK' and IK' from CK and IK, and EMSK from CK' and IK'. Furthermore, the ME may generate KAUSF from the EMSK, the KSEAF from the KAUSF, and the KAMF from the ABBA received together with the EAP-request/AKA'-challenge message, and the KSEAF as described in 3GPP TS 33.501 [24], and create a partial native 5G NAS security context identified by the ngKSI value received together with the EAP-request/AKA'-challenge message in subclause 5.4.1.2.4.2 in the volatile memory of the ME. If the KAMF and the partial native 5G NAS security context are created, the ME shall store the KAMF in the created partial native 5G NAS security context, and shall send an EAP-response/AKA'-challenge message as specified in IETF RFC 5448 [40].

If the EAP-request/AKA'-challenge message contains AT_RESULT_IND attribute, the UE may include AT_RESULT_IND attribute in the EAP-response/AKA'-challenge message as specified in IETF RFC 5448 [40].

[TS 24.501, clause 5.4.1.2.2.8]

Upon receiving an EAP-success message, if the ME has not generated a partial native 5G NAS security context as described in subclause 5.4.1.2.2.3, the ME shall:

a)	generate the KAUSF from the EMSK, the KSEAF from the KAUSF, and the KAMF from the ABBA that was received with the EAP-success message, and the KSEAF as described in 3GPP TS 33.501 [24];

b)	create a partial native 5G NAS security context identified by the ngKSI value in the volatile memory of the ME; and

c)	store the KAMF in the created partial native 5G NAS security context.

The UE shall consider the procedure complete.

[TS 24.501, clause 5.4.1.2.2.11]

Upon receiving an EAP-failure message, the UE shall delete the partial native 5G NAS security context if any was created as described in subclause 5.4.1.2.2.3.

The UE shall consider the procedure complete.

If the EAP-failure message is received in an AUTHENTICATION REJECT message:

1)	if the AUTHENTICATION REJECT message has been successfully integrity checked by the NAS:

-	The UE shall set the update status to 5U3 ROAMING NOT ALLOWED, delete the stored 5G-GUTI, TAI list, last visited registered TAI and ngKSI;

In case of PLMN, the USIM shall be considered invalid until switching off the UE or the UICC containing the USIM is removed;

In case of SNPN, the entry of the "list of subscriber data" with the SNPN identity of the current SNPN shall be considered invalid until the UE is switched off or the entry is updated. Additionally, the UE shall consider the USIM as invalid for the current SNPN until switching off or the UICC containing the USIM is removed;

-	The UE shall set:

i)	the counter for "SIM/USIM considered invalid for GPRS services" events, the counter for "USIM considered invalid for 5GS services over non-3GPP access" events, and the counter for "SIM/USIM considered invalid for non-GPRS services" events if maintained by the UE, in case of PLMN; or

ii)	the counter for "the entry for the current SNPN considered invalid for 3GPP access" events and the counter for "the entry for the current SNPN considered invalid for non-3GPP access" events in case of SNPN;

to UE implementation-specific maximum value;

[TS 24.501, clause 9.12.1]

The serving network name (SNN) is used:

-	in the Network name field of the AT_KDF_INPUT attribute defined in IETF RFC 5448 [40];

-	in KAUSF derivation function as specified in 3GPP TS 33.501 [24] annex A; and

-	in RES* and XRES* derivation function as specified in 3GPP TS 33.501 [24] annex A.

SNN shall contain a UTF-8 string without terminating null characters.

SNN is of maximum length of 1020 octets.

SNN consists of SNN-service-code and SNN-network-identifier, delimited by a colon.

SNN-network-identifier identifies the serving PLMN or the serving SNPN.

MCC and MNC in the SNN-PLMN-ID are MCC and MNC of the serving PLMN. If the MNC of the serving PLMN has two digits, then a zero is added at the beginning.

MCC and MNC in the SNN-SNPN-ID are MCC and MNC of the serving SNPN. If the MNC of the serving SNPN has two digits, then a zero is added at the beginning.

SNN-NID contains an NID in hexadecimal digits.

ABNF syntax of SNN is specified in table 9.12.1.1

Table 9.12.1.1: ABNF syntax of SNN

SNN = SNN-service-code ":" SNN-network-identifier

SNN-service-code = %x35.47 ; "5G"

SNN-network-identifier = SNN-PLMN-ID / SNN-SNPN-ID

SNN-PLMN-ID = SNN-mnc-string SNN-mnc-digits "." SNN-mcc-string SNN-mcc-digits "." SNN-3gppnetwork-string "." SNN-org-string ; applicable when not operating in SNPN access operation mode.

SNN-SNPN-ID = SNN-mnc-string SNN-mnc-digits "." SNN-mcc-string SNN-mcc-digits "." SNN-3gppnetwork-string "." SNN-org-string ":" SNN-NID ; applicable when operating in SNPN access operation mode.

SNN-mnc-digits = DIGIT DIGIT DIGIT ; MNC of the PLMN ID

SNN-mcc-digits = DIGIT DIGIT DIGIT ; MCC of the PLMN ID

SNN-mnc-string = %x6d.6e.63 ; "mnc" in lower case

SNN-mcc-string = %x6d.63.63 ; "mcc" in lower case

SNN-3gppnetwork-string = %x33.67.70.70.6e.65.74.77.6f.72.6b ; "3gppnetwork" in lower case

SNN-org-string = %x6f.72.67 ; "org" in lower case

SNN-NID = 11SNN-hexadecimal-digit ; NID in hexadecimal digits

SNN-hexadecimal-digit = DIGIT / %x41 / %x42 / %x43 / %x44 / %x45 / %x46

NOTE:	SNN-service-code allows for distinguishing of ANID specified in 3GPP TS 24.302 [16] and SNN as either of SNN or ANID can be carried in the AT_KDF_INPUT attribute.

EXAMPLE 1:	In case of a PLMN, if PLMN ID contains MCC = 234 and MNC = 15, SNN is 5G:mnc015.mcc234.3gppnetwork.org.

EXAMPLE 2:	In case of an SNPN, if SNPN ID contains a PLMN ID of MCC = 234 and MNC = 15 and an NID of 123456ABCDEH, SNN is 5G:mnc015.mcc234.3gppnetwork.org:123456ABCDE.

9.1.11.3.3	Test description

9.1.11.3.3.1	Pre-test conditions

System Simulator:

-	SNPN cell NGC Cell is configured according to Table 6.3.2.2-1 broadcasting default SNPN ID as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-12 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NGC Cells.

UE:

-	The UE is in Automatic SNPN selection mode.

-	The UE is provisioned with a “list of subscriber data” to allow access to SNPN identified by NGC Cell A.

Preamble:

-	NGC Cell A is set to “Serving Cell”.

-	The UE is in state Switched OFF (state 0-A).

9.1.11.3.3.2	Test procedure sequence

Table 9.1.11.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-4 | The UE establishes RRC connection and initiates registration procedure by executing steps 2-4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 5 | SS transmits a correct AUTHENTICATION REQUEST message with an EAP-Request/AKA'-challenge message. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 6 | Check: Does the UE respond with a correct AUTHENTICATION RESPONSE message, with an EAP-Response/AKA'-challenge message? | --> | 5GMM: AUTHENTICATION RESPONSE | 1 | P |
| 7 | The SS transmits an “EAP-failure” message within AUTHENTICATION REJECT | <-- | 5GMM: AUTHENTICATION REJECT | - | - |
| 8 | SS releases the RRC connection | - | - | - | - |
| 9 | Check: Does the UE transmit an RRCSetupRequest message for initial registration procedure within the next 30 seconds? | --> | NR RRC: RRCSetupRequest | 3 | F |
| 10 | The UE is switched off by executing generic procedure in Table 4.9.6.4-1 in TS 38.508-1 [4]. | - | - | - | - |
| 11 | The UE is switched on. | - | - | - | - |
| 12-14 | The UE establishes RRC connection and initiates registration procedure by executing steps 2-4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 15 | SS transmits a correct AUTHENTICATION REQUEST message with an EAP-Request/AKA'-challenge message. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 16 | Check: Does the UE respond with a correct AUTHENTICATION RESPONSE message, with an EAP-Response/AKA'-challenge message? | --> | 5GMM: AUTHENTICATION RESPONSE | 1 | P |
| 17 | SS transmits an AUTHENTICATION RESULT message with an EAP-success message. | <-- | 5GMM: AUTHENTICATION RESULT | - | - |
| 18-24 | The registration procedure is performed by executing steps 8-14 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 25 | Check: Does the UE transmits a REGISTRATION COMPLETE message? | --> | 5GMM: REGISTRATION COMPLETE | 2 | P |
| 26 | Steps 19a1 of Table 4.5.2.2-2 in TS 38.508-1 [4] are performed | - | - | - | - |

9.1.11.3.3.3	Specific message contents

Table 9.1.11.3.3.3-1: Message AUTHENTICATION RESPONSE (step 6, 16, Table 9.1.11.3.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EAP message | EAP-Response/AKA'-Challenge | RES* equal to the XRES* calculated in the SS with the parameters provided/indicated in the AUTHENTICATION REQUEST | EAP-AKA |

Table 9.1.11.3.3.3-2: Message AUTHENTICATION RESULT (step 17, Table 9.1.11.3.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EAP message | EAP-Success |  | EAP-AKA |

Table 9.1.11.3.3.3-3: AUTHENTICATION REJECT (step 7, Table 9.1.11.3.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended protocol discriminator | 5GMM |  |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| EAP message | EAP-failure | EAP-failure |  |
| NOTE: This message is sent within SECURITY PROTECTED 5GS NAS MESSAGE message with Integrity protected and ciphered. |  |  |  |