3GPP TSG-RAN5 Meeting #94-e	R5-221134r1

# Electronic Meeting, 21st Feb – 04th Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2273 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating test frequencies for NR band n1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CBW 45MHz is introduced for band n1 in R17. The test frequencies for n1 need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addning test frequencies for 45MHz for n1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequencies for n1 is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: correcting typo in table 4.3.1.1.1.1-1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.3.1.1.1	NR operating bands in FR1

4.3.1.1.1.1	Reference test frequencies for NR operating band n1

Table 4.3.1.1.1.1-1: Test frequencies for NR operating band n1 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2119.39 | 423878 | 102 |  | 5350 | 427970 | 8 | 1 | 0 (0) | 103 |
|  |  |  | High | 2167.5 | 433500 | 2074.53 | 414906 | 504 |  | 5418 | 433470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1857.03 | 371406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1977.5 | 395500 | 1974.17 | 394834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2116.96 | 423392 | 102 |  | 5344 | 427490 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 2165 | 433000 | 2069.6 | 413920 | 504 |  | 5405 | 432490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1854.6 | 370920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1975 | 395000 | 1969.24 | 393848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2114.53 | 422906 | 102 |  | 5338 | 427010 | 0 | 0 | 1 (2) | 104 |
|  |  |  | High | 2162.5 | 432500 | 2064.67 | 412934 | 504 |  | 5395 | 431570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1852.17 | 370434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1972.5 | 394500 | 1964.31 | 392862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2112.1 | 422420 | 102 |  | 5332 | 426530 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 2160 | 432000 | 2059.74 | 411948 | 504 |  | 5382 | 430590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1849.74 | 369948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1970 | 394000 | 1959.38 | 391876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2109.67 | 421934 | 102 |  | 5326 | 426050 | 4 | 0 | 1 (2) | 104 |
|  |  |  | High | 2157.5 | 431500 | 2054.81 | 410962 | 504 |  | 5369 | 429610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1920.53 | 384106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1847.31 | 369462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1967.5 | 393500 | 1954.45 | 390890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2107.24 | 421448 | 102 |  | 5320 | 425570 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 2155 | 431000 | 2049.88 | 409976 | 504 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1935 | 387000 | 1920.6 | 384120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1844.88 | 368976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1965 | 393000 | 1949.52 | 389904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2102.2 | 420440 | 102 |  | 5308 | 424610 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 2150 | 430000 | 2039.84 | 407968 | 504 |  | 5330 | 426490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1940 | 388000 | 1920.56 | 384112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1839.84 | 367968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1960 | 392000 | 1939.48 | 387896 | 6 |  | - | - | - | - | - | - |
| 45 | 242 | Downlink | Low | 2132.5 | 426500 | 2110.72 | 422144 | 0 | 15 | 5284 | 422690 | 2 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2099.86 | 419972 | 102 |  | 5302 | 424130 | 6 | 1 | 1(2) | 105 |
|  |  |  | High | 2147.5 | 429500 | 2035 | 407000 | 504 |  | 5320 | 425570 | 10 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1920.72 | 384144 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1837.5 | 367500 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1957.5 | 391500 | 1934.64 | 386928 | 6 |  | - | - | - | - | - | - |
| 50 | 270 | Downlink | Low | 2135 | 427000 | 2110.7 | 422140 | 0 | 15 | 5282 | 422650 | 2 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 2140 | 428000 | 2097.34 | 419468 | 102 |  | 5296 | 423650 | 2 | 0 | 2 (4) | 106 |
|  |  |  | High | 2145 | 429000 | 2029.98 | 405996 | 504 |  | 5307 | 424590 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1945 | 389000 | 1920.7 | 384140 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1834.98 | 366996 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1955 | 391000 | 1929.62 | 385924 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.1-2: Test frequencies for NR operating band n1 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2098.96 | 419792 | 102 |  | 5350 | 427970 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 2165 | 433000 | 1979.24 | 395848 | 504 |  | 5411 | 432970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1764.24 | 352848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1975 | 395000 | 1968.52 | 393704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2140 | 428000 | 2096.44 | 419288 | 102 |  | 5344 | 427490 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2162.5 | 432500 | 1974.22 | 394844 | 504 |  | 5401 | 432050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1761.72 | 352344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1972.5 | 394500 | 1963.5 | 392700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2094.1 | 418820 | 102 |  | 5338 | 427010 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 2160 | 432000 | 1969.38 | 393876 | 504 |  | 5388 | 431070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1759.38 | 351876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1970 | 394000 | 1958.66 | 391732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2091.58 | 418316 | 102 |  | 5332 | 426530 | 2 | 0 | 2 (7) | 218 |
|  |  |  | High | 2157.5 | 431500 | 1964.36 | 392872 | 504 |  | 5375 | 430090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1920.8 | 384160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1756.86 | 351372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1967.5 | 393500 | 1953.64 | 390728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2089.24 | 417848 | 102 |  | 5326 | 426050 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2155 | 431000 | 1959.52 | 391904 | 504 |  | 5362 | 428930 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1935 | 387000 | 1920.96 | 384192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1754.52 | 350904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1965 | 393000 | 1948.8 | 389760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2084.2 | 416840 | 102 |  | 5314 | 425090 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 2150 | 430000 | 1949.48 | 389896 | 504 |  | 5336 | 426970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1940 | 388000 | 1920.92 | 384184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1749.48 | 349896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1960 | 392000 | 1938.76 | 387752 | 6 |  | - | - | - | - | - | - |
| 45 | 119 | Downlink | Low | 2132.5 | 426500 | 2111.08 | 422216 | 0 | 15 | 5290 | 423170 | 6 | 0 | 3(8) | 16 |
|  |  |  | Mid | 2140 | 428000 | 2081.86 | 416372 | 102 |  | 5308 | 424610 | 10 | 0 | 2(7) | 218 |
|  |  |  | High | 2147.5 | 429500 | 1944.64 | 388928 | 504 |  | 5326 | 426050 | 14 | 0 | 1(6) | 1020 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1921.08 | 384216 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1747.14 | 349428 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1957.5 | 391500 | 1933.92 | 386784 | 6 |  | - | - | - | - | - | - |
| 50 | 133 | Downlink | Low | 2135 | 427000 | 2111.06 | 422212 | 0 | 15 | 5288 | 423130 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 2140 | 428000 | 2079.34 | 415868 | 102 |  | 5302 | 424130 | 18 | 0 | 2 (7) | 218 |
|  |  |  | High | 2145 | 429000 | 1939.62 | 387924 | 504 |  | 5313 | 425070 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1945 | 389000 | 1921.06 | 384212 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1744.62 | 348924 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1955 | 391000 | 1928.9 | 385780 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.1-3: Test frequencies for NR operating band n1 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  | Mid | 2140 | 428000 | 2062.6 | 412520 | 102 |  | - | 427568 |
|  |  |  | High | 2165 | 433000 | 1798.16 | 359632 | 504 |  | - | 432568 |
|  |  | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1583.16 | 316632 | 504 |  | - | - |
|  |  |  | High | 1975 | 395000 | 1966.72 | 393344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  | Mid | 2140 | 428000 | 2060.08 | 412016 | 102 |  | - | 427064 |
|  |  |  | High | 2162.5 | 432500 | 1793.14 | 358628 | 504 |  | - | 431564 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1580.64 | 316128 | 504 |  | - | - |
|  |  |  | High | 1972.5 | 394500 | 1961.7 | 392340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  | Mid | 2140 | 428000 | 2057.92 | 411584 | 102 |  | - | 426632 |
|  |  |  | High | 2160 | 432000 | 1788.48 | 357696 | 504 |  | - | 430632 |
|  |  | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1578.48 | 315696 | 504 |  | - | - |
|  |  |  | High | 1970 | 394000 | 1957.04 | 391408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
|  |  |  | Mid | 2140 | 428000 | 2055.4 | 411080 | 102 |  | - | 426128 |
|  |  |  | High | 2157.5 | 431500 | 1783.46 | 356692 | 504 |  | - | 429628 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1921.34 | 384268 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1575.96 | 315192 | 504 |  | - | - |
|  |  |  | High | 1967.5 | 393500 | 1952.02 | 390404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
|  |  |  | Mid | 2140 | 428000 | 2052.88 | 410576 | 102 |  | - | 425624 |
|  |  |  | High | 2155 | 431000 | 1778.44 | 355688 | 504 |  | - | 428624 |
|  |  | Uplink | Low | 1935 | 387000 | 1921.32 | 384264 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1573.44 | 314688 | 504 |  | - | - |
|  |  |  | High | 1965 | 393000 | 1947 | 389400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  | Mid | 2140 | 428000 | 2048.2 | 409640 | 102 |  | - | 424688 |
|  |  |  | High | 2150 | 430000 | 1768.76 | 353752 | 504 |  | - | 426688 |
|  |  | Uplink | Low | 1940 | 388000 | 1921.64 | 384328 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1568.76 | 313752 | 504 |  | - | - |
|  |  |  | High | 1960 | 392000 | 1937.32 | 387464 | 6 |  | - | - |
| 45 | 58 | Downlink | Low | 2132.5 | 426500 | 2111.62 | 422324 | 0 | 15 | - | 422684 |
|  |  |  | Mid | 2140 | 428000 | 2045.68 | 409136 | 102 |  | - | 424184 |
|  |  |  | High | 2147.5 | 429500 | 1763.74 | 352748 | 504 |  | - | 425684 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1921.62 | 384324 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1566.24 | 313248 | 504 |  | - | - |
|  |  |  | High | 1957.5 | 391500 | 1932.3 | 386460 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 2135 | 427000 | 2111.6 | 422320 | 0 | 15 | - | 422680 |
|  |  |  | Mid | 2140 | 428000 | 2043.16 | 408632 | 102 |  | - | 423680 |
|  |  |  | High | 2145 | 429000 | 1758.72 | 351744 | 504 |  | - | 424680 |
|  |  | Uplink | Low | 1945 | 389000 | 1921.6 | 384320 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1563.72 | 312744 | 504 |  | - | - |
|  |  |  | High | 1955 | 391000 | 1927.28 | 385456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

<End of Changes>