3GPP TSG-RAN5 Meeting #94-e                                                                 R5-221164r1

# Electronic Meeting, 21st Feb 2022 - 4th Mar 2022

Agenda item:	5.3.1.17

Source: 	Qualcomm Inc

Title: 	Discussion on fading crest factor

Document for:	Discussion and Endorsement

## 1.	Introduction

In TS 38.521-4 [5], there are still few 64QAM test points which are untestable for 28GHz bands due to achievable SNR lower than the required SNR. Lot more test points for 39GHz (n260) and 41GHz(n259) remains untestable due to similar reason as 28GHz bands. Also 256QAM test points which were later added to the spec have not been evaluated yet.

This contribution provides simulation results for PAPR calculations for all the FR2 Demod and CSI test points currently in [5] and discusses the way forward for further improving the achievable SNR.

## 2.	Discussion

In [3], we provided the simulation results for the 64QAM test point with the highest SNR requirement in the spec. Based on that data, 17.71 dB backoff value was chosen in coming up with achievable SNR for IFF system. 
Table 7.1.1_1-1 of TS 38.521-4 [5] summarizes the current achievable max SNR for different bands and channel BW.

Table 7.1.1_1-2 and Table 8.1.1_1-2 summarizes the current testability of the various test points for different bands and channel BW.

## 3.	Simulation results and analysis

We ran the simulations for all the FR2 test points (including 256QAM) currently defined in 38.521-4 [5]. Below table summarizes the data. All simulations were run for close to 1E+08 samples.  The worst case signal PAPR and faded signal PAPR is captured.

In the context of PAPR, it is better to reference the PAPR number to a probability. For 1e+8 samples, 1e-6 probability would be reasonable to avoid significant run-to-run variation in PAPR results  (100 occurrences on average at 1e-6 probability).

So the PAPR values at 1e-6 probability is also provided in the below table.

Table 3-1: Faded signal PAPR for all test points defined in TS 38.521-4

|  | Test# | Fading | MCS | Test SNR | numSamples | Mean Signal Pwr (Rx0/Rx1) | Mean Faded signal Pwr (Rx0/Rx1) | Signal PAPR (Rx0/Rx1) | Faded Signal PAPR (Rx0/Rx1) | Faded Signal PAPR at 1e-6 clipping prob |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1-1 | TDLC60-300 Low | QPSK | 1.4 | 1.23E+08 | -4.44¦-4.45 | -1.39¦-1.37 | 12.89¦13.08 | 16.87¦17.40 |  |
| 2 | 1-2 | TDLA30-300 Low | 16QAM | 3.6 |  |  |  |  | leverage from 2-2 |  |
| 3 | 1-3 | TDLA30-300 XPL Med | 64QAM | 14.2 | 1.23E+08 | -4.44¦-4.45 | -1.52¦-1.39 | 13.04¦13.15 | 17.78¦18.20 | 15.72¦16.21 |
| 4 | 1-4 | TDLD30-75 Low | 256QAM | 21.9 | 9.22E+07 | -4.58¦-4.59 | -1.72¦-1.60 | 13.31¦13.29 | 17.0¦16.24 | 15.08¦14.82 |
| 5 | 2-1 | TDLA30-75 Low | QPSK | 5.8 |  |  |  |  | leverage from 2-6 |  |
| 6 | 2-2 | TDLA30-300 Low | 16QAM | 16 | 1.23E+08 | -4.44¦-4.45 | -1.5¦-1.43 | 12.95¦12.96 | 16.37¦16.52 |  |
| 7 | 2-6 | TDLA30-75 Low | 64QAM | 20.3 | 3.98E+08 | -4.44 | -1.54 | 13.77¦13.23 | 17.77¦17.01 | 14.73¦14.54 |
| 8 | 3-1 | TDLA30-75 ULA Medium | 16QAM | 20.7 | 1.23E+08 | -4.44¦-4.45 | -1.71¦-1.78 | 12.95¦12.96 | 16.03¦16.48 | 14.19¦14.55 |
| 9 | 8.2.2.2.2 | TDLA30-35 ULA High | 256QAM | 6/7, 12/13, 7/8, 20/21 | 9.22E+07 | -4.59¦-4.59 | -1.70¦-1.90 | 12.86¦12.82 | 19.24¦20.72 | 17.69¦19.05 |
| 10 | 8.3.2.2.1 | TDLA30-35 Low | 16QAM |  | 1.23E+08 | -4.45¦-4.45 | -1.86¦-1.96 | 12.76¦12.85 | 17.80¦17.19 | 15.35¦15.45 |
| 11 | 8.4.2.2 Test1/2 | TDLA30-35 Low | 64QAM | 0,16 | 1.23E+08 | -4.45¦-4.45 | -1.86¦-2.0 | 13.46¦12.90 | 15.98¦15.86 |  |
| 12 | 8.4.2.2 Test3 | TDLA30-35 XP High | 64QAM | 16 | 1.23E+08 | -4.45¦-4.45 | -1.80¦-2.07 | 13.25¦13.27 | 16.99¦18.05 | 15.57¦15.54 |
| 13 | 8.4.2.2 Test3 | TDLA30-35 XP High | 64QAM | 16 | 1.23E+08 | -4.45¦-4.45 | -1.86¦-2.08 | 13.46¦12.90 | 15.42¦15.77 |  |
| 14 | 8.2.2.2.1.1 | AWGN only | 64QAM | 8/9, 14/15 |  | Signal PAPR already available as part of other cases |  |  |  |  |

Among all the 64QAM test points, previously we had gone with the worst case backoff 17.71 dB based on test point 2-6 which we thought would give the highest PAPR value. But as seen in the above table, test 1-3 has a slightly higher PAPR value.

Like also mentioned in [3] and [4], our proposal is to go with the worst case PAPR value obtained at 1e-6 clipping probability.

Proposal 1: Use 16.21 dB (replacing the current assumption of 17.71 dB) as the overall backoff for maximum testable SNR calculation for 64QAM scenarios.

Among the 256QAM test points, 8.2.2.2.2 scenario requires the highest PAPR value. Looking at the 1e-6 clipping probability point, we propose backoff value of 19.05 dB for 256QAM scenarios

Proposal 2: Use 19.05 dB as the overall backoff for maximum testable SNR calculation for 256QAM scenarios.

This ~1.5 dB improvement in testable SNR for 64QAM scenarios will make 4 more test points testable for n260 which means only 2 test points will remain untestable. For n259 band, this improvement in testable SNR will not help make any more test points testable. 7 test points still remain untestable for n259.

With the data for 256QAM, all the test points can be deemed testable for 28 GHz bands (n257, n258, n261). Testability issue remains for n259 and n260 bands.

Given we have exhausted all the options as listed in [4] under RAN5 control to improve the testable SNR, our proposal is to send a LS to RAN4 highlighting the remaining testability issue and get their input on the following options to improve the testability for remaining test points.

- If any of the improvements that are being studied as part of the new testability SI will help here

- Feasibility of configuring BWP with very low numRB for these test points which will result in improving the achievable SNR.

Proposal 3: Send a LS to RAN4 highlighting the remaining testability issues and get their input on some of the ways to improve the testability of remaining test points

Based on Table 3-1, we have the test points for {64QAM, 256QAM} * {Demod, CSI} cases which have the highest PAPR requirement.

For these test points we simulated the faded signal clipping that a UE under test will be subjected to and ran the throughput vs SNR curve for different faded signal clipping values.

The results are summarized in table 3-2

Table 3-2: Throughput values for key test points at different faded signal clipping probabilities

| Test# | Test SNR | Scenario | 1E-06 | 1E-04 | 1E-03 | 1E-02 |
| --- | --- | --- | --- | --- | --- | --- |
| 1-3 | 14.2 | 64QAM Demod | 100% | 100% | 100% | 100% |
| 2-6 | 20.3 | 64QAM Demod | 100% | 100% | 100% | 100% |
| 1-4 | 22 | 256QAM Demod | 100% | 100% | 100% | 83% |
| 8.2.2.2.2 | 13 | 64QAM CSI | 100% | 100% | 100% | 98% |
| 8.2.2.2.2 | 21 | 256QAM CSI | 100% | 100% | 97% | 85% |
| Note: All throughput values normalized to the no faded signal clipping case |  |  |  |  |  |  |

Observation 1: For 64QAM scenarios, Demod test points shows no throughput degradation across SNR for 1e-2 or better faded signal clipping probability. Whereas for CSI test point, no throughput degradation for 1e-3 or better clipping.

Observation 2: For 256QAM scenarios, Demod test point shows no throughput degradation for 1e-3 or better faded signal clipping probability and CSI test point shows no throughput degradation for 1e-4 or better faded signal clipping.

In Table 3-3 and Table 3-4, we have summarized the additional SNR improvement needed for Demod and CSI test points for different bands to make them testable.

Table 3-3: Additional SNR improvement needed for Demod test points to make them testable

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | -3.9 dB | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 16 | x | x | -5.7 dB | -1.5 dB | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | -2.1 dB | x | x |
|  | 2-4 | 200 | Yes | 15.8 | x | x | -8.8 dB | -1.3 dB | x |
|  | 2-5 | 50 | Yes | 16 | x | x | -1.4 dB | x | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | -10 dB | -5.8 dB | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | -10.4 dB | -6.2 dB | x |
| 7.2.2.2.1_3 | 1-4 | 50 | Yes | 22 | -1.2 dB | -1.2 dB | -11.7 dB | -7.5 dB | -1.2 dB |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |
| 7.3.2.2.3 | 1-1 | 100 | Yes | 4.7 | x | x | x | x | x |

Table 3-4: Additional SNR improvement needed for CSI test points to make them testable

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 8.2.2.2.1.1 | 1 | 100 | No | 9 | x | x | x | x | x |
|  | 2 | 100 | No | 15 | x | x | x | x | x |
| 8.2.2.2.2.1 | 1 | 100 | Yes | 7 | x | x | x | x | x |
|  | 2 | 100 | Yes | 13 | x | x | -2.7 dB | x | x |
| 3 | 50 | Yes | 8 | TBD | TBD | TBD | TBD | TBD |
|  | 4 | 50 | Yes | 21 | TBD | TBD | -7.4 dB | -3.3 dB | TBD |
| 8.4.2.2.1 | 1 | 100 | Yes | 0 | x | x | x | x | x |
|  | 2 | 100 | Yes | 16 | x | x | - | - | x |
|  | 3 | 100 | Yes | 16 | x | x | - | - | x |

In Table 3-5, we summarize the different fading backoff values for 64QAM and 256QAM scenarios at different faded signal clipping probability. This is based on the simulation results summarized in table 3-1.

Table 3-5: Fading backoff values for 64QAM and 256QAM test points at different faded signal clipping probabilities

| Fading backoff (dB) | 64QAM Demod | 256QAM Demod | 64QAM CSI | 256QAM CSI |
| --- | --- | --- | --- | --- |
| current | 17.77 | TBD |  |  |
| 1E-6 clipping prob |  |  |  |  |
| 1E-4 clipping prob |  | 12.62 |  | 15.36 |
| 1E-3 clipping prob | 11.08 | 10.96 |  | 12.96 |
| 1E-2 clipping prob | 8.34 |  | 8.34 |  |

Since we observe throughput degradation at the configured test SNR compared to the no signal clipping case with 1e-2 faded signal clipping probability, our recommendation is to go with the backoff margin corresponding to the 1e-3 probability for all 64QAM scenarios.

Proposal 4: For 64QAM scenarios (both Demod and CSI), consider fading backoff margin of 11.08 dB (replacing the current working assumption of 17.71 dB) corresponding to the 1e-3 faded signal clipping probability.

Proposal 5: For 256QAM scenarios (both Demod and CSI), consider fading backoff margin of 12.62 dB (worst case) corresponding to 1e-4 fading signal clipping probability.

Plugging the fading backoff margin values from prop4 and prop5 in the TR 38.903 [6] Demod SNR range calculator spreadsheet, we get the updated testability of various points as summarized in table 3-6 and table 3-7.

Table 3-6: Testability of Demod test points based on prop4 and prop5

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | x | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  |  | 2-2 |  | 100 |  |  | Yes | 16 |  |  | x |  |  | x |  | x |  | x | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | x | x | x |
|  |  | 2-4 |  | 200 |  |  | Yes | 15.8 |  |  | x |  |  | x |  | -1.7 dB |  | x | x |
|  | 2-5 | 50 | Yes | 16 | x | x | x | X | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | -1.5 dB | x | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | -1.9 dB | x | x |
| 7.2.2.2.1_3 |  | 1-4 |  | 50 |  |  | Yes | 22 |  |  | x |  |  | x |  | -3.2 dB |  | x | x |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |
| 7.3.2.2.3 | 1-1 | 100 | Yes | 4.7 | x | x | x | x | x |

Table 3-7: Testability of CSI test points based on prop4 and prop5

| Test Case | Test point |  | CHBW / MHz |  | Fading |  | SNR test requirement |  | Test Point Applicability |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | n257 |  | n258 |  |  | n259 |  | n260 |  |  | n261 |  |
| 8.2.2.2.1.1 | 1 |  | 100 |  | No |  | 9 |  | x |  | x |  |  | x |  | x |  |  | x |  |
|  | 2 |  | 100 |  | No |  | 15 |  | x |  | x |  |  | x |  | x |  |  | x |  |
| 8.2.2.2.2.1 | 1 |  | 100 |  | Yes |  | 7 |  | x |  | x |  |  | x |  | x |  |  | x |  |
|  | 2 |  | 100 |  | Yes |  | 13 |  | x |  | x |  |  | x |  | x |  |  | x |  |
| 3 |  |  |  | 50 |  |  |  | Yes |  |  | 8 |  |  |  | x | x |  |  |  |  | x |  | x |  | x |  |
|  |  |  |  | 4 |  |  |  | 50 |  |  |  | Yes |  |  | 21 |  |  |  |  | x |  | x |  | -2.2 dB |  | x |  | x |  |
| 8.4.2.2.1 | 1 |  | 100 |  | Yes |  | 0 |  | x |  | x |  |  | x |  | x |  |  | x |  |
|  | 2 |  | 100 |  | Yes |  | 16 |  | x |  | x |  |  | x |  | x |  |  | x |  |
|  | 3 |  | 100 |  | Yes |  | 16 |  | x |  | x |  |  | x |  | X |  |  | x |  |

With prop4 and prop5, all the test points (64QAM and 256QAM) can be deemed testable for 28GHz (n257, n258, n261) and 39 GHz (n260) bands. For n259, some of the previously untestable points became testable. But some still remain untestable.

Proposal 6: All the test points (both 64QAM and 256QAM) can be deemed testable for 28GHz and 39 GHz bands. Update the corresponding sections in TS 38.521-4 and TR 38.903 with the updated testability for all applicable bands.

## 4.	Conclusion

Proposal 1: Use 16.21 dB (replacing the current assumption of 17.71 dB) as the overall backoff for maximum testable SNR calculation for 64QAM scenarios.

Proposal 2: Use 19.05 dB as the overall backoff for maximum testable SNR calculation for 256QAM scenarios.

Proposal 3: Send a LS to RAN4 highlighting the remaining testability issues and get their input on some of the ways to improve the testability of remaining test points

Observation 1: For 64QAM scenarios, Demod test points shows no throughput degradation across SNR for 1e-2 or better faded signal clipping probability. Whereas for CSI test point, no throughput degradation for 1e-3 or better clipping.

Observation 2: For 256QAM scenarios, Demod test point shows no throughput degradation for 1e-3 or better faded signal clipping probability and CSI test point shows no throughput degradation for 1e-4 or better faded signal clipping.

Proposal 4: For 64QAM scenarios (both Demod and CSI), consider fading backoff margin of 11.08 dB (replacing the current working assumption of 17.71 dB) corresponding to the 1e-3 faded signal clipping probability.

Proposal 5: For 256QAM scenarios (both Demod and CSI), consider fading backoff margin of 12.62 dB (worst case) corresponding to 1e-4 fading signal clipping probability.

Proposal 6: All the test points (both 64QAM and 256QAM) can be deemed testable for 28GHz and 39 GHz bands. Update the corresponding sections in TS 38.521-4 and TR 38.903 with the updated testability for all applicable bands.

## 5.		References

- R5-205702, On Fading Impact on CREST and Demod Testability Analysis, Rohde & Schwarz

- R5-206614, On the achievable SNR for FR2 demod and perf test cases, Anritsu

[3]  R5-211083, Input on fading crest factor margin for FR2 Demodulation test cases, Qualcomm Inc

[4]  R5-215829, Discussion on max testable SNR for Demod scenarios, Qualcomm Inc

[5]  TS 38.521-4 v16.10.0

[6]  TR 38.903 v16.10.1