3GPP TSG-RAN5 Meeting #94-e	R5-221261r2

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2283 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of mid test channel bandwidth for band n25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, T-Mobile USA Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Mid channel bandwidth for UEs supporting 40 MHz CHBW is not closest to the mean of all CHBWs of band n25 which is a common approach for determining mid CHBW. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mid channel bandwidth for band n25 has been corrected for UEs supporting 40MHz CHBW. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs supporting 40 MHz CHBW in band n25 will not be tested with a channel bandwidth closest to the average of all channel bandwidths of band n25. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: added T-Mobile USA Inc. as co-source r2: added note 6 to 15 MHz CHBW for band n25. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1 and Table 4.3.1.0A-2 for FR1 and FR2 respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz] |
| --- | --- |
| n1 | 156, 257 |
| n2 | 15 |
| n3 | 20 |
| n5 | 15 |
| n7 | 156, 257 |
| n8 | 15 |
| n12 | 10 |
| n14 | 10 |
| n20 | 15 |
| n24 | 10 |
| n25 | 156, 209 |
| n26 | 108 |
| n28 | 15 |
| n29 | 102 |
| n30 | 10 |
| n34 | 10 |
| n38 | 15 |
| n39 | 20 |
| n40 | 30 |
| n41 | 60 |
| n48 | 204, 405 |
| n50 | 20 |
| n51 | 5 |
| n53 | 10 |
| n65 | 15 |
| n66 | 20 |
| n70 | 15 |
| n71 | 10 |
| n74 | 15 |
| n75 | 152 |
| n76 | 52 |
| n77 | 50 |
| n78 | 50 |
| n79 | 60 |
| n80 | 203 |
| n81 | 153 |
| n82 | 153 |
| n83 | 153 |
| n84 | 153, 6, 257 |
| n86 | 203 |
| n95 | 103 |
| n97 | 30 |
| n99 | 103 |
| Note 1: For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest channel BW to the average channel BW of all supported channel bandwidths among all SCSs. If there are two channel bandwidths that have same distance to the mathematical center, the higher one is selected. This shall apply only for Rel 15 UEs. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: This Mid test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 20MHz. Note 7: This Mid test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 50MHz. Note 8: This Mid test channel bandwidth is chosen since it is more commonly used. Note 9: This Mid test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 40MHz. |  |

Table 4.3.1.0A-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 200 |
| n258 | 200 |
| n260 | 200 |
| n261 | 200 |
| Note 1: For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest channel BW to the average channel BW of all supported channel bandwidths among all SCSs. If there are two channel bandwidths that have same distance to the mathematical center, the higher one is selected. This shall apply only for Rel 15 UEs. |  |

< End of changes >