3GPP TSG-RAN5 Meeting #94-e                                                    R5-221281r1

Electronic Meeting, 8th – 19th November 2021

Title: 	Discussion on Transparent Tx diversity work plan

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.38.6

Document for:	Discussion and Endorsement

# 1.	Introduction

New work item on UE Conformance – UE RF requirements for Transparent Tx Diversity (TxD) for NR was endorsed in RAN5#93e ([1]) and finally agreed in RAN#94e ([2]).

In RAN5#94e, a draft work plan is needed so the work can progress in coming meetings.

This document provides company views on such work plan.

# 2.	Discussion

There are several aspects from parent work item ([3]) that need to be analysed in order to draft a proper Work plan:

- New sections with a new clause suffix (G) have been added to define core requirements for Tx diversity ([4]):

  - UE maximum output power

  - MPR

  - A-MPR

  - Configured transmitted power

  - Minimum output power

  - Transmit OFF power

  - Transmit ON/OFF time mask

  - Power Control

  - Frequency error

  - Transmit modulation quality

  - Occupied bandwidth

  - Out of band emission

  - Spurious emissions

  - Transmit intermodulation

  - Annex: EVM measurement for dual Tx

Proposal 1: Define in the Work Plan one test case per each of the core requirements identified with clause suffix G in core requirements.

- In RAN4#101e, it was endorsed to move PC2, PC1.5 dual-Tx related MPR requirements from Clause 6.2.2, 6.2G.2 to Clause 6.2D.2 ([9]). To be noticed that:

  - This change has not reached core specifications yet.

  - Despite the requirements were moved from clause 6.2G.2 to 6.2D.2, clause 6.2G.2 is not voided but it includes a pointer to core requirements in 6.2D.2.

  - The discussion in RAN4 about this movement indicated that this was just a documentation issue ([8]:

  - To be noticed that, from RAN5 test procedure definition point of view, signalling involved in UL MIMO test cases and TxD test cases is different despite applicable requirements might be the same.

  - Mixing UL MIMO and TxD under the same test procedure will cause test system validation and device certification very complex and misleading.

Proposal 2: Keep the definition of TxD MPR under a test case with clause suffix G to facilitate a correct test system validation and device certification.

- A new capability has been defined so the UE can indicate the support of transparent Tx diversity requirements specified in the suffix G clauses in 38.101-2 ([5], [6]):

- RAN4 intention is to define this feature applicable from Rel-15 (still TBC – refer to [7], endorsed during RAN4#101e but not yet in core requirements specification):

- As old devices might not declare the txDiversity-r16 capability, RAN4 has agreed that devices that don’t declare TxD meets 1 Tx requirements and has at least one full power PA ([8]):

Proposal 3: Work plan should consider the new txDiversity-r16 capability either for test cases applicability definition as well as for test specifications 38.508-2 and 38.522, explicitly indicating that if the new txDiversity-r16 capability is not declared, test cases with clause G won’t be applicable. All new added test cases are applicable from [Rel-15] onwards if the device supports txDiversity-r16 capability.

# 3. Conclusion

This document provides company views on UE Conformance – UE RF requirements for Transparent Tx Diversity (TxD) for NR work plan and makes the following proposals:

Proposal 1: Define in the Work Plan one test case per each of the core requirements identified with clause suffix G in core requirements.

Proposal 2: Keep the definition of TxD MPR under a test case with clause suffix G to facilitate a correct test system validation and device certification.

Proposal 3: Work plan should consider the new txDiversity-r16 capability either for test cases applicability definition as well as for test specifications 38.508-2 and 38.522, explicitly indicating that if the new txDiversity-r16 capability is not declared, test cases with clause G won’t be applicable. All new added test cases are applicable from [Rel-15] onwards if the device supports txDiversity-r16 capability.

# 4.	References

[1] R5-217759, “New WID on UE Conformance – UE RF requirements for Transparent diversity (TxD) for NR”, Huawei, HiSilicon, RAN5#93e.

[2] RP-213229, “New WID on UE Conformance – UE RF requirements for Transparent diversity (TxD) for NR”, Huawei, HiSilicon, RAN5#93e.

[3] RP-211940, “WID on UE RF requirements for Transparent Tx Diversity (TxD) for NR”, Qualcomm, RAN#93e.

[4] R4-2115100, “CR for TS 38.101-1 Tx diversity requirements”, Huawei, HiSilicon, vivo, OPPO, CMCC, Qualcomm, RAN4#100e

[5] R2-2111503, “CR on 38.306 for introducing UE capability of txDiversity”, CMCC, CATT, China Telecom, China Unicom. Huawei, HiSilicon, OPPO, vivo , RAN2#116e

[6] R2-2111502, “CR on 38.331 for introducing UE capability of txDiversity”, CMCC, CATT, China Telecom, China Unicom. Huawei, HiSilicon, OPPO, vivo, RAN2#116e

[7] R4-2119976, “CR for TS 38.307: release independent requirements for TxD”, Huawei, HiSilicon, RAN4#101e

[8] R4-2119923, “Email discussion summary for [101-e][123] NR_TxD”, Qualcomm Incorporated, RAN4#101e

[9] R4-2119977, “Draft CR TS 38.101-1: Move PC2, PC1.5 MPR to Clause 6.2D”, Huawei, HiSilicon, Qualcomm, RAN4#101e