3GPP TSG-RAN WG5 Meeting #94-e 	R5-221316r1

# Electronic Meeting, 21th February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1325 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Rx Requirements for 4 Rel-17 ENDC combos |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update reference sensitivity test requirement for four ENDC combos DC_2A_n77A, DC_5A_n77A, DC_13A_n77A, and DC_66A_n77A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added exceptional requirement into Table 7.3B.2.0.3.1-1 and Table 7.3B.2.0.3.1-2 for 4 ENDC combos - Updated Note 13, added Notes 15, 16, 17 into Table 7.3B.2.0.3.1-1 based on 38.101 Table 7.3B.2.3.1-1 - Added exceptional requirement into Table 7.3B.2.0.3.2-1 and Table 7.3B.2.0.3.2-2 for DC_2A_n77A and DC_13A_n77A - Updated notes 9 and 11 of Table 7.3B.2.0.3.2-1 - Updated Table 7.3B.2.0.3.5.1-1 for 4 ENDC combos and note section of the table based on 38.101 Table 7.3B.2.3.5.1-1 - Updated Table 7.3B.2.0.3.5.1-1a and note section for 4 ENDC combos - Updated Table 7.3B.2.3.4.2.1-6 for 4 combos - Updated Table 7.3B.2.3.5-1 for 4 combos and note 13 and 17 of the table based on 38.101 Table 7.3B.2.3.1-1 corresponding notes. - Updated Table 7.3B.2.3.5-2 for DC_2A_n77A and DC_13A_n77A - Updated Table 7.3B.2.3.5-4 for 4 combos |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core requirement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR38.905 CR R5-221320 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | r1: Editorial change for Note 13 of Table 7.3B.2.0.3.1-1 |
| --- | --- |

### <Start of Changes>

### 7.3B.2	Reference sensitivity for EN-DC

#### 7.3B.2.0	Minimum Conformance Requirements of Reference sensitivity for EN-DC

##### 7.3B.2.0.1	Intra-band contiguous EN-DC

For intra-band contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels. The reference sensitivity requirements apply with all uplink carriers and all downlink carriers active for EN-DC configuration and Uplink EN-DC configuration listed in Table 5.5B.2-1 and Table 5.5B.3-1, as supported by the UE. For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements apply with single uplink transmission. The downlink carrier(s) from the cell group with uplink shall be configured closer to the uplink operating band than any of the downlink carriers from the cell group without uplink.

Sensitivity degradation is allowed for Intra-band contiguous EN-DC configurations listed in Table 7.3B.2.0.1-1, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3B.2.0.1-1 and E-UTRA and NR single carrier requirements do not apply.

Table 7.3B.2.0.1-1: Reference sensitivity (MSD) for intra-band contiguous EN-DC

| EN-DC configuration/channel allocations/MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 |  |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 | FDD |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 |  |
|  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 | FDD |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [5], with additional limits on configured maximum output power for the uplink according to subclause 6.2B.4. |  |  |  |  |  |  |  |

##### 7.3B.2.0.2	Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels.

For DC_3A_n3A intra-band non-contiguous EN-DC combination, only single switched UL is supported in rel.15, therefore, no MSD is specified and E-UTRA and NR single carrier requirements apply.

##### 7.3B.2.0.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

7.3B.2.0.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.0.3.1-1 with uplink configuration of the aggressor band (low) specified in Table 7.3B.2.0.3.1-2.

Table 7.3B.2.0.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band |  | 5 MHz (dB) |  | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1, 3 |  | n772, 13 |  |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  |  | n773 |  |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 2 |  |  | n772, 13 |  |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  |  |  | n773 |  |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 |  | n782, 13 |  |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  |  | n783 |  |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 3 |  | n782, 13 |  |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  |  | n783 |  |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 5 |  | n786,7 |  |  |  | 10.5 | 8.9 | 7.8 |  |  | 5.4 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| 5 | n776, 7, 17 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 | 2.8 | 2.3 | 2.1 | 1.4 |
|  | n774, 5, 17 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 8 |  | n418,9 |  |  |  | 13 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 |  | 4.3 | 3.9 | 3.5 |
| 8 |  | n776,7 n786,7 |  |  |  | 10.8 | 9.1 | 8 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| 8 |  | n794,5 |  |  |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| 12 |  | n668,9,10 |  | 10 |  | 7.5 | 6.2 | 5.5 |  |  | 2.4 |  |  |  |  |  |  |
| 12 |  | n784,5 |  |  |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 13 |  |  | n774, 5 |  |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 18, 19 |  | n774,5 n784,5 |  |  |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 28 |  | n512,13 |  | 27.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n513 |  | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  | n774,5 n784,5 |  |  |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 20 |  | n776,7 n786,7 |  |  |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| 26 |  | n418,9 |  |  |  | 10.3 | 8.4 | 7.4 |  |  | 5 | 4.3 | 3.9 |  | 3.1 | 2.7 |  |
| 26 |  | n776,7 n786,7 |  |  |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| n28 |  | 18,9,10 |  | 10.2 |  | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
| n71 |  | 211 |  | 4.6 |  | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
|  |  | 212 |  | 1.7 |  | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
| n71 |  | 76,7 |  | 14.6 |  | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |  |
| 66 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 |  | n782, 13 |  |  |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  |  | n783 |  |  |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n66 |  | 482,13 |  | 27.3 |  | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |  |
|  |  | 483 |  | 1.9 |  | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE NOTE 15: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 16: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL. This band is subject to 2nd harmonic fall in B21 also which MSD is not specified. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band |  |  | SCS of UL band (kHz) |  | 5 MHz (LCRB) |  | 10 MHz (LCRB) |  | 15 MHz (LCRB) |  | 20 MHz (LCRB) |  | 25 MHz (LCRB) |  | 30 MHz (LCRB) |  | 40 MHz (LCRB) |  | 50 MHz (LCRB) |  | 60 MHz (LCRB) |  | 70 MHz (LCRB) |  | 80 MHz (LCRB) |  | 90 MHz (LCRB) |  | 100 MHz (LCRB) |
| 1 |  | n77 |  |  | 15 |  |  |  | 25 |  | 36 |  | 50 |  |  |  |  |  | 100 |  | 100 |  | 100 |  |  |  | 100 |  | 100 |  | 100 |
| 2 | n77 |  |  | 15 |  |  |  | 25 |  | 36 |  | 50 |  | 50 |  | 50 |  | 50 |  | 50 |  | 50 |  | 50 |  | 50 |  | 50 |  | 50 |
| 2 |  | n78 |  |  | 15 |  |  |  | 25 |  | 36 |  | 50 |  |  |  |  |  | 50 |  | 50 |  | 50 |  |  |  | 50 |  | 50 |  | 50 |
| 3 |  | n77 |  |  | 15 |  |  |  | 25 |  | 36 |  | 50 |  |  |  |  |  | 50 |  | 50 |  | 50 |  |  |  | 50 |  | 50 |  | 50 |
| 3 |  | n78 |  |  | 15 |  |  |  | 25 |  | 36 |  | 50 |  |  |  |  |  | 50 |  | 50 |  | 50 |  |  |  | 50 |  | 50 |  | 50 |
| 5 | n77 |  |  | 15 |  |  |  | 16 |  | 25 |  | 25 |  | 25 |  | 25 |  | 25 |  | 25 |  | 25 |  | 25 |  | 25 |  | 25 |  | 25 |
| 5 |  | n78 |  |  | 15 |  | 8 |  | 16 |  | 25 |  | 25 |  |  |  |  |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  | n41 |  |  | 15 |  |  |  | 16 |  | 25 |  | 25 |  |  |  |  |  | 25 |  | 25 |  | 25 |  |  |  | 25 |  | 25 |  | 25 |
| 8 |  | n77 n78 |  |  | 15 |  |  |  | 16 |  | 25 |  | 25 |  |  |  |  |  | 25 |  | 25 |  | 25 |  |  |  | 25 |  | 25 |  | 25 |
| 8 |  | n79 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 |  | 25 |  | 25 |  |  |  | 25 |  |  |  | 25 |
| 12 |  | n66 |  |  | 15 |  | 8 |  | 16 |  | 20 |  | 20 |  |  |  |  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  | n78 |  |  | 15 |  |  |  | 10 |  | 15 |  | 20 |  |  |  |  |  | 25 |  | 25 |  | 25 |  |  |  | 25 |  | 25 |  | 25 |
| 13 | n77 |  |  | 15 |  |  |  | 15 |  | 20 |  | 20 |  | 20 |  | 20 |  | 20 |  | 20 |  | 20 |  | 20 |  | 20 |  | 20 |  | 20 |
| 18 |  | n77, n78 |  |  | 15 |  |  |  | 16 |  | 25 |  | 25 |  |  |  |  |  | 25 |  | 25 |  | 25 |  |  |  | 25 |  | 25 |  | 25 |
| 19 |  | n77, n78 |  |  | 15 |  |  |  | 16 |  | 25 |  | 25 |  |  |  |  |  | 25 |  | 25 |  | 25 |  |  |  | 25 |  |  |  | 25 |
| 20 |  | n77 n78 |  |  | 15 |  |  |  | 16 |  | 25 |  | 251, 252 |  |  |  |  |  | 25 |  | 25 |  | 25 |  |  |  | 25 |  | 25 |  | 25 |
| 26 |  | n41 |  |  | 15 |  |  |  | 16 |  | 25 |  | 25 |  |  |  |  |  | 25 |  | 25 |  |  |  |  |  |  |  |  |  |  |
| 26 |  | n77 n78 |  |  | 15 |  |  |  | 16 |  | 25 |  | 25 |  |  |  |  |  | 25 |  | 25 |  | 25 |  |  |  | 25 |  | 25 |  | 25 |
| n28 |  | 1 |  |  | 15 |  | 8 |  | 16 |  | 25 |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  | n51 |  |  | 15 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  | n77 n78 |  |  | 15 |  |  |  | 10 |  | -15 |  | 20 |  |  |  |  |  | 25 |  | 25 |  | 25 |  |  |  | 25 |  | 25 |  | 25 |
| 66 | n77 |  |  | 15 |  |  |  | 25 |  | 36 |  | 50 |  | 64 |  | 80 |  | 100 |  | 100 |  | 100 |  | 100 |  | 100 |  | 100 |  | 100 |
| 66 |  | n78 |  |  | 15 |  |  |  | 25 |  | 36 |  | 50 |  |  |  |  |  | 100 |  | 100 |  | 100 |  |  |  | 100 |  | 100 |  | 100 |
| n66 |  | 48 |  |  | 15 |  | 12 |  | 25 |  | 36 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 |  | 2 |  |  | 15 |  | 254 85 |  | 254 85 |  | 204 85 |  | 204 85 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 |  | 7 |  |  | 15 |  | 8 |  | 16 |  | 25 |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1 [2] for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1 [2] applies. NOTE 2: Void. NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 4: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 5: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 6: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.0.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) are specified in Table 7.3B.2.0.3.2-1 with uplink configuration of the aggressor band (high) specified in Table 7.3B.2.0.3.2-2.

Table 7.3B.2.0.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| n77 |  | 2 |  | 6.1 |  | 5.0 |  | 4.0 |  |  | 3.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 132 |  | 31 |  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n78 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 282 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |
| n78 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n79 | 114 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 192 | 29.5 | 26.5 | 24.7 |  |  |  |  |  |  |  |  |
| n79 | 214 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 262 | 27 | 24 | 22.2 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHBfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and in MHz and fULHBfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: Void. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 2 | n71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n41 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n77 | 2 |  | 15 |  | 25 |  | 50 |  | 75 |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 |  | 13 |  | 15 |  | 25 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n78 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n79 | 11 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1[2]. NOTE 4: Unless otherwise stated, the UL resource blocks allocation is applied at the centre of the channel bandwidth. The note applies to the entire table. NOTE 5: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### <Skipped unhanged sections>

7.3B.2.0.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex M  the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [5] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [5] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [5], with parameters specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.0.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving two bands

Table 7.3B.2.0.3.5.1-1: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration |  |  | EUTRA or NR band |  |  | UL Fc (MHz) |  |  |  |  |  | UL/DL BW (MHz) |  |  |  | UL LCRB |  |  |  |  | DL Fc (MHz) |  |  |  |  | MSD (dB) |  |  |  | IMD order |  |  |  |  |
| DC_1_n3 |  |  | 1 |  |  | 1950 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2140 |  |  |  |  | 23 |  |  |  | IMD3 |  |  |  |  |
|  |  |  | n3 |  |  | 1760 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1855 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_1A_n5A |  |  | 1 |  |  | 1965 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2155 |  |  |  |  | 6 |  |  |  | IMD4 |  |  |  |  |
|  |  |  | n5 |  |  | 836.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 876.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_1A_n77A |  |  | 1 |  |  | 1950 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2140 |  |  |  |  | 29.8 |  |  |  | IMD23 |  |  |  |  |
| DC_1A_SUL_n77A- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 32.54 |  |  |  |  |  |  |  |  |
| n84A |  |  | n77, n78 |  |  | 4090 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 4090 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_1A_n77A |  |  | 1 |  |  | 1950 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2140 |  |  |  |  | 8.0 |  |  |  | IMD43 |  |  |  |  |
| DC_1A-SUL_n77A-n84A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |  |  |  |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A |  |  | n77, n78 |  |  | 3710 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3710 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_1A_n78A, |  |  | 1 |  |  | 1950 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2140 |  |  |  |  | 8.0 |  |  |  | IMD43 |  |  |  |  |
| DC_1A_SUL_n78A- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |  |  |  |  |
| n84A |  |  | n78 |  |  | 3710 |  |  |  |  |  | 10 |  |  |  | 25 |  |  |  |  | 3710 |  |  |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_2A_n66A |  |  | 2 |  |  | 1855 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1935 |  |  |  |  | 20 |  |  |  | IMD3 |  |  |  |  |
|  |  |  | n66 |  |  | 1775 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_2A_n66A |  |  | 2 |  |  | 1883.3 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | n66 |  |  | 1750 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2150 |  |  |  |  | 4 |  |  |  | IMD5 |  |  |  |  |
| DC_2A_n77A DC_2A-2A_n77A |  | 2 |  |  | 1855 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1935 |  |  |  |  | 26 |  |  |  |  | IMD2 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 3790 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3790 |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  |
|  |  | 2 |  |  | 1900 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1980 |  |  |  |  | 8.0 |  |  |  |  | IMD4 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 3720 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3720 |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  |
|  |  | 2 |  |  | 1885 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1965 |  |  |  |  | 5 |  |  |  |  | IMD5 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.74 |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  | 3810 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3810 |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  |
|  |  |  | 2 |  |  | 1855 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1935 |  |  |  |  | 26 |  |  |  | IMD23 |  |  |  |  |
| DC_2A_n78A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  |  | 3790 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3790 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | 2 |  |  | 1885 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1965 |  |  |  |  | 8.0 |  |  |  | IMD43 |  |  |  |  |
| DC_2A_n78A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  |  | 3690 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3700 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_3_n1 |  |  | 3 |  |  | 1760 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1855 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | n1 |  |  | 1950 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2140 |  |  |  |  | 23 |  |  |  | IMD3 |  |  |  |  |
|  |  |  | 3 |  |  | 1771 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 1866 |  |  |  |  | 4 |  |  |  | IMD4 |  |  |  |  |
| DC_3_n5 |  |  | n5 |  |  | 838 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 883 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | 3 |  |  | 1721 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 1816 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | n5 |  |  | 838 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 883 |  |  |  |  | 24 |  |  |  | IMD23 |  |  |  |  |
| DC_3A_n7A |  |  | 3 |  |  | 1730 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_3C_n7A |  |  | n7 |  |  | 2535 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 2655 |  |  |  |  | 10.25 |  |  |  | IMD4 |  |  |  |  |
|  |  |  | 3 |  |  | 1775 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1870 |  |  |  |  | 4 |  |  |  | IMD4 |  |  |  |  |
| DC_3A_n20A |  |  | n20 |  |  | 840 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 799 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | 3 |  |  | 1735 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1830 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | n20 |  |  | 847 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 806 |  |  |  |  | 9 |  |  |  | IMD4 |  |  |  |  |
| DC_3A_n41A |  |  | 3 |  |  | 1740 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1835 |  |  |  |  | 8.2 |  |  |  | IMD4 |  |  |  |  |
| DC_3C_n41A |  |  | n41 |  |  | 2657.5 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 2657.5 |  |  |  |  | N/A |  |  |  | IMD4 |  |  |  |  |
| DC_3A_SUL_n41A-n80A, |  |  | 3 |  |  | 1740 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1835 |  |  |  |  | 8.2 |  |  |  | IMD4 |  |  |  |  |
| DC_3C_SUL_n41A-n80A |  |  | n41 |  |  | 2657.5 |  |  |  |  |  | 10 |  |  |  | 52 |  |  |  |  | 2657.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_3A_n77A |  |  | 3 |  |  | 1740 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1835 |  |  |  |  | 26 |  |  |  | IMD23 |  |  |  |  |
| DC_3A_SUL_n77A-n80A DC_3A_n78A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |  |  |  |  |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  |  | n77, n78 |  |  | 3575 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3575 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_3A_n77A |  |  | 3 |  |  | 1765 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1860 |  |  |  |  | 8.0 |  |  |  | IMD43 |  |  |  |  |
| DC_3A_SUL_n77A-n80A, DC_3A_n78A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |  |  |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  |  | n77, n78 |  |  | 3435 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3435 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_5A_n66A |  |  | 5 |  |  | 838 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 883 |  |  |  |  | 30 |  |  |  | IMD23 |  |  |  |  |
|  |  |  | n66 |  |  | 1721 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2121 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_5A_n78A |  |  | 5 |  |  | 844 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 889 |  |  |  |  | 8.3 |  |  |  | IMD4 |  |  |  |  |
|  |  |  | n78 |  |  | 3421 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3421 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_5A_n77A8 | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | 5.5 | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 | N/A | N/A |
| DC_7_n3 |  |  | 7 |  |  | 2535 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 2655 |  |  |  |  | 13 |  |  |  | IMD4 |  |  |  |  |
|  |  |  | n3 |  |  | 1730 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1825 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_7_n5 |  |  | 7 |  |  | 2547 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 2667 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | n5 |  |  | 834 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 879 |  |  |  |  | 12 |  |  |  | IMD33 |  |  |  |  |
| DC_7A_n77A |  |  | 7 |  |  | 2540 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2660 |  |  |  |  | 7.1 |  |  |  | IMD4 |  |  |  |  |
|  |  |  | n77 |  |  | 3870 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3870 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_7A_n66A |  |  | 7 |  |  | 2535 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 2655 |  |  |  |  | 15 |  |  |  | 4th IMD |  |  |  |  |
| DC_7A-7A_n66A DC_7C_n66A |  |  | n66 |  |  | 1730 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2130 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_8A_n1A |  |  | 8 |  |  | 887.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 932.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | n1 |  |  | 1965 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2155 |  |  |  |  | 6 |  |  |  | IMD4 |  |  |  |  |
|  |  |  | 8 |  |  | 900 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 945 |  |  |  |  | 8 |  |  |  | IMD43 |  |  |  |  |
| DC_8A_n3A |  |  | n3 |  |  | 1755 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 1850 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | 8 |  |  | 897.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 942.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | n3 |  |  | 1747.5 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 1842.5 |  |  |  |  | 6.4 |  |  |  | IMD5 |  |  |  |  |
| DC_8A_n41A |  |  | 8 |  |  | 882.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 927.5 |  |  |  |  | 12.1 |  |  |  | IMD33 |  |  |  |  |
|  |  |  | n41 |  |  | 2685 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_8A_SUL_n41A- |  |  | 8 |  |  | 882.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 927.5 |  |  |  |  | 12.1 |  |  |  | IMD33 |  |  |  |  |
| n81A |  |  | n41 |  |  | 2685 |  |  |  |  |  | 10 |  |  |  | 52 |  |  |  |  | 2685 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_8A_n77A DC_8A_n78A |  |  | 8 |  |  | 897.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 942.5 |  |  |  |  | 8.3 |  |  |  | IMD4 |  |  |  |  |
| DC_8A-n79C, DC_8A-SUL_n78A-n81A |  |  | n77, n78 |  |  | 3635 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3635 |  |  |  |  | N/A |  |  |  | H4 |  |  |  |  |
| DC_12_n78 |  |  | 12 |  |  | 710 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 740 |  |  |  |  | 5.5 |  |  |  | IMD5 |  |  |  |  |
|  |  |  | n78 |  |  | 3580 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3580 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_13A_n77A |  | 13 |  |  |  |  |  | 784.5 |  |  |  |  | 5 |  |  |  |  | 20 |  |  |  |  | 753.5 |  |  |  |  | 5.5 |  |  |  | IMD5 |  |  |
|  |  | n77 |  |  |  |  |  | 3891.5 |  |  |  |  | 10 |  |  |  |  | 50 |  |  |  |  | 3891.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |
|  |  |  | 20 |  |  | 840 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 799 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_20A_n3A |  |  | n3 |  |  | 1775 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1870 |  |  |  |  | 4 |  |  |  | IMD4 |  |  |  |  |
|  |  |  | 20 |  |  | 847 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 806 |  |  |  |  | 9 |  |  |  | IMD4 |  |  |  |  |
|  |  |  | n3 |  |  | 1735 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1830 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_8A_n79A |  |  | 8 |  |  | 897.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 942.5 |  |  |  |  | 4.8 |  |  |  | IMD5 |  |  |  |  |
| DC_8A-SUL_n79A-n81A |  |  | n79 |  |  | 4532.5 |  |  |  |  |  | 40 |  |  |  | 216 |  |  |  |  | 4532.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_18A_n77A DC_18A_n78A |  |  | 18 |  |  | N/A |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  | IMD4 |  |  |  |  |
|  |  |  | n77, n78 |  |  | N/A |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_19A_n78A |  |  | 19 |  |  | N/A |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  | IMD4 |  |  |  |  |
|  |  |  | n78 |  |  | N/A |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_20A_n8A |  |  | 20 |  |  | 849.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 808.5 |  |  |  |  | 21 |  |  |  | IMD3 |  |  |  |  |
|  |  |  | n8 |  |  | 892.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 937.5 |  |  |  |  | 21 |  |  |  | IMD3 |  |  |  |  |
| DC_20A_n77A DC_20A_n78A, |  |  | 20 |  |  | 850 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 809 |  |  |  |  | 11 |  |  |  | IMD4 |  |  |  |  |
| DC_20A-SUL_n78A-n82A |  |  | n77, n78 |  |  | 3359 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3359 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_20A_n77A |  |  | 20 |  |  | 840 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 799 |  |  |  |  | 6.5 |  |  |  | IMD54 |  |  |  |  |
|  |  |  | n77 |  |  | 4159 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 4159 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_21A_n79A |  |  | 21 |  |  | 1457.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1505.5 |  |  |  |  | 18.4 |  |  |  | IMD3 |  |  |  |  |
|  |  |  | n79 |  |  | 4420.5 |  |  |  |  |  | 40 |  |  |  | 216 |  |  |  |  | 4420.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_26A_n41A |  |  | 26 |  |  | 839 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 884 |  |  |  |  | 15.6 |  |  |  | IMD3 |  |  |  |  |
|  |  |  | n41 |  |  | 2562 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 2562 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | 28 |  |  | 730 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 775 |  |  |  |  | 15.3 |  |  |  | IMD 2 |  |  |  |  |
|  |  |  | n50 |  |  | 1500 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 1500 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_28_n50 |  |  | 28 |  |  | 740 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 785 |  |  |  |  | 6 |  |  |  | IMD 4 |  |  |  |  |
|  |  |  | n50 |  |  | 1500 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 1500 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | 28 |  |  | 740 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 785 |  |  |  |  | 0.5 |  |  |  | IMD 5 |  |  |  |  |
|  |  |  | n50 |  |  | 1500 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 1500 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_28A_n51A |  |  | 28 |  |  | 742.3 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 797.3 |  |  |  |  | 5 |  |  |  | IMD 4 |  |  |  |  |
|  |  |  | n51 |  |  | 1429.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1429.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_26A_n77A, |  |  | 26 |  |  | 836.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 881.5 |  |  |  |  | 11.1 |  |  |  | IMD4 |  |  |  |  |
| DC_26A_n78A |  |  | n77, n78 |  |  | 3391 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3391 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_28A_n77A, |  |  | 28 |  |  | 705.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 760.5 |  |  |  |  | 5.5 |  |  |  | IMD5 |  |  |  |  |
| DC_28A_n78A, DC_28A-SUL_n78A-n83A |  |  | n77, n78 |  |  | 3582.5 |  |  |  |  |  | 10 |  |  |  | 50 |  |  |  |  | 3582.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_48A_n66A |  |  | 48 |  |  | 3630 |  |  |  |  |  | 20 |  |  |  | 100 |  |  |  |  | 3630 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | n66 |  |  | 1715 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2115 |  |  |  |  | 4 |  |  |  | IMD5 |  |  |  |  |
|  |  |  | 66 |  |  | 1775 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_66A_n2A |  |  | n2 |  |  | 1855 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1935 |  |  |  |  | 20 |  |  |  | IMD3 |  |  |  |  |
|  |  |  | 66 |  |  | 1750 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2150 |  |  |  |  | 4 |  |  |  | IMD5 |  |  |  |  |
|  |  |  | n2 |  |  | 1883.3 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_66A_n5A |  |  | n5 |  |  | 838 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 883 |  |  |  |  | 30 |  |  |  | IMD23 |  |  |  |  |
|  |  |  | 66 |  |  | 1721 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2121 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | 66 |  |  | 1775 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2175 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | n25 |  |  | 1855 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1935 |  |  |  |  | 20 |  |  |  | IMD3 |  |  |  |  |
| DC_66A_n25A |  |  | 66 |  |  | 1712.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2112.5 |  |  |  |  | 23 |  |  |  | IMD3 |  |  |  |  |
|  |  |  | n25 |  |  | 1912.5 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1992.5 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
|  |  |  | 66 |  |  | 1750 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2150 |  |  |  |  | 4 |  |  |  | IMD5 |  |  |  |  |
|  |  |  | n25 |  |  | 1883.3 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 1963.3 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_66A_n71A |  |  | 66 |  |  | 1750 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2150 |  |  |  |  | 5 |  |  |  | IMD4 |  |  |  |  |
|  |  |  | n71 |  |  | 675 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 629 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| DC_66A_n77A DC_66-66_n77A DC_66-66-66_n77A |  | 66 |  | 1775 |  |  |  |  |  | 5 |  |  |  |  |  |  | 25 |  |  |  |  | 2175 |  |  |  |  | 31.0 |  |  |  | IMD2 |  |  |  |
|  |  | n77 |  | 3950 |  |  |  |  |  | 10 |  |  |  |  |  |  | 50 |  |  |  |  | 3950 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 66 |  | 1760 |  |  |  |  |  | 5 |  |  |  |  |  |  | 25 |  |  |  |  | 2160 |  |  |  |  | 5.0 |  |  |  | IMD5 |  |  |  |
|  |  | n77 |  | 3720 |  |  |  |  |  | 10 |  |  |  |  |  |  | 50 |  |  |  |  | 3720 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |
| DC_66A_n78A |  |  | 66 |  |  | 1730 |  |  |  |  |  | 5 |  |  |  | 25 |  |  |  |  | 2130 |  |  |  |  | 5.0 |  |  |  | IMD5 |  |  |  |  |
|  |  |  | n78 |  |  | 3660 |  |  |  |  |  | 5 |  |  |  | 50 |  |  |  |  | 3660 |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### <Skipped unhanged sections>

#### 7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

### <Skipped unhanged sections>

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Band |  |  |  |  | Range |  |  |  |  |  | NRB |  |  |  |  |  |  |  |  |  |  | Band |  |  |  |  |  |  | Range |  |  |  |  | NRB |  |  |  |  |  |  |
|  |  | UL MOD |  |  |  |  | DL MOD |  |  |  |  |  | CH BW |  |  |  |  |  |  | DLalloc / UL alloc |  |  |  | UL MOD |  |  |  |  |  |  | DL MOD |  |  |  |  | UL/DL Ch BW |  | DLalloc / UL alloc |  |  |  |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 |  | 1 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  | All RBs / 0 |  |  |  |  |
| 24,10 |  | 1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 36 |  | 1 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  | All RBs / 0 |  |  |  |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | 3710 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 2 |  |  |  |  | 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 3720 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 23 |  | 2 |  |  |  |  | 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 3700 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  | All RBs / 0 |  |  |  |  |
| 3 8 | 2 |  |  | 1860 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3400 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 20 MHz | All RBs / 0 |  |  |
| 45 | 2 |  |  | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3920 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 4a 9 | 2 |  |  | DL 1950 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 4000 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 54 | 2 |  |  | UL 1855/DL 1935 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3790 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 64 | 2 |  |  | UL 1900/DL 1980 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3720 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 74 | 2 |  |  | UL 1885/DL 1965 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3810 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 |  | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 50 MHz |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |
| 26 |  | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 50 MHz |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |
| 34 |  | 3 |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 26 |  | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 50 MHz |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |
| 34, 10 |  | 3 |  |  |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  |  | 2657.5 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_3A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | 3495 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 23 |  | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | 3525 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  | All RBs / 0 |  |  |  |  |
| 35 |  | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | 3685 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 3a9 |  | 3 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 |  |  |  |  |  |  | All RBs/0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 44 |  | 3 |  |  |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | 3575 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 54 |  | 3 |  |  |  |  | UL 1765 / DL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | 3435 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 5 |  |  |  |  | 829 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 3316 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 23 |  | 5 |  |  |  |  | 829 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 4145 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 38 | 5 |  |  | 840 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3600 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 10 |  |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 44, 13 | 5 |  |  | UL 844/DL 889 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3421 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 54, 13 | 5 |  |  | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 4177.5 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 7 |  |  |  |  | DL 2655 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | DL 1825 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 8 |  |  |  |  | DL 932.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  |  | DL 2155 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 8 |  |  |  |  | DL 945 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | DL 1850 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 |  | 8 |  |  |  |  | DL 942.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | DL 1842.5 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_8A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 8 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 3590 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 2,8 |  | 8 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 3520 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 34 |  | 8 |  |  |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 3635 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | 11 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | 4457.7 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | QPSK |  |  |  |  | 100 MHz |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 2,9 |  | 11 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | 4512.7 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | QPSK |  |  |  |  | 100 MHz |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 13 |  |  |  |  | 782 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 3910 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 1a8 | 13 |  |  | 782 |  |  |  | 10 |  |  |  |  |  | n77 |  |  |  |  | 3600 |  |  | 100 NHz |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 10 |  |  |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 25 | 13 |  |  | DL Mid |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3755 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2a9 | 13 |  |  | DL 760 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3950 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 34 | 13 |  |  | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3891.5 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | 19 |  |  |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 2,9 |  | 19 |  |  |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  |  | 4445.02 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 20 |  |  |  |  | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | UL 1775 / DL 1870 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 |  | 20 |  |  |  |  | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  |  | UL 1735 / DL 1830 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 |  | 20 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 23 |  | 20 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | 3388 MHz |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  | All RBs / 0 |  |  |  |  |
| 34 |  | 20 |  |  |  |  | UL 850 / DL 809 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | 3359 MHz |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | 2524.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  | 15 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  | 50 MHz |  |  |  |  |  |  | All RBs / 0 |  |
| 2,8 |  | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | 2572 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  | 15 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  | 50 MHz |  |  |  |  |  |  | All RBs / 0 |  |
| 34 |  | 26 |  |  |  |  | UL 839 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | 2562 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3366 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  | 15 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / 0 |  |
| 2,8 |  | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3446 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  | 15 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / 0 |  |
| 34 |  | 26 |  |  |  |  | UL 836.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3391 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4432.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  | 15 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 60 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| 22,9 |  | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4470.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  | 15 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 60 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 28 |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  |  | 3540 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  |  | QPSK |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |
| 28 |  |  | 28 |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  |  | QPSK |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |
| 34 |  |  | 28 |  |  |  | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  |  | 3582.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  |  | QPSK |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| 45 |  |  | 28 |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3815 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  |  |  | QPSK |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | 40 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3525 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| 2,9 |  | 40 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | 41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3750 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| 26 |  | 41 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  |  |  |  | QPSK |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |
| 36 |  | 41 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  | 20MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / 0 |  |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  | 48 |  |  |  | 3557.6MHZ |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | 1778.8 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |
| 23 |  |  |  |  | 48 |  |  |  | 3581.0 MHz |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | 1777.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | N/A |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 40 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |
| 34 |  |  |  |  | 48 |  |  |  | 3630 MHz |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1715 / DL 2115 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 |  |  |  |  | 66 |  |  |  | UL1775 MHz/ DL2175 MHz |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  |  | UL 1855 / DL 1935MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| 24, 10 |  |  |  |  | 66 |  |  |  | UL 1750 MHz/ DL 2150 MHz |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  |  | UL 1883.3 / DL 1963.3MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  | 66 |  |  |  | 1720 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 3440 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  |  | QPSK |  |  |  |  |  | 20 |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 100 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |
| 23 |  |  |  |  | 66 |  |  |  | 1720 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | 3420 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  |  | QPSK |  |  |  |  |  | 20 |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |
| 38 |  |  | 66 |  | 1720 |  |  |  |  |  |  |  | n77 |  |  |  |  | 3600 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  | 20 |  | All RBs / REFSENS_ENDC_1 |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |  |
| 44 |  |  | 66 |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  | n77 |  |  |  |  | 3950 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 54 |  |  | 66 |  | UL 1760 / DL 2160 |  |  |  |  |  |  |  | n77 |  |  |  |  | 3720 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84.. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

7.3B.2.3.4.2.2	Test procedure

For test points in Table 7.3B.2.3.4.2.1-0:

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-0 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-0 Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8], Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

For test points in Table 7.3B.2.3.4.2.1-6:

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for the E-UTRA CC and NR CC to the appropriate REFSENS value defined in Tables 7.3B.2.3.5-1 to 7.3B.2.3.5-4. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.3.4.2.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable for two, three, four and five bands operation.

Table 7.3B.2.3.5-1: Reference sensitivity due to UL harmonic for EN-DC in NR FR1

| UL band | DL band |  | SCS (kHz) |  | 5 MHz (dBm) |  |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | 25 MHz (dBm) |  | 30 MHz (dBm) |  | 40 MHz (dBm) |  | 50 MHz (dBm) |  | 60 MHz (dBm) |  | 80 MHz (dBm) |  | 90 MHz (dBm) |  | 100 MHz (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n772 |  | 15 |  |  |  |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  |  |  |  |  | -71.2 +TT |  | -71.3 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -71.7 +TT |  | -71.5 +TT |  | -71.5 +TT |  |  |  |  |  | -71.3 +TT |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  | -71.3 +TT |  | -71.3 +TT |  |
|  |  |  | 60 |  |  |  |  | -72.1 +TT |  | -71.8 +TT |  | -71.7 +TT |  |  |  |  |  | -71.5 +TT |  | -71.5 +TT |  | -71.5 +TT |  | -71.4 +TT |  | -71.4 +TT |  | -71.4 +TT |  |
|  | n773 |  | 15 |  |  |  |  | -94.2 +TT |  | -92.7 +TT |  | -91.9 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -94.5 +TT |  | -92.8 +TT |  | -92.1 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | -94.9 +TT |  | -93.1 +TT |  | -92.3 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | n772, 13 |  | 15 |  |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  |  |  |  |  | -71.2 +TT |  | -71.3 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | -71.7 +TT |  | -71.5 +TT |  | -71.5 +TT |  |  |  |  |  | -71.3 +TT |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  | -71.3 +TT |  | -71.3 +TT |  |
|  |  |  |  | 60 |  |  | -72.1 +TT |  | -71.8 +TT |  | -71.7 +TT |  |  |  |  |  | -71.5 +TT |  | -71.5 +TT |  | -71.5 +TT |  | -71.4 +TT |  | -71.4 +TT |  | -71.4 +TT |  |
|  |  | n773 |  | 15 |  |  | -94.2 +TT |  | -92.7 +TT |  | -91.9 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | -94.5 +TT |  | -92.8 +TT |  | -92.1 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | -94.9 +TT |  | -93.1 +TT |  | -92.3 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | n782 |  | 15 |  |  |  |  | -71.9+TT |  | -71.9 +TT |  | -71.8 +TT |  |  |  |  |  | -71.7 +TT |  | -71.8 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -72.2 +TT |  | -72.0 +TT |  | -72.0 +TT |  |  |  |  |  | -71.8 +TT |  | -71.9 +TT |  | -71.9 +TT |  | -71.8 +TT |  | -71.8 +TT |  | -71.8 +TT |  |
|  |  |  | 60 |  |  |  |  | -72.6 +TT |  | -72.3 +TT |  | -72.2 +TT |  |  |  |  |  | -72.0 +TT |  | -72.0 +TT |  | -72.0 +TT |  | -71.9 +TT |  | -71.9 +TT |  | -71.9 +TT |  |
| 2 | n783 |  | 15 |  |  |  |  | -94.7 +TT |  | -93.2 +TT |  | -92.4 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -94.9 +TT |  | -93.3 +TT |  | -92.6 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | --95.4 +TT |  | -93.6 +TT |  | -92.8 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | n782 |  | 15 |  |  |  |  | -71.9 +TT |  | -71.9 +TT |  | -71.8 +TT |  |  |  |  |  | -71.7 +TT |  | -71.8 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -74.1 +TT15 |  | -74.1 +TT15 |  | -74.0 +TT15 |  |  |  |  |  | -73.9 +TT15 |  | -74.0 +TT15 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -72.2 +TT |  | -72.0 +TT |  | -72.0 +TT |  |  |  |  |  | -71.8 +TT |  | -71.9 +TT |  | -71.9 +TT |  | -71.8 +TT |  | -71.8 +TT |  | -71.8 +TT |  |
|  |  |  |  |  |  |  |  | -74.4 +TT15 |  | -74.2 +TT15 |  | -74.2 +TT15 |  |  |  |  |  | -74.0 +TT15 |  | -74.1 +TT15 |  | --74.1 +TT15 |  | -74.0 +TT15 |  | -74.0 +TT15 |  | -74.0 +TT15 |  |
|  |  |  | 60 |  |  |  |  | -72.6 +TT |  | -72.3 +TT |  | -72.2 +TT |  |  |  |  |  | -72.0 +TT |  | -72.0 +TT |  | -72.0 +TT |  | -71.9 +TT |  | -71.9 +TT |  | -71.9 +TT |  |
|  |  |  |  |  |  |  |  | -74.8 +TT15 |  | -74.5 +TT15 |  | -74.4 +TT15 |  |  |  |  |  | -74.2 +TT15 |  | -74.2 +TT15 |  | -74.2 +TT15 |  | -74.1 +TT15 |  | -74.1 +TT15 |  | -74.1 +TT15 |  |
|  | n783 |  | 15 |  |  |  |  | -94.7 +TT |  | -93.2 +TT |  | -92.4 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -96.9 +TT15 |  | -95.4 +TT15 |  | -94.6 +TT15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -95.0 +TT |  | -93.3 +TT |  | -92.6 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.2 +TT15 |  | -95.5 +TT15 |  | -94.8 +TT15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | -95.4 +TT |  | -93.6 +TT |  | -92.8 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.6 +TT15 |  | -95.8 +TT15 |  | -95.0 +TT15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  | n776, 7, 17 |  | 15 |  |  | -84.8 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  |  |  | -84 +TT |  | -83.9 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | -85.1 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  |  |  | -84.1 +TT |  | -84 +TT |  | -83.9 +TT |  | -83.8 +TT |  | -83.5 +TT |  | -83.7 +TT |  |
|  |  |  |  | 60 |  |  | -85.5 +TT |  | -85 +TT |  | -84.8 +TT |  |  |  |  |  | -84.3 +TT |  | -84.1 +TT |  | -84 +TT |  | -83.9 +TT |  | -83.6 +TT |  | -83.8 +TT |  |
|  |  | n774, 5, 17 |  | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  |  |  | -84.4 +TT |  | -84.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  |  |  | -84.5 +TT |  | -84.5 +TT |  | -84.4 +TT |  | -84.4 +TT |  | -84.4 +TT |  | -84.4 +TT |  |
|  |  |  |  | 60 |  |  | -85.6 +TT |  | -85 +TT |  | -84.8 +TT |  |  |  |  |  | -84.7 +TT |  | -84.6 +TT |  | -84.5 +TT |  | -84.5 +TT |  | -84.5 +TT |  | -84.5 +TT |  |
| 5 | n78 |  | 15 |  |  |  |  | -84.2 +TT |  | -84.4 +TT |  |  |  |  |  |  |  | -84.2 +TT |  | -84.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -84.3 +TT |  | -84.5 +TT |  | -84.4 +TT |  |  |  |  |  | -84.3 +TT |  | -84.5 +TT |  | -84.4 +TT |  | -84.3 +TT |  | -84.0 +TT |  | -84.2 +TT |  |
|  |  |  | 60 |  |  |  |  | -84.5 +TT |  | -84.6 +TT |  | -84.5 +TT |  |  |  |  |  | -84.5 +TT |  | -84.6 +TT |  | -84.5 +TT |  | -84.4 +TT |  | -84.1 +TT |  | -84.3 +TT |  |
| 8 | n776,7 n786,7 |  | 15 |  |  |  |  | -84.5 +TT |  | -84.4 +TT |  | -84.2 +TT |  |  |  |  |  | -84.0 +TT |  | -83.9 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -84.8 +TT |  | -84.5 +TT |  | -84.4 +TT |  |  |  |  |  | -84.1 +TT |  | -84.0 +TT |  | -83.9 +TT |  | -83.8 +TT |  | -83.5 +TT |  | -83.7 +TT |  |
|  |  |  | 60 |  |  |  |  | -85.2 +TT |  | -84.8 +TT |  | -84.6 +TT |  |  |  |  |  | -84.3 +TT |  | -84.1 +TT |  | -84.0 +TT |  | -83.9 +TT |  | -83.6 +TT |  | -83.8 +TT |  |
| 8 | n41 |  | 15 |  |  |  |  | -81.8 +TT |  | -81.7 +TT |  | -81.7 +TT |  |  |  |  |  | -81.6 +TT |  | -81.5 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -82.1 +TT |  | -81.8 +TT |  | -81.9 +TT |  |  |  |  |  | -81.7 +TT |  | -81.6 +TT |  | -81.4 +TT |  | -81.3 +TT |  | -81.2 +TT |  | -81.2 +TT |  |
|  |  |  | 60 |  |  |  |  | -82.5 +TT |  | -82.1 +TT |  | -81.1 +TT |  |  |  |  |  | -81.9 +TT |  | -81.7 +TT |  | -81.6 +TT |  | -81.3 +TT |  | -81.2 +TT |  | -81.2 +TT |  |
| 8 | n794,5 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -82.8 +TT |  | -82.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -82.9 +TT |  | -82.5 +TT |  | -82.3 +TT |  | -81.7 +TT |  |  |  | -81.2 +TT |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -83.1 +TT |  | -82.6 +TT |  | -82.4 +TT |  | -81.8 +TT |  |  |  | -81.3 +TT |  |
| 12 | n66 |  | 15 |  | -89.5 +TT |  |  | -88.8 +TT |  | -88.3 +TT |  | -87.8 +TT |  |  |  |  |  | -87.7 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -89.1 +TT |  | -88.4 +TT |  | -88.0 +TT |  |  |  |  |  | -87.8 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | -89.5 +TT |  | -88.7 +TT |  | -88.2 +TT |  |  |  |  |  | -87.9 +TT |  |  |  |  |  |  |  |  |  |  |  |
| 12 | n78 |  | 15 |  |  |  |  | -85.4 +TT |  | -85.1 +TT |  | -84.9 +TT |  |  |  |  |  | -84.9 +TT |  | -84.9 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -85.7 +TT |  | -85.2 +TT |  | -85.1 +TT |  |  |  |  |  | -85.0 +TT |  | -85.0 +TT |  | -84.9 +TT |  | -84.9 +TT |  | -84.9 +TT |  | -84.9 +TT |  |
|  |  |  | 60 |  |  |  |  | -86.1 +TT |  | -85.5 +TT |  | -85.3 +TT |  |  |  |  |  | -85.2 +TT |  | -85.1 +TT |  | -85.0 +TT |  | -85.0 +TT |  | -85.0 +TT |  | -85.0 +TT |  |
| 13 |  | n774, 5 |  | 15 |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  |  |  | -84.4 +TT |  | -84.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  |  |  | -84.5 +TT |  | -84.5 +TT |  | -84.4 +TT |  | -84.4 +TT |  | -84.4 +TT |  | -84.4 +TT |  |
|  |  |  |  | 60 |  |  | -85.6 +TT |  | -85 +TT |  | -84.8 +TT |  |  |  |  |  | -84.7 +TT |  | -84.6 +TT |  | -84.5 +TT |  | -84.5 +TT |  | -84.5 +TT |  | -84.5 +TT |  |
| 18， 19 | n774,5 |  | 15 |  |  |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  |  |  | -84.4 +TT |  | -84.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  |  |  | -84.5 +TT |  | -84.5 +TT |  | -84.4 +TT |  | -84.4 +TT |  | -84.4 +TT |  | -84.4 +TT |  |
|  |  |  | 60 |  |  |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  |  |  | -84.7 +TT |  | -84.6 +TT |  | -84.5 +TT |  | -84.5 +TT |  | -84.5 +TT |  | -84.5 +TT |  |
| 28 | n51 |  | 15 |  | -72.2 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n774,5 n784,5 |  | 15 |  |  |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  |  |  | -84.4 +TT |  | -84.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  |  |  | -84.5 +TT |  | -84.5 +TT |  | -84.4 +TT |  | -84.4 +TT |  | -84.4 +TT |  | -84.4 +TT |  |
|  |  |  | 60 |  |  |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  |  |  | -84.7 +TT |  | -84.6 +TT |  | -84.5 +TT |  | -84.5 +TT |  | -84.5 +TT |  | -84.5 +TT |  |
| 20 | n776,7 n786,7 |  | 15 |  |  |  |  | -84.5 +TT |  | -84.4 +TT |  | -84.2 +TT |  |  |  |  |  | -83.1 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -84.8 +TT |  | -84.5 +TT |  | -84.4 +TT |  |  |  |  |  | -83.2 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | -85.2 +TT |  | -84.8 +TT |  | -84.6 +TT |  |  |  |  |  | -83.4 +TT |  |  |  |  |  |  |  |  |  |  |  |
| 26 | n41 |  | 15 |  |  |  |  | -84.5 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  |  |  | -83.6 +TT |  | -83.3 +TT |  | 3.9 +TT |  | 3.1 +TT |  | 2.7 +TT |  |  |  |
|  |  |  | 30 |  |  |  |  | -84.8 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  |  |  | -83.7 +TT |  | -83.4 +TT |  | -83.0 +TT |  | -82.5 +TT |  | -82.4 +TT |  |  |  |
|  |  |  | 60 |  |  |  |  | -85.2 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  |  |  | -83.9 +TT |  | -83.5 +TT |  | -83.2 +TT |  | -82.5 +TT |  | -82.4 +TT |  |  |  |
| 26 | n776,7 n786,7 |  | 15 |  |  |  |  | -85.0 +TT |  | -84.9 +TT |  | -84.7 +TT |  |  |  |  |  | -83.6 +TT |  | -84.6 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -85.3 +TT |  | -85.0+TT |  | -84.9 +TT |  |  |  |  |  | -83.7 +TT |  | -84.7 +TT |  | -84.7 +TT |  | -84.6 +TT |  | -84.6 +TT |  | -84.6 +TT |  |
|  |  |  | 60 |  |  |  |  | -85.7 +TT |  | --85.3 +TT |  | --85.1 +TT |  |  |  |  |  | -83.9 +TT |  | -84.8 +TT |  | -84.8 +TT |  | -84.7 +TT |  | -84.7 +TT |  | -84.7 +TT |  |
| 26 | n774,5 |  | 15 |  |  |  |  | -84.9 +TT |  | -84.6 +TT |  | -84.4 +TT |  |  |  |  |  | -84.4 +TT |  | -84.4 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -85.2 +TT |  | -84.7 +TT |  | -84.6 +TT |  |  |  |  |  | -84.5 +TT |  | -84.5 +TT |  | -84.4 +TT |  | -84.4 +TT |  | -85.6 +TT |  | -84.4 +TT |  |
|  |  |  | 60 |  |  |  |  | -85.6 +TT |  | -85.0 +TT |  | -84.8 +TT |  |  |  |  |  | -84.7 +TT |  | -84.6 +TT |  | -84.5 +TT |  | -84.5 +TT |  | -85.7 +TT |  | -84.5 +TT |  |
| n28 | 18,9,10 |  | N/A |  | -89.1 |  |  | -88.7 |  | -88.3 |  | -88.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 211 |  | N/A |  | -92.7 |  |  | -93.3 |  | -91.8 |  | -90.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 212 |  | N/A |  | -95.6 |  |  | -93.3 |  | -91.8 |  | -90.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 7 |  | N/A |  | -82.7 |  |  | -82.6 |  | -82.4 |  | -82.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 66 |  | n772, 13 |  | 15 |  |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  |  |  |  |  | -71.2 +TT |  | -71.3 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | -71.7 +TT |  | -71.5 +TT |  | -71.5 +TT |  |  |  |  |  | -71.3 +TT |  | -71.4 +TT |  | -71.4 +TT |  | -71.3 +TT |  | -71.3 +TT |  | -71.3 +TT |  |
|  |  |  |  | 60 |  |  | -72.1 +TT |  | -71.8 +TT |  | -71.7 +TT |  |  |  |  |  | -71.5 +TT |  | -71.5 +TT |  | -71.5 +TT |  | -71.4 +TT |  | -71.4 +TT |  | -71.4 +TT |  |
|  |  | n773 |  | 15 |  |  | -94.2 +TT |  | -92.7 +TT |  | -91.9 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | -94.5 +TT |  | -92.8 +TT |  | -92.1 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | -94.9 +TT |  | -93.1 +TT |  | -92.3 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 66 | n782 |  | 15 |  |  |  |  | -71.9 +TT |  | -71.9 +TT |  | -71.8 +TT |  |  |  |  |  | -71.7 +TT |  | -71.8 +TT |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -72.2+TT |  | -72.0 +TT |  | -72.0 +TT |  |  |  |  |  | -71.8 +TT |  | -71.9 +TT |  | -71.9 +TT |  | -71.8 +TT |  | -71.8 +TT |  | -71.8 +TT |  |
|  |  |  | 60 |  |  |  |  | -72.6 +TT |  | -72.3 +TT |  | -72.2 +TT |  |  |  |  |  | -72.0 +TT |  | -72.0 +TT |  | -72.0 +TT |  | -71.9 +TT |  | -71.9 +TT |  | -71.9 +TT |  |
|  | n783 |  | 15 |  |  |  |  | -94.7 +TT |  | -93.2 +TT |  | -92.4 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | -95.0 +TT |  | -93.3 +TT |  | -92.6 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | -95.4 +TT |  | -93.6 +TT |  | -92.8 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 48,2, 13 |  | N/A |  | -70.7 |  |  | -70.6 |  | -70.8 |  | -70.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 48,3 |  | N/A |  | -96.1 |  |  | -93.6 |  | -92.3 |  | -91.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 15: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-1a: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-2 with uplink configuration specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k for each specific EN-DC combination scenario.

Table 7.3B.2.3.5-2: Reference sensitivity due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) |  | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 15 | -70.4 +TT | -70.4 +TT |  | -70.4 +TT | -70.4 +TT |  |  |  |  |  |  |  |
|  |  | 30 |  | -70.7 +TT |  | -70.7 +TT | -71.8 +TT |  |  |  |  |  |  |  |
|  |  | 60 |  | -72.4 +TT |  | -72.7 +TT | -77.0 +TT |  |  |  |  |  |  |  |
| n41 | 264 | N/A | -72.5 | -69.5 |  | -69.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | N/A | -91.2 | -89.3 |  | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.910 | -9210 |  | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n77 | 13 | N/A |  | -65.3 | -65.3 |  |  |  |  |  |  |  |  |
| n77 | 418 | N/A | -86.9 | -83.9 |  | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 282 | N/A | -69.8 | -69.8 |  | -69.8 | -68.3 |  |  |  |  |  |  |  |
| n78 | 3 | N/A | -90.6 | -89.3 |  | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.310 | -92.010 |  | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n78 | 40 | N/A | -86.9 | -83.9 |  | -82.1 | -80.9 |  |  |  |  |  |  |  |
| n78 | 418 | N/A | -86.9 | -83.9 |  | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | N/A | -69.8 | -69.8 |  | -69.8 |  |  |  |  |  |  |  |  |
| n79 | 262 | N/A | -69.8 | -69.8 |  | -69.8 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### <Skipped unchaed sections>

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n5A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS |  | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n77A | 2 | N/A | -71.3 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -89.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n78A | 2 | N/A | -71.3 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.3 | - | - | - | - | N/A |  |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| CA_3A-n20A | 3 | N/A | -92.3 | - | - | - | - | IMD4 | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | -88.0+TT | - | - | - | - | IMD4 | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  |  |  | -90.87 | - | - | - | - |  |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -70.3 | - | - | - | - | IMD23 | FDD |
|  |  |  | -70.67 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  |  |  | -88.37 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n77A8 | 5 | N/A | -89 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 5 | N/A | -91.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n78A | 5 | N/A | -89.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n3A | 7 | N/A | - | -81.3 | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | H4 | TDD |
| DC_8A_n79A DC_8A-SUL_n79A-n81A | 8 | N/A | -91.5 | - | - | - | - | IMD5 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | - |  | N/A |  | TDD |  |
| DC_13A_n77A |  | 13 |  | N/A |  | -90.8 |  | - |  | - |  | - |  | - |  | IMD5 |  | FDD |  |
|  |  | n77 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | - |  | N/A |  | TDD |  |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20A_n77A | 20 | N/A | -85.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 20 | N/A | -89.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_28_n50 | 28 | N/A | - | -79.5 | - | - | - | IMD 2 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -88.8 | - | - | - | IMD 4 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -94.3 | - | - | - | IMD 5 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n77A | 66 | N/A | -67.8 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

### <End of Changes>