3GPP TSG RAN Meeting #94-e	                      R5-221346r1

Electronic Meeting, 21st Feb – 4th Mar 2022

Agenda item:	5.3.1.17

Source:	Apple Portugal

Title:       Avoiding Scell Drop in FR2 RF UL-CA tests

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

There have been extensive discussions over the course of several meetings regarding the SCell drop in FR2 UL-CA RF test cases. The purpose of this discussion paper is to summarize current status in RAN5 on the issue of FR2 EN-DC UEs dropping NR SCell during the TRx measurement and thereby ceasing to be in UL-CA mode. This is in lieu of the DUT following the prioritization rules in TS 38.101-2 [1] and TS 38.213 [2]. The issue has been discussed in length in RAN5 [5] [6] [7] [8], [9].We further propose some solutions for agreement.

It should also be noted that there was a way forward in RAN4 (R4-2202341) as of RAN4#101-bise which discussed possible ways to limit the serving cell power to prevent excessive power scaling of secondary cells, however there is no immediate solution seemingly available. While the actual method is still under discussion, the impact of this to conformance testing and possible options to consider in RAN5 spec are discussed.

# 2	Discussion

## 2.1	Core and Test Specification background

It would be useful to recap the background from TS 38.213 [2] and TS 38.101-2 [1]. Associated clauses are extracted in the appendix at the bottom of this contribution. In summary:

- In TS 38.213 clause 7.5, UE is defined to prioritize the primary cell in case of same priority order of transmission and for operation with CA, which means the primary cell of the SCG (NR cell group) is prioritized than secondary cell in the EN-DC UE case. Moreover, this is also the default mode of operation in real deployments.

Observation 1: As per TS 38.213 prioritization rules, PCC is prioritized over SCC. This is also the mode of operation in the field

- In TS 38.101-2 clause 6.2A.4, configured transmitted power for CA is defined only with PCMAX, which is the total power of CCs as NR. Thus, it is possible to configure the output power of each component carrier with an imbalanced output power. This led to the question on whether “equal power spectral density for each RB in each component carrier” should be a mandatory condition or not.

Extract from TS 38.101-2 clause 6.2A.4
For uplink intra-band contiguous carrier aggregation, MPR is specified in clause 6.2A.2. PCMAX is calculated under the assumption that power spectral density for each RB in each component carrier is same.

In addition, RAN4 has confirmed the below

To this end, RAN4 has discussed the following two options for verification of the CA test cases:

- Option 1: Equal PSD between CCs.

- Option 2: Measure the UE as is even SCC output may be scaled down under CA mode.

RAN4 considers that equal PSD is a preferred test condition to verify the UL CA requirements. However, considering the actual UE behaviour in the field, which is subject to the prioritization rules in 38.213.RAN4 recognizes that testing details (configures/procedures) are ultimately up to RAN5.

Observation 2: RAN4 clarified via LS [2] to RAN5 that equal PSD is preferred test condition. However, actual UE behavior in the field has to be considered as well as prioritization rules in TS 38.213 to identify testing details.

Observation 3: RAN5 has agreed that the way forward recognizes prioritization rules in TS 38.213 [10]

Observation 4: RAN5 has to analyse and define test procedures which will ensure testability for intra-band UL-CA scenarios. Considering the ongoing RAN4 discussion, a potential solution from RAN4 cannot be precluded.

As discussed during RAN5#93 a summary of the LS communication between RAN4 and RAN1 was available.

Observation 5: RAN1 confirms that no changes to prioritization rules is planned as there is no known issue as such. Further any UE-specific configuration of power limits to prevent SCell drop would not conflict with RAN1 specifications but a confirmation on this is dependent on the actual RAN4 solution.

## Discussion on possible ways forward for RAN5

### 2.3.1	Background: Previous RAN5 discussion on FR2 RF Tx tests impacted by this issue

As part of the discussions during earlier RAN5 meetings, a list of tests that are impacted was identified. This information is useful to determine the real impact on coverage solely because of the SCell drop issue and how much bandwidth to spend/how best to identify a temporary workaround for the issue before a RAN4/RAN2 solution is identified.

Observation 6: There are 3 (three) completed FR2 RF UL-CA tests that are blocked solely by SCell drop issue. Other tests are not complete in the spec : either have multiple other blocking issues or are not yet introduced in the spec even in Release 15 work plan for the past many RAN5 meetings.

| TS38.521-2 clause number | Test case title | Coverage Blocked by SCell Drop Issue? | Comments |
| --- | --- | --- | --- |
| 6.2A.1 | UE maximum output power for CA | No | The MOP CA test case requires to be tested at MPR=0 and according to RAN 4 this condition is met only with 1CC. This means SCell drop cannot happen in this test case |
| 6.2A.2 | UE maximum output power reduction for CA | Yes, if other incomplete items are addressed | Multiple other incomplete items in test so SCell Drop is not only issue |
| 6.2A.3 | A-MPR for CA | No | No contents in the test case (FFS). |
| 6.2A.4 | Configured output power for CA | No | No contents in the test case (FFS). |
| 6.3A.4.2 | Absolute power tolerance | No | MU/TT is FFS |
| 6.3A.4.3 | Relative power tolerance | No | No contents in the test case (FFS). |
| 6.3A.4.4 | Aggregate power tolerance | No | No contents in the test case (FFS). |
| 6.5A.1 | Occupied bandwidth for CA | No | MU/TT is FFS, TP Analysis has been pending |
| 6.5A.2.1 | Spectrum emission mask for CA | Yes. |  |
| 6.5A.2.2 | Adjacent channel leakage ratio for CA | Yes. |  |
| 6.5A.3.0 | General spurious emissions for CA | No | Test is incomplete |
| 6.5A.3.1 | Spurious emission band UE co-existence for UL CA | No | Other incomplete items in test |
| 6.5A.3.2 | Additional spurious emissions | No | Other incomplete items in test |

### 2.3.2	Testability issues raised for approach using Pcell power limiting via TPC approach.

### The Aspect of beam peak search

It was identified by some contributors that the peak beam search will have to be performed uniquely for CA tests unless there is a UE vendor declaration of reusing the beam peak result from single carrier [9]. With the approach of limiting power on the PCell, one aspect that needs to be discussed is if this would apply during the peak beam search process as well. If so, then the limiting of PCell power would potentially prevent the SCell drop and peak beam search can be carried out in CA mode. However, the concern about time to obtain peak beam search while trying to limit the PCell power and attain equal PSD on both component carriers was raised at RAN5#92. For intra-band CA, it can be assumed that the peak beam would not differ significantly across component carriers. This has been confirmed by agreement on R5-215819 which can be extended to UL-CA.

Observation 7: RAN5 has agreed that PCC beam peak direction as the beam peak for intra-band DL CA for a frequency separation of up to 800 MHz. This can be extended to UL-CA.

### Dynamic Range Issues and Noise Impact

Another testability issue that has been raised is dynamic range issues and the need to incorporate impact of Noise on testability and MU. An analysis of some test points performed earlier for the MPR FR2 UL-CA test indicates a few aspects

- The impact of noise floor increases with channel bandwidth

- Impact is lower for lower MPR values

- Since measurement for UL-CA tests occurs on per CC basis, some of the dynamic range impact can be approximated to the single carrier case.

[12] provided an extensive analysis of the testability limit and MU value for “influence of noise” for FR2 CA tests where MPR tests.

Observation 8: Focusing on a reduced set of test points for MPR UL-CA tests allows to avoid testability issues related to noise impact while enabling testing for UL-CA configurations. This can be based on

- - Limited to up to aggregated channel bandwidth of 800 MHz

- - Limit test points to MPR of 5 dB (from <=200 MHz CBW Single Carrier MPRwt Table) and 3 dB (from 400 MHZ CBW Single Carrier MPRwt Table).

- Additional analysis and selecting a subset of test points would help arrive at a stable set of MPR FR2 UL-CA test points where noise impact to measurements are minimal.

2.3.3   Option 1: Use TPC to limit PCell Power

Most of the focus of RAN5 has been to explore test mode versus non test mode options to accomplish the desired objective of the FR2 RF CA test scenarios. There has been no agreement to adopt one or the other and in lieu of the ongoing RAN4 discussion.

As stated in [4] and [7], one way of preventing SCell dropping would be to limit the maximum power for the PCell to reserve power for SCell transmissions, at least for particular transmissions e.g. for PUSCH without UCI. Moreover, by limiting the SCell maximum power in addition, a behavior similar to that for LTE in which scaling applies uniformly for each serving cell as noted previously, from 36.213,

an “equal PSD” condition, could be achieved for PUSCH transmissions.

For FR1, the configured maximum output power is specified at the antenna connector and can be determined by . For FR2, on the other hand, both the configured power per cell  and the total configured power   are specified in implementation-specific plane of references internal to the UE. Absolute power limits configured by the network are therefore not viable for these parameters. Moreover, the UE power class for FR2 is specified in terms of EIRP that is impossible to control for UE operations in the field.

Again from [4], it can be gathered that a limit relative to the configured power can be specified to work around the issue of PCell and SCell power getting limited. This would also account for the actual power back-off (up to MPR) that is applied by the UE, which is unknown to the network but included in the PHR determination. The network would then configure the UE with UE-specific relative limits Xmax,f,c on the PCell and possibly also one or more SCells

relative to an absolute reference power Pref  that could be implementation specific. The relative limitation must not necessarily apply to all transmissions, only to specific transmissions like PUSCH without UCI or of priority 0. Other transmissions would not be limited.

From 38.521-2 clause 6.2.4.3, we know that

PPowerclass – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

Also PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [24].

For intra-band CA, the MPR for each serving cell is the same as that for the total power. Hence the same power back-off is normally assumed for both the  and the . Suppose we pick . If the network configures the UE with a value of  Xmax,f,PCell > 0 dB, then the  for the PCell should be reduced by this value and power for SCells would therefore be ensured as the total power  is unchanged.

An example of intra-band UL CA with allowed MPR for FR2 UE configured with a limit relative to    and four UL component carriers is shown in Figure 4.1. The ordinate shows the transmitted power density of the component carriers in relation to the , the   and the  as seen in the respective plane of reference (different at least for the power class). The power back-off up to the allowed MPR is measured relative to the power class. Configuration of the relative limit   reduces the configured power for the PCell, the remaining power up to  is available for the SCells. Setting  = 6 dB would make possible transmissions with equal power spectral density on all configured cells if the same limit is configured for the SCells.

Figure 4.1: UL intra-band contiguous CA – avoiding power limitation on SCell in FR2

Based on reference diagram in [4] with additional parameters added

Observation 9: To prevent SCell dropping due to a large power reduction, the discussed approach in RAN5 aligns with ongoing discussions in RAN4 that seek to attain the same objective i.e. limit the PCell power.

Option 1: To prevent SCell dropping due to a large power reduction, configure a power back-off on the PCC power via TPC so that remaining power up to Pcmax is available for Scells. Start with a limited set of test points on MPR CA tests to unblock UL-CA testing.

Observation 10: One aspect that kept arising with this option, however, is the MU impact of the dynamic range/noise issues discussed in this section. Based on some feedback from TE vendors, this aspect could not really be resolved and has been a constant limitation of Option 1.

2.3.4	Option 2: Use Conformance Test Function to apply limit on PCell Power (no impact on PCC prioritization rules)

To help avoid some of the testability issues that are of concern when adjusting the power levels of PCC and SCCs during UL-CA RF conformance tests, it is proposed to have a test function that can apply the backoff or limit of power on the PCell. This will enable to UE to apply the backoff in response to the conformance test function and enable the SCells to stay active. This will enable testing of UL-CA configuration with prioritization rules enabled per TS 38.213. However, considering the impact on UEs that are already commercialized, this option should be limited to Release 16 and forward until and if a RAN4 solution is available since the RAN4/RAN2 solution is planned in Release 17. This option potentially serves as a compromise among the solutions available.

Option 2: Introduce a conformance test function, for Release 16 onward testing only, to apply the backoff Xmax,f,Pcell that the UE can apply during the UL-CA conformance tests that are configured to test at max transmit power.

Observation 11a: Based on prior discussions, this approach effectively puts the responsibility on the UE to backoff/set it transmit power in response to the test function.

Based on clarification shared during email discussions the impact of Noise has to be evaluated assuming a back-off of Xmax,f,c to the PCell power applied via a conformance test function as well.

| Number of CCs | Xmax,f,c (dB) |
| --- | --- |
| 2 | 3.0103 |
| 3 | 4.771213 |
| 4 | 6.0206 |
| 5 | 6.9897 |
| 6 | 7.781513 |
| 7 | 8.45098 |
| 8 | 9.0309 |

Based on the above the below computation of the delta SNR can be performed using the approach outlined in

|  |  | Xmax | 4.77 | dB |  |
| --- | --- | --- | --- | --- | --- |
| MPR SNR | ACP SNR | DSNR 3 CC |  |  |  |
|  |  | 400MHz | 200MHz | 100MHz | 50Mhz |
| 24.48 | 7.48 | 0.71 | 0.37 | 0.19 | 0.10 |
| 22.48 | 5.48 | 1.08 | 0.58 | 0.30 | 0.15 |
| 21.98 | 4.98 | 1.20 | 0.64 | 0.33 | 0.17 |
| 21.48 | 4.48 | 1.32 | 0.71 | 0.37 | 0.19 |
| 20.98 | 3.98 | 1.46 | 0.79 | 0.41 | 0.21 |
| 19.98 | 2.98 | 1.77 | 0.98 | 0.52 | 0.27 |
| 19.48 | 2.48 | 1.95 | 1.08 | 0.57 | 0.30 |
| 17.98 | 0.98 | 2.55 | 1.46 | 0.79 | 0.41 |
| 17.48 | 0.48 | 2.78 | 1.61 | 0.88 | 0.46 |
| 15.98 | -1.02 | 3.55 | 2.13 | 1.19 | 0.64 |
| 15.48 | -1.52 | 3.84 | 2.33 | 1.32 | 0.71 |
| 13.98 | -3.02 | 4.78 | 3.02 | 1.76 | 0.97 |
| 13.48 | -3.52 | 5.12 | 3.27 | 1.94 | 1.08 |
| 12.98 | -4.02 | 5.47 | 3.55 | 2.12 | 1.19 |
| 12.48 | -4.52 | 5.83 | 3.83 | 2.32 | 1.32 |
| 11.98 | -5.02 | 6.21 | 4.13 | 2.54 | 1.45 |
| 11.48 | -5.52 | 6.59 | 4.44 | 2.77 | 1.60 |
| 10.98 | -6.02 | 6.99 | 4.77 | 3.01 | 1.76 |
| 10.48 | -6.52 | 7.39 | 5.11 | 3.27 | 1.93 |
| 9.98 | -7.02 | 7.81 | 5.46 | 3.54 | 2.12 |
| 9.48 | -7.52 | 8.23 | 5.83 | 3.83 | 2.32 |
| 6.98 | -10.02 | 10.43 | 7.80 | 5.46 | 3.53 |
| 6.48 | -10.52 | 10.89 | 8.22 | 5.82 | 3.82 |
| 5.98 | -11.02 | 11.35 | 8.65 | 6.19 | 4.12 |
| 5.48 | -11.52 | 11.82 | 9.08 | 6.58 | 4.43 |

Observation 11b: Based on preliminary analysis (additional feedback and analysis is welcome), it is likely there will be some limitation on the number of testable points even with test function usage.

2.3.5.    Await RAN4/RAN2 configuration updates

As per the way forward identified in RAN4 [11], which will be worked on in Release 17 timeframe (but could be backward release compatible), the discussion includes “a ‘RAN4 only solution’ but changes to RAN2 specifications also needed: configured power limits for UL serving cells and MAC-CE for enabling/disabling limits”.

Observation 12: RAN4 solution being worked on in Release 17 with possibility of RAN2 updates

Option 3: Based on the Observation 7 and Observation 11, the option is to wait for RAN4/RAN2 solution (configured power limits for PCell/SCell and MAC-CE for enabling/disabling limits).

Observation 13: One aspect that has arisen from discussions is that even this approach will have the same issue of uncertainty on the UE side with respect to setting transmit power as Option 2 (same as Observation 11b).

Considering:

- Persistent MU (due to dynamic range/noise) impact seen with relying on TPC to back off PCell power (Option 1)

- Option 2 and 3 will need similar analysis in terms of which UL-CA test points are testable (both options effectively limit PCell power to enable Scell to stay active)

- Considering Option 3 is still ongoing discussion in RAN4 and not yet discussed in RAN2, timeline favors Option 2 as it will enable RAN5 to understand the effects of UL-CA conformance testing using the PCell Limit approach. From technical standpoint, both options seem to have similar disadvantages and disadvantages.

Observation 14: The RAN4 WF from RAN4#101-bis is added as reference under Appendix A.1. This gives an idea about the number of FFS items and options being discussed for this topic

Proposal 1: Prefer Option 2 (test function) to enable coverage for the testable UL-CA FR2 RF tests with no core WG dependency.

Option 3 would become the default way forward if there are strong concerns with Option2. RAN5 company inputs/preferences to both these options are critical to achieve consensus either way.

# 3	Summary

In this contribution, the following observations were made

Observation 1: As per TS 38.213 prioritization rules, PCC is prioritized over SCC. This is also the mode of operation in the field

Observation 2: RAN4 clarified via LS [2] to RAN5 that equal PSD is preferred test condition. However, actual UE behavior in the field has to be considered as well as prioritization rules in TS 38.213 to identify testing details.

Observation 3: RAN5 has agreed that the way forward recognizes prioritization rules in TS 38.213 [10]

Observation 4: RAN5 has to analyse and define test procedures which will ensure testability for intra-band UL-CA scenarios. Considering the ongoing RAN4 discussion, a potential solution from RAN4 cannot be precluded.

Observation 5: RAN1 confirms that no changes to prioritization rules is planned as there is no known issue as such. Further any UE-specific configuration of power limits to prevent SCell drop would not conflict with RAN1 specifications but a confirmation on this is dependent on the actual RAN4 solution.

Observation 6: There are 3 (three) completed FR2 RF UL-CA tests that are blocked solely by SCell drop issue. Other tests are not complete in the spec : either have multiple other blocking issues or are not yet introduced in the spec even in Release 15 work plan for the past many RAN5 meetings.

Observation 7: RAN5 has agreed that PCC beam peak direction as the beam peak for intra-band DL CA for a frequency separation of up to 800 MHz. This can be extended to UL-CA

Observation 8: Focusing on a reduced set of test points for MPR UL-CA tests allows to avoid testability issues related to noise impact while enabling testing for UL-CA configurations

Observation 9: To prevent SCell dropping due to a large power reduction, the discussed approach in RAN5 aligns with ongoing discussions in RAN4 that seek to attain the same objective i.e. limit the PCell power.

Option 1: To prevent SCell dropping due to a large power reduction, configure a power back-off on the PCC power via TPC so that remaining power up to Pcmax is available for Scells. Start with a limited set of test points on MPR CA tests to unblock UL-CA testing.

Observation 10: One aspect that kept arising with this option, however, is the MU impact of the dynamic range/noise issues discussed in this section. Based on some feedback from TE vendors, this aspect could not really be resolved and has been a constant limitation of Option 1.

Option 2: Introduce a conformance test function, for Release 16 onward testing only, to apply the backoff Xmax,f,Pcell that the UE can apply during the UL-CA conformance tests that are configured to test at max transmit power.

Observation 11a: Based on prior discussions, this approach effectively puts the responsibility on the UE to backoff/set its transmit power in response to the test function.

Observation 11b: Based on preliminary analysis (additional feedback and analysis is welcome), it is likely there will be some limitation on the number of testable points even with test function usage.

Observation 12: RAN4 solution being worked on in Release 17 with possibility of RAN2 updates

Option 3: Based on the Observation 7 and Observation 11, the option is to wait for RAN4/RAN2 solution (configured power limits for PCell/SCell and MAC-CE for enabling/disabling limits).

Observation 13: One aspect that has arisen from discussions is that Option 3 will require similar analysis of testability and UE side uncertainty with respect to setting transmit power as Option 2.

Observation 14: The RAN4 WF from RAN4#101-bis is added as reference under Appendix A.1. This gives an idea about the number of FFS items and options being discussed for this topic

Proposal 1: Prefer Option 2 (test function) to enable coverage for the testable UL-CA FR2 RF tests with no core WG dependency.

Option 3 would become the default way forward if there are strong concerns with Option2. RAN5 company inputs/preferences to both these options are critical to achieve consensus either way.

# 4	References

Action_Points_RAN5#90-e_atRFsession

R5-2103124, LS from RAN4 to RAN5 on SCell Dropping due to PCC Prioritization, 3GPP TSG RAN WG5 Meeting #91-e, February 2021

R5-211227, Discussion on PCC prioritization, 3GPP TSG RAN WG5 Meeting #90-e, February 2021

R4-2101722 Scell Dropping behavior and verification thereof, Ericsson, 3GPP TSG RAN WG4 Meeting #98-e, February 2021

R5-212812 Discussion on Tx power allocation prioritization for FR2 UL CA testing, Qualcomm, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-213809 Discussion on SCell dropping and UL CA testing in NR FR2, Ericsson, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-213818 Discussion on FR2 test procedure updates to handle PCC Prioritization, Apple, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-212919, Discussion on FR1 and FR2 PCC priorization, Huawei, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-213809, Consideration on Scell drop issue for FR2 UL CA, Anritsu, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-211941, Discussion on PCC prioritization for FR1 and FR2 UL CA testing, Qualcomm Finland, 3GPP TSG RAN WG5 Meeting #90-e, February 2021

[11]   R4-2114949, WF on solution for Scell dropping issue, Huawei, RAN4#100

[12]   R5-206611, Noise impact for FR2 CA test case where MPR applies, Anritsu, RAN5#89e

[13]	R5-211104, Correction of parameters of FR2 ACLR test case, RAN5#90e

# APPENDIX

A.1 RAN4 WF from RAN4#101-bis

Background

SCell dropping proposals

- Option 1: the configured maximum power Pcmax,f,c for the serving cells are modified by UE-specific configured power limits, and can be modified/enabled/disabled by MAC/CE for fast adaptation to changing radio conditions and applies for concurrent transmissions; The relative limits apply for concurrent UL transmissions, if only transmission scheduled on one cell this would get all available power up to PCMAX

- Option 2: Power distribution among PCell and SCell proportionally should be considered at NW side according to the RB resource scheduling info for CCs, and the power ratio for PCell and SCell(s) can be configured to UE. The power ratio can be configured via RRC on UE specific basis, and enable/disable via DCI or MAC-CE for fast adaption of the dynamic RB resource allocation for PCell and SCell(s).

- Option 3: Define new parameter to indicate priority between configured UL cells for the UE. Supporting Ran4 based solution introducing any new network controlled parameters should be optional for the UE.

- Option 4: RAN4 will not agree a solution before receiving RAN1 feedback about the feasibility of one of the proposed solutions.

- Option 5: Considering postpone this work to future release if no consensus can be made.

Pcmax,CA and PHR for CA Proposals

- Consider reporting Pcmax,CA and total PHR for band combination.

Way forward

SCell dropping solution if needed, the following aspects are to be further discussed

Agreement:

- Consider adding a new RRC signalling in the feature list, details depends on the final solution if any

  - Discuss whether to test delta-Pcmax to minimize the efforts in RAN5 testing

- FFS: Configured maximum power Pcmax,f,c for serving cells can be modified by a UE-specific parameter, which is configured by network

- FFS the network configured parameter

  - can be semi-persistent/dynamic configured

  - can be fast enabled/disabled

  - can be adjusted dynamically due to the allocated resource in PCell/Scells

  - need to make sure the priority is not always on PCell

  - can be fast enable/disable or modified by MAC-CE

  - can guarantee equal PSD among CCs, though equal PSD is not always the case

- The solution should have no RAN1 impact in Rel-17

- Whether and how to implement the RAN4 requirements based on the final solution if any

- FFS on the measurement, i.e. whether to reflect the network configured parameter in Pumax

- FFS whether Pcmax,CA and PHR for CA is needed considering the following issues

  - 1) Whether the proposal is mandatory from now on

  - 2) Whether it override the current Per-CC PHR reporting? Or what is NW expected to do if receiving both per-CC and per-BC PHR reports? Or there is only one report, either per-CC or per-BC, but not both?

  - 3) Does network really need to know the PCMAX,CA?

- FFS on how to proceed if no consensus can be reached.

A.2 Summary of feedback and Company preferences/support during RAN5#93e

| Options | Companies Supporting [ ] -> not opposed to | Comments |
| --- | --- | --- |
| Option 1 Configure a power back-off on the PCC power via TPC so that remaining power up to Pcmax is available for Scells. Start with one prioritized scenario (For example 2CC 100+100 MHz QPSK modulation) in MPR CA tests to unblock UL-CA testing. | Huawei, Apple, [Qualcomm], [Ericsson], [DISH] | Apple associated CR R5-217717 aligned with this option Pros - Aligns with potential solution being discussed in RAN4 (limit PCell power) but accomplishes it with RAN5 test procedure updates - Some test complexity indicated previously (beam peak search) can be resolved via earlier agreements to use PCC based beam peak direction which can be extended to UL-CA, and by limiting test points - Applicable from Rel.15 onwards Cons - Additional analysis needed for some pending testability items and to be captured in Editor’s notes if adopted (MU, power tolerance, stability impact especially with higher # of CCs) |
| Option 2 Introduce a conformance test function, for Release 16 testing only, to apply the backoff Xmax,f,Pcell (no impact on prioritization rules) that the UE can apply during the UL-CA conformance tests that are configured to test at max transmit power. | [Apple], Qualcomm, Verizon, Anritsu | Example CR implementation listed in Annex of Discussion Paper. Pros - Enables use of test function to limit PCell power; while aligning with prioritization rules (as expected by 38.213/real network behavior) and potentially simplifying procedure (Per Ericsson this needs to be investigated as RAN4 approach could be similar and open issues have been identified). - Applicable in Rel16 and forward Cons - UE tested in “conformance test only” mode not aligned with real network - Additional UE implementation of TF. Updates needed across specs other than 38.521-2 (38.508-1, 38.509). |
| Option 3 Based on the Observation 7* and Observation 11*, the option is to wait for RAN4/RAN2 solution targeted in Release 17 | Ericsson, DISH | - Ericsson CR R5-217652 (RAN4 dependent) was aligned with this option Pros - Default option. Helps align with way forward from core WG - Long term solution, when available Cons - Timeline and RAN4/RAN2 agreements are TBD. No conclusion at RAN4#100 (discussion will continue at RAN4#101-bis). RAN2 discussion pending start. - As of now, applicable Rel17 and forward only [Ericsson] - although might be early implementable in Rel-16). |
| Option4: Implement option1 for Rel 15 and Option2 for Rel16 and forward. | Orange, [Apple], [Qualcomm] | Pros - Allows test to be completed for Rel16 and above via Option 2 and updated for Rel15 with Editor’s notes capturing pending items for Option 1. Cons - Spec update becomes complicated to manage for two releases. Will need maintenance/update within test case. - Device validations will be different in Rel.15 and Rel.16 as test procedure and MU impact is different in each (although one option might potentially have lower MU impact) |