3GPP TSG-RAN5 Meeting #94-e	R5-221348

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0056 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test function to limit Pcell power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This new conformance test function allows limiting PCell power during UL-CA test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces new test function to limit PCell Power |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL-CA tests blocked from execution due to SCell Drop Issue |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11, 6.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Dependent on Discussion Paper R5-221346 r1 makes the following changes - Resolves 3GU issue due to ME not being checked on cover page - Updates the General Section 5.1.1 with more details - Adds diagram in correct format along with title added - Adds section 5.11.2 with procedure - Adds message definitions under clause 6.11 |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

## 5.11	UE Power Limit Function (UPLF)

### 5.11.1	General

The UE Power Limit Function is intended for the SS to communicate to the UE to apply a backoff of transmitted power on the NR primary component carrier when in FR2 carrier aggregation mode.  On activation of this test function, the UE shall apply a configured power backoff on the primary component carrier to provide sufficient power head room for the other (secondary) component carrier(s).

The SS uses the UE Power Limit test mode activation procedure to send a power limit request to the UE. The transmission from the SS  shall be a vector of values, where the number of the reported values equals the back-off or power limit to be applied by the UE.

Figure 5.11.1-1: UE Power Limit test mode procedure

### 5.11.2	UE Power Limit Procedure

### 5.11.2.1	Initiation

The SS requests the UE to apply the power limit by transmitting an APPLY POWER LIMIT REQUEST message. The UE confirms that the power limit function is activated by sending a APPLY POWER LIMIT RESPONSE.

#### 5.11.2.2	Reception of APPLY POWER LIMIT REQUEST by UE

#### When the UE receives the APPLY POWER LIMIT REQUEST message, then the UE shall:

1>	if the UE is operating in FR2 AND is in RRC_CONNECTED state:

2>	if APPLY POWER LIMIT REQUEST = 000

3>	UE limits power on PCell cprresponding to NR FR2 2CA

2>	else if APPLY POWER LIMIT REQUEST = 001

3>	UE limits power on PCell cprresponding to NR FR2 3CA

2>	else if APPLY POWER LIMIT REQUEST = 010

3>	UE limits power on PCell cprresponding to NR FR2 4CA

2>	else if APPLY POWER LIMIT REQUEST = 011

3>	UE limits power on PCell cprresponding to NR FR2 5CA

2>	else if APPLY POWER LIMIT REQUEST = 100

3>	UE limits power on PCell cprresponding to NR FR2 6CA

2>	else if APPLY POWER LIMIT REQUEST = 101

3>	UE limits power on PCell cprresponding to NR FR2 7CA

2>	else if APPLY POWER LIMIT REQUEST = 101

3>	UE limits power on PCell cprresponding to NR FR2 8CA

2>	Transmit APPLY POWER LIMIT RESPONSE message

1>	else:

2>	the UE behaviour is unspecified.

The APPLY POWER LIMIT REQUEST & APPLY POWER LIMIT RESPONSE Message Octets are defined below in Section 6.11.

<Unchanged sections skipped>

## 6.11	UE Power Limit messages

### 6.11.1	APPLY POWER LIMIT REQUEST

This message is only sent in the direction SS to UE. The values sent the SS as part of this test function maps to the values defined in Table [6.2A.2.1.4-1]  of TS 38.521-2 [13]

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [7], subclause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [7], subclause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| UE Apply Power Limit Request |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | octet 1 |

where UE APPLY POWER LIMIT REQUEST is a single octet IE:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE APPLY POWER LIMIT REQUEST |  |  |  |  |  |  |  | octet 1 |

UE Apply Power Limit Request values

Bits

8 7 6 5 4 3 2 1

0 0 0 0 0 0 0 0	NR FR2 2CA

0 0 0 0 0 0 0 1	NR FR2 3CA

0 0 0 0 0 0 1 0	NR FR2 4CA

0 0 0 0 0 0 1 1	NR FR2 5CA

0 0 0 0 0 1 0 0	NR FR2 6CA

0 0 0 0 0 1 0 1	NR FR2 7CA

0 0 0 0 0 1 1 0	NR FR2 8CA

### 6.11.2	APPLY POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| UE Apply Power Limit Response |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | octet 1 |

<End of changes>