3GPP TSG-RAN5 Meeting #95-e	R5-222100

# Online, , 9th - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0360 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TC 7.24a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is not clear in what message the UE includes the SDP answer for dialog 2 (although PRACK is the only possibility). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clarification that the SDP answer is sent in PRACK Fixed editorials as found. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test spec remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.24a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to R5s220237. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## 7.24a	MTSI MO Voice Call / Forking / UE receives two preliminary responses and one early dialog termination / 5GS

7.24a.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use preconditions and to not use GRUU and to not suppress forking }

ensure that {

when { UE is made to start a voice call, processes the call setup until before completion, and then receives another early dialog indication }

then { UE processes the second early dialog until before completion }

}

(2)

with { UE having proceeded two early dialogs }

ensure that {

when { UE receives 199 Early Dialog Terminated for the first dialog, followed by 200 OK for the second dialog }

then { UE sends ACK on the second dialog and maintains it }

}

7.24a.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.3.1]:

Upon receiving a 199 (Early Dialog Terminated) provisional response to an established early dialog the UE shall release resources specifically related to that early dialog.

7.24a.3	Profile Requirements (Informative)

[GSMA NG.114 V1.0, clause 2.3.7]:

It is also possible that the network or the terminating UE will need to release an early dialog using the 199 (Early Dialog Terminated) response defined in IETF RFC 6228 [85]. To support this, the originating UE must include the "199" option tag in the Supported header field in the initial INVITE request and must understand a 199 (Early Dialog Terminated) response code and act as specified in section 5.1.3.1 of 3GPP TS 24.229 [8].

7.24a.4	Test description

7.24a.4.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use preconditions.

-	UE is configured to not use GRUU.

-	UE is configured to not suppress forking via the no-fork directive.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS

7.24a.4.2	Test procedure sequence

Table 7.24a.4.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to start a voice call. | - | - | - | - |
| 2-8 | Steps 2-8 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 9-12 | UE continues call setup (dialog 1) (steps 2-5 of Annex A.4.1) | - | - | - | - |
| 13-15 | SS triggers resource reservation: Steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 16-20 | UE continues call setup (dialog 1) (steps 6-10 of A.4.1) | - | - | - | - |
| 21 | SS sends 183 Session Progress with a different to-tag (dialog 2) (step 3 of Annex A.4.1) | <-- | 183 Session Progress | - | - |
| 22 | UE sends PRACK, including SDP answer as specified in A.4.1 step 6 (dialog 2) (step 4 of Annex A.4.1) | --> | PRACK | 1 | P |
| 23 | SS responds to PRACK (step 5 of Annex A.4.1) | <-- | 200 OK | - | - |
| 24 | SS sends 180 Ringing reliably (step 8 of A.4.1) | <-- | 180 Ringing | - | - |
| 25 | UE acknowledges reception of 180 Ringing (step 9 of A.4.1) | --> | PRACK | - | - |
| 26 | SS responds to PRACK | <-- | 200 OK | - | - |
| 27 | SS sends 199 Early Dialog Terminated (dialog 1) | <-- | 199 Early Dialog Terminated | - | - |
| 28 | SS sends 200 OK for INVITE (dialog 2) | <-- | 200 OK | - | - |
| 29 | UE sends ACK (dialog 2) | --> | ACK | 2 | P |
|  | UE maintains dialog 2 by not sending BYE | - | - | 2 | F |
| 30-31 | SS waits 5 seconds and releases the call (dialog 2) (Annex A.8) | - | - | - | - |

7.24a.4.3	Specific message contents

Table 7.24a.4.3-1: INVITE (in steps 2-8, table 7.24a.4.2-1)

| Derivation Path: Annex A.4.1, Step 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Supported |  |  |  |  |
| option-tag |  | 199 |  | RFC 6228 [49] |

Table 7.24a.4.3-2: 183 Session Progress (step 21, table 7.24a.4.2-1)

| Derivation Path: Annex A.4.1, Step 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To |  |  |  | RFC 3261 [6] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | any value different from the one used for dialog 1 |  |  |
| Contact |  |  |  | RFC 3261 [6] |
| addr-spec |  | px_IMS_CalleeContactUri2 |  |  |
| Message-body |  | o=- 1111111112 1111111111 IN (addrtype) (unicast-address for SS) |  | RFC 4566 [38] |

Table 7.24a.4.3-3: 199 Early Dialog Terminated (step 27, table 7.24a.4.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.26

Table 7.24a.4.3-4: 200 OK (step 28, table 7.24a.4.2-1)

Derivation Path: Annex A.4.1, step 11, with same to tag as used in step 21 of Test procedure sequence

Table 7.24a.4.3-5: ACK (step 29, table 7.24a.4.2-1)

Derivation Path: Annex A.4.1, step 12, with same to tag as used in step 21 of Test procedure sequence

# *** Last Change ***